# 3GPP TSG-RAN WG4 Meeting #114	R4-2503337

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.5.2

Source: 	Xiaomi

Title: 						Methods for reducing the number of UE Rx chains

Document for:	Discussion

- Introduction

Rel-19 agreeded a study item to study the feasibility and requirements impact on using shared Rx chain to support mutilple fragemented carriers in single band under intra-band NC CA.

In previous RAN4 meeting, the reference archirecture and target scope has been clarified and agreed as following:

#### Sub-topic 1-2: Study scope clarification Agreement: - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below: #### Merge issue 2-0/2-1 discussion: Agreement: The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

In previous RAN4 meeting, following agreements were reached for evaulation of RF requirements impact:

RAN4#114 - How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers Option 1: Legacy band combination reporting scheme + Rx sharing capability indication - FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”. Other options are not precluded - Discussion on semi-statically switch hardware resources procedure and fallback considerations - Triggering condition to enable ”One RF Rx chain” FFS: - FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network. - Triggering condition for UE working in one Rx RF chain is needed - Triggering condition for UE working in legacy Rx RF chains is needed - The switching should be not be dynamic and be under network control with UE indication of condition. - Discussion on applicability for PCell and SCell - Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

In this contribution, views on Methods for reducing the number of UE Rx chains were provided.

- Discussion

Signaling related

On signalling aspect, our preference we shall take this feature based on existing band combination structure with addtional necessary indications.

When UE access to the network, network will inquire necessary information from UE and if UE supported fragemented carriers feature and network have such fragemented operation deployment scenario which may further inquire the supporting indications.

Proposal 1 : Supporting option 1 ” Legacy band combination reporting scheme + Rx sharing capability indication”.

- The indication can be per band per band combinations.

On Rx sharing capability indication, UE capability on supporting the max separation between fragemented carriers can be different which is related to both RF and BB capacity.  We sugguest to introduce Rx sharing capability indication based on max frequency seperation  across carriers and number of CCs.

Proposal 2: Introduce ” Rx sharing capability indication” based on max frequency seperation  across carriers and number of CCs.

NR intra-band non-contiguous DL CA frequency separation classes with single Rx chain mode

| NR NC DL CA frequency separation class | Maximum allowed frequency separation | Maximum allowed number of CCs |
| --- | --- | --- |
| A | 100 MHz | 2 |
| B | 200MHz | 2 |
| C | 200 MHz | 4 |

- Number of Rx and MIMO layers

On 2Rx applicability, in previous meeting two options discussed as captured in moderator summary as following:

- Recommended WF - Discuss if the case of the fragmented carrier operation in the bands n7 and n41 could relieve the UE, only when configured in fragmented carrier operation in these bands, of the requirement to support 4Rx. - Option 1: UE is allowed to only support 2Rx when UE is configured DL NCCA in the mandatory 4Rx band when configured in fragmented carrier operation - Option 2: UE shall still support 4Rx when UE is configured DL NCCA in the mandatory 4Rx band when configured in fragmented carrier operation

Firstly, we need to clarify companies’ understanding on existing 4Rx mandatory applicable rules in the specifications.

Refer to TS 38.101-1 as captured below:

## 7.2 Diversity characteristics The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination. Unless otherwise stated, the following applicability rules apply, - For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for four and eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports. - For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, the UE shall be verified with eight Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with eight Rx antenna ports unless the UE does not support eight Rx ports for band(s) in a band combination in which case those band(s) shall be verified with four Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports. - The above rules apply for all clauses except for clause 7.9. A Redcap UE is required to be equipped with a minimum of single Rx antenna port and maximum of two Rx antenna ports. Clause 7 requirements for four Rx antenna ports do not apply to a RedCap UE. If aUE indicates intraBandNR-CA-non-collocated-r18, both Rx power imbalance requirements as specified in clause 7.10A and Rx requirements in clauses 7.3 – 7.9 shall be verified.

From above general statement in existing TS 38.101-1, our understanding on the bands which 4Rx is mandatory, this 4Rx mandatory applied for both standalone single CC or CA.

Observation 1: 4Rx mandatory is applied for both standalone single CC or CA as captured in TS 38.101-1.

Refer to UE capability in terms of MIMO layers (no dedicated signaling on Rx antenna ports) in TR 38.823/TS 38.306

RAN2 capability signaling on PDSCH MIMO layer IE is specified as per CC per band per band combination basis with most flexibility.

Observation 2: RAN2 PDSCH MIMO layer capability IE is specified as per CC per band per band combination basis.

This SI target to enable single Rx RF chain to support multiple fragmented carriers with benefits of supporting high order MIMO layers or high order CA combinations. Reducing number of MIMO layers per CC can be another way to support more fragmented carriers with limited number of Rx chains.

The story of intra-band NC CA is different which target to enable intra-band NC CA by reduced MIMO layers per CC under the limitation Rx chains.

Theoretically, supporting more fragmented carriers can be achieved by shared RF chain and/or reduced MIMO layers/RX chain per carrier. However, this is out of the scope of this SI for reducing MIMO layers to achieve more fragmented carriers or high modulation orders and actually the existing mechanism/UE capability signaling already allow such operation.

Proposal 3: UE shall maintain 4Rx mandatory applicable rule as existing specification under intra-band NC CA regardless when operating in fragmented operation.

Under single RF Rx chain operation condition with limited Rx RF chain; UE may increase MIMO layers on supported carriers or extend supporting high order band combinations with more carriers pending on RF capacity and baseband processing capacity.

Proposal 4: UE shall allow to indicate separate MIMO layer capacity under single Rx chain operation condition and fallback normal condition.

Triggering condition to enable ”One RF Rx chain”

Based on initial evaluation on Rx requirement impact due to Tx leakage and in-gap interference under single Rx chain mode, the receiver performance impact is diverse pending on configuration (DL-UL separation, Wgap length, CHBW) ,UE implementation performance on AGC adjustment and ADC design and the in-gap power level and PSD between wanted carrier(s) and interference.

On how to decide to enable ”one RF Rx chain“ or fall-back to normal mode, this can be left to UE implemention as well as UE can guranteen the Rx performance.  Once UE detect ”single Rx chain operation” not suitable, then UE can indiate to NW and fall-back to single CC by defualt, NW can reconfigure UE with single CC or normal CA mode.

Proposal 5: UE shall allow to switch between “normal CA operation” and “one RF chain operation” by indication to NW.

Proposal 6: RAN4 can further study any measurement gap/interruption required for UE to monitor “in-gap interference’.

- Conclusion

In this contribution, views on Methods for reducing the number of UE Rx chains were provided.

Proposal 1 : Supporting option 1 ” Legacy band combination reporting scheme + Rx sharing capability indication”.

- The indication can be per band per band combinations.

Proposal 2: Introduce ” Rx sharing capability indication” based on max frequency seperation  across carriers and number of CCs.

NR intra-band non-contiguous DL CA frequency separation classes with single Rx chain mode

| NR NC DL CA frequency separation class | Maximum allowed frequency separation | Maximum allowed number of CCs |
| --- | --- | --- |
| A | 100 MHz | 2 |
| B | 200MHz | 2 |
| C | 200 MHz | 4 |

Observation 1: 4Rx mandatory is applied for both standalone single CC or CA as captured in TS 38.101-1.

Observation 2: RAN2 PDSCH MIMO layer capability IE is specified as per CC per band per band combination basis.

Proposal 3: UE shall maintain 4Rx mandatory applicable rule as existing specification under intra-band NC CA regardless when operating in fragmented operation.

Proposal 4: UE shall allow to indicate separate MIMO layer capacity under single Rx chain operation condition and fallback normal condition.

Proposal 5: UE shall allow to switch between “normal CA operation” and “one RF chain operation” by indication to NW.

Proposal 6: RAN4 can further study any measurement gap/interruption required for UE to monitor “in-gap interference’.

Reference

[1] R4-2420337, WF on NR FR1 DL fragmented carriers study, MTK

[2] R4-2502873, WF on NR FR1 DL fragmented carriers study