# 3GPP TSG-RAN WG4 Meeting #114	R4-2503338

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.5.3

Source: 	Xiaomi

Title: 						Impacts on UE RF requirements and DL performance

Document for:	Discussion

- Introduction

Rel-19 agreeded a study item to study the feasibility and requirements impact on using shared Rx chain to support mutilple fragemented carriers in single band under intra-band NC CA.

In previous RAN4 meeting, the reference archirecture and target scope has been clarified and agreed as following:

#### Sub-topic 1-2: Study scope clarification Agreement: - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below: #### Merge issue 2-0/2-1 discussion: Agreement: The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

In previous RAN4 meeting, following agreements were reached for evaulation of RF requirements impact:

RAN4#113 #### Issue 3-3: Discussion of impacts on DL performance for UE enabling fragmented carrier mode Companies are encouraged to bring analysis with their own assumptions following the table format #### Issue 3-4: Discussion on in-gap requirements RAN4 evaluate performance impact on the cases: - image of in-gap interference overlaps with wanted carrier - For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC - image of in-gap blocking interference does not overlap with wanted carrier RAN4#114 #### Issue 3-1: Discussion on maximum input level for fragmented carriers - Maximum power level including all RF signals and interference at the UE input does not exceed -25 dBm #### Issue 3-2: Discussion of impacts on DL performance for UE enabling one Rx RF chain mode - Relaxation on REFSENS for both PCC and SCC will be evaluated and captured in the study outcome report. - DL performance degradation can be considered based on configurations - Configurations leading to no relaxation are not precluded pending on the evaluation results. The exact values and applicable conditions shall be decided in the WI phase if there is corresponding WI - Maintain the table format agreed in previous RAN4 meeting. Detailed evaluation assumptions such as imbalanced PRx/DRx noise level and other assumptions are encouraged to be provided from companies’ evaluation TPs #### Issue 3-3: Discussion on in-gap interference requirements - For image of in-gap interference not overlap with wanted CC: Evaluate whether relaxation on ACS/IBB is needed based on evaluated ΔRIBNC on PCC and SCC. - For image of in-gap interference(ACS/IBB) overlap with wanted CC: companies are encouraged to provide the impact of interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC. #### Issue 3-4: Discussion on DL fragmented carriers in TDD bands - Option 2: Separate LOs are assumed for UL and DL for TDD band

In this contribution, views on receiver requirements impact were provided.

- Discussion

Issue 3-2 Impact to REFSENS requirements due to Tx leakage

As evaluated by companies in previous RAN4 meeting, major impact on REFSENS requirements was Tx leakage due to reduced attenuation on UL carrier frequency with wideband LPF under “single Rx chain mode”.  The performance impact can be diverse pending on the configuration (DL and UL frequency separation, operating frequency) and UE implementation (ADC performance, and NF increase due to AGC adjustment, LPF performance).

Initial evaluation based on collected experienced performance from products were provided in below tables on band n3 with configuration as CA_n3(2A), 5MHz-65MHz-5MHz (PCC-Wgap-SCC):

| Parameters | Set1 | Set2 |
| --- | --- | --- |
| Duplex Isolation | 50dBc | 50dBc |
| LPF attenuation | 12 dBc | 12 dBc |
| NF increase due to AGC adjustment | 10 | <5 |
| ADC | 11 bits with 12dBFS | 12bits with 6dBFS |
| SNR after ADC (dB) (additional ΔRIBNC) | PCC: -14 SCC: -9.9 | PCC: -4.5 SCC: -3.4 |

Observation 1: Additional REFSENS performance degradation due to Tx leakage are varied pending on configuration (DL-UL separation, Wgap length, CHBW) and UE implementation performance on AGC adjustment and ADC design.

Observation 2: With initial evaluation based on experienced product performance, REFSENS performance degradation (extra ΔRIBNC ) can be varied from 14 dB ~4.5dB for PCC and 9.9~3.4dB for SCC for CA_n3(2A), 5MHz-65MHz-5MHz (PCC-Wgap-SCC).

Based on above observation, we suggest to introduce separate REFSENS requirements for “Single Rx Chain mode” as per band per configuration basis.

Proposal 1: During WI phase if any, separate REFSENS requirements for “single Rx chain mode” shall be introduced (with extra ΔRIBNC ) as per band per configuration basis

  - The requirements shall be discussed and decided case by case manner per operators’ request on target band and associated configuration.

Issue 3-3 Impact to in gap requirements - Scenario 1: For image of in-gap interference not overlap with wanted CC

For image of in-gap interference not overlap with wanted CC: we need to evaluate whether relaxation on ACS/IBB is needed.

For ACS case 2, the wanted carrier power is fixed value as -56.5dBm/-25dBm for wanted carrier and interference carrier. The received power wanted carrier is much higher than Tx leakage power and the impact due to Tx leakage is negligible considering duplex can provide ~50dBc attenuation.

Observation 3: No impact to ACS case 2 requirements due to Tx leakage.

For ACS case 1and IBB testing, the wanted carrier power is relative value compared to REFSENS.

Based on initial analysis, if wanted signal power can be set based on adjust REFSENS then the impact to ACS and IBB requirements is marginal.

Observation 4: If same relaxation on REFSENS can be applied for ACS/IBB test cases, then the impact to ACS/IBB requirements is marginal.

Proposal 2: At least same relaxation for REFSENS shall be applied for ACS/IBB test cases on “Single Rx mode”.

Issue 3-3 Impact to in gap requirements - Scenario 2: For image of in-gap interference overlap with wanted CC

With in-gap interference overlapping with wanted CC, by assuming UE receiver still have capability with 25dBc image rejection capability.

The potential impact to ACS/IBB requirements with image of in-gap interference overlap with wanted CC is pending on the power ration between in gap interference and wanted signal power.

To avoid the impact to ACS/IBB , restriction with power difference between wanted carrier and interference in gap less than 20dB can solve the issue.

Proposal 3: Restrict the received power difference between wanted carrier and interference in gap shall be less than [20dB] as one  condition for “single Rx operation mode”.

- Conclusion

In this contribution, views on receiver requirements impact were provided.

Observation 1: Additional REFSENS performance degradation due to Tx leakage are varied pending on configuration (DL-UL separation, Wgap length, CHBW) and UE implementation performance on AGC adjustment and ADC design.

Observation 2: With initial evaluation based on experienced product performance, REFSENS performance degradation (extra ΔRIBNC ) can be varied from 14 dB ~4.5dB for PCC and 9.9~3.4dB for SCC for CA_n3(2A), 5MHz-65MHz-5MHz (PCC-Wgap-SCC).

Proposal 1: During WI phase if any, separate REFSENS requirements for “single Rx chain mode” shall be introduced (with extra ΔRIBNC ) as per band per configuration basis

  - The requirements shall be discussed and decided case by case manner per operators’ request on target band and associated configuration.

Observation 3: No impact to ACS case 2 requirements due to Tx leakage.

Observation 4: If same relaxation on REFSENS can be applied for ACS/IBB test cases, then the impact to in gap requirements is marginal.

Proposal 2: At least same relaxation for REFSENS shall be applied for ACS/IBB test cases on “Single Rx mode”.

Proposal 3: Restriction of the power difference between wanted carrier and interference in gap  less than [20dBc] as one condition for “single Rx chain operation mode”.

Reference

[1] R4-2420337, WF on NR FR1 DL fragmented carriers study, MTK

[2] R4-2502873, WF on NR FR1 DL fragmented carriers study