# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503340

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	On optimizing CSSF for FR2-1 L3 measurement delay

Source: 	OPPO

Agenda item:	7.17.2.2

Document for:	Discussion

Introduction

In last meeting, a WF [1] was approved on FR2-1 SSB based L3 measurement delay reduction.

In this contribution, we will provide discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSFoutside_gap.

Discussion

In last meeting, RAN4 agreed on solution 1 and solution 3 for CSSF reduction, and send LS to RAN2 to inform them of the potential signaling impact.

Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap.

After discussion internally, our RAN2 guys think it is not very clear about the two-level network indication.

Firstly, usually the measurement configuration can already reflect the measurement purposes and cared carriers from network side. If some SCC is configured in one MO, it should be taken care of by UE during the measurement and report, and the network can reconfigure and adapt the measurement configuration according the measurement reports and real-time demand. In this case, the other SCCs not selected can be taken as low priority or even released from MO list. The benefit of network indication to enable CSSF enhancement seems quite limited.

Secondly, about the additional network indication to measure a SCC, some sub-case is also not reasonable to RAN2. In RAN2’s assumption, if UE is configured with the measurement either in servingcellMO or MOs for neighbor cell measurements, the related reports are expected to be configured together, either for serving cell or neighbor cell measurements. It may happen that one UE is performing measurements but does not fulfill the condition of reporting within some time duration. But it seems not correct configuration for only measurement or report on one CC.

Based on what RAN4 agreed in last meeting, IF there is no SCCs on which UE is configured to report SSB based measurements in band (including both serving cell and neighbor cell measurement), then it means there is no measurement requirement for these SCC in band. Then it is quite strange that why network need further to indicate to measure such SCC and UE shall follow the indication.

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

We can understand the intention to reduce the measurements on both serving and neighbour cells. But the discussed case is redundant to us. So we suggest to remove the sub-case and update the agreements as follows:

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

Observation 1: The MO (e.g., either for serving cell or neighbor cell) is assumed to configured with report configuration based on RAN2’s spec.

Proposal 1: For solution 1, remove the sub-case and update the agreements as follows:

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

If RAN2 sent to LS reply and pointed out the issue of redundant configuration, we are not sure whether RAN4 needs to open the discussion again. For the potential revision of RAN4 agreements or negative reply, RAN4 can take them into account.

We think the SCC decided by UE can be more accurate than that indicated by the network, and such carrier can represent the quality of carriers in the same band. From UE side, UE can select one suitable SCC based on both network configuration and real-time channel conditions, which leave some flexibility to UE implementation.

Furthermore, even without the new signaling indication, we think the requirements under the solution 1 of the selected SCC are not broken. At this stage, RAN4 may focus on the CSSF itself and related RAN4’s requirements, e.g., mainly the tables of CSSF outside gap in clause 9.1.5 of TS 38.133.

Observation 2: Even without the new signaling indication of a specific SCC per-band for measurement, RAN4’s RRM requirements under the solution 1 are not broken.

Proposal 2: RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update.

Conclusions

In this paper we provided our views on CSSF optimization.

Observation 1: The MO (e.g., either for serving cell or neighbor cell) is assumed to configured with report configuration based on RAN2’s spec.

Proposal 1: For solution 1, remove the sub-case and update the agreements as follows:

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

Observation 2: Even without the new signaling indication of a specific SCC per-band for measurement, RAN4’s RRM requirements under the solution 1 are not broken.

Proposal 2: RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update.

Reference

- R4-2502592	WF on RRM requirements for NR_RRM_Ph5_Part1

- R4-2502662	LS on CSSF optimization for NR RRM Phase 5