3GPP TSG-RAN WG4 Meeting #114bis											R4-2503345

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	OPPO

Title: 	On RRM requirement for on-demand SIB1

Agenda Item:	7.25.2.3

Document for:	Discussion

Introduction

In last meeting, RAN4 approved a WF on OD-SIB1 as below.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 Agreement: RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

In this document, we provide our views on the RRM requirement impacts from on-demand SIB1 in R19 NES.

Discussion

Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signalling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized Specify the corresponding core requirements, for the above features [RAN4].

Based on RAN1/2 agreements for OD-SIB1, the UE procedure of acquiring OD-SIB1 can be summarized as follows.

    - UE receives UL WUS configuration from Cell A via Cell A’s SIB, which is used for wake-up of NES cell. Triggering condition of the UL WUS can be configured to the UE in the UL WUS configuration.

    - After UE meets the WUS triggering conditions, UE shall send UL WUS to the NES cell for NES cell’s SIB1 request. The UL WUS uses PRACH formats, but not triggering any initial access to any cell.

    - After UE transmits the PRACH, UE waits for RA response (RAR) from NES Cell.

    - After successful decoding of RAR, UE is able to obtain the SIB1 from NES cell within a time window. SIB1 repetition is also performed within this window.

    - UE obtains the SIB1 from NES cell and evaluates whether UE shall camp on the NES Cell.

In legacy cell re-selection requirements, the interruption on paging reception is defined in Clause 4.2.2.6 in TS 38.133, e.g., the interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. However, for NES OD-SIB1, we need to consider additional parts at least including UL WUS requests and RAR reception. The interruption requirements may need to be extended due to UL WUS and RAR.

Proposal 1: RAN4 to discuss paging interruption due to OD-SIB1 related procedure.

Proposal 2: The interruption requirements may need to be extended due to UL WUS and RAR.

TS 38.133 v18.8.0 Clause 4.2.2.6 - 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

As stated in the WID, UL WUS is sent to NES cell, but not cell A. For OD-SIB1 related procedure, the SSB from NES cell is always available, at least in this release. Therefore, the reference timing for UL WUS transmission needs to be clarified in TS 38.133. Note that this is not the first time to clarify UL timing reference for an UL signal in idle/inactive state. The UL timing for SRS during idle mode can be referred. RAN4 is suggested to discuss whether additional conditions on Te requirements are needed

Proposal 3: Clarify UL timing related requirements/conditions for UL-WUS in RAN4.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: RAN4 to discuss paging interruption due to OD-SIB1 related procedure.

Proposal 2: The interruption requirements may need to be extended due to UL WUS and RAR.

Proposal 3: Clarify UL timing related requirements/conditions for UL-WUS in RAN4.

- References

[1] R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2