# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503359

# Wuhan, China, 7th – 11st April, 2025

Title: 	Discussion on RRM requirements for low band carrier aggregation via switching

Source: 	OPPO

Agenda item:	7.7.3

Document for:	Approval

Introduction

In RAN#106 meeting, a new WID [1] on low band carrier aggregation via switching was agreed with the following objectives:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In last meeting, RAN4 discussed the related RRM requirements and the conclusions were captured in the approved WF [2]. In this document, we will further discuss the RRM requirements based on the open issues in [3].

Discussion

Issue 1-1-1: RRM work scope - Interruption requirement - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung) - Capabilities for UEs with and without interruption (Samsung) - Alt2: RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion.

In last meeting, the RRM scope was discussed and it was agreed not to define switching delay and synchronization requirements. But whether to define interruption requirements are still open. Interruption requirements are the allowed interruption on other active serving cell when switching, but based on the following agreement in RF session, only two bands are considered for low band carrier aggregation via switching in Rel-19. So no interruption requirements are need in RRM since there are no other active serving cells.

Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

Proposal 1: RAN4 RRM not to define additional interruption or scheduling restriction.

Regarding the SDL SCC related, the following agreements were reached in last meeting:

- SDL SCC related RRM requirement - activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - de-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used. - other SDL SCell requirements Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA

For activated SDL SCC, it was agreed not to define SSB-less based requirements. For SSB based SCell operation, we understand UE can perform SSB based L3 measurement when switching to SDL, and the measurement requirements can be defined based on SMTC/SSB periodicity and the camping time on SDL SCC according to the switching pattern defined by RAN1.

For deactivated SDL SCC, since UE cannot connect to PCC and SCC simultaneously, if deactivated SCell measurement is needed, UE needs to switch to SCC for performing measurement and switch back to PCC to report. When defining the deactivated SCell measurement, the SMTC/SSB periodicity, measCycleSCell and the camping time on SDL SCC according to the switching pattern need to be considered. But since we understand the switch between FDD and SDL is to offload the DL traffic, RAN4 need to first clarify the scenarios where UE need to switch to deactivated SDL SCC.

Proposal 2: If SSB based SCell operation is used, RAN4 to define SDL activated SCell L3 measurement requirement considering SMTC/SSB periodicity and the camping time on SDL SCC according to the switching pattern.

Proposal 3: RAN4 to clarify the scenarios where UE need to switch to deactivated SDL SCC.

Proposal 4: If SDL de-activated SCell L3 measurement requirement is defined, the SMTC/SSB periodicity, measCycleSCell and the camping time on SDL SCC according to the switching pattern need to be considered.

For SDL SCell activation, the existing normal SCell activation procedure can be used as baseline, but there are some issues to be resolved. E.g., whether the known condition is needed or whether only unknown case is considered. And which Cell is used for receiving SCell activation command, if PCC can be used, the switching delay need to be considered. The ending point of SCell activation also needs to be considered since the CSI reporting can only be performed on PCC.

Proposal 5: RAN4 to consider the following issues for SDL SCell activation:

- whether the known condition is needed

- the cell used for receiving SCell activation command

- the ending point of SCell activation

For the other BFD, CBD, L1-RSRP/SINR requirements for both PCell and SCell, the existing requirements can be used as baseline. And the reasonable design is to guarantee the corresponding measurement during a certain time. But we can wait for RAN1 design on switching pattern and discuss the potential impact after that.

Proposal 6: RAN4 to discuss the impact on BFD, CBD, L1-RSRP/SINR measurement after switching pattern is defined in RAN1.

Issue 1-1-2: Assumptions for R19 LB CA via switching - (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us

For the assumption of LB CA via switching in Rel-19, considering the limited time, we suggest starting from SSB based operation and define requirements for SSB-less operation if time allows.

For the MRTD assumption, based on the WID, co-located and synchronized network deployment for both carriers are considered, and both carriers are in a single TAG. Referring to legacy MRTD requirements for CA, we think 3us can be reused.

Proposal 7: RAN4 to start from SSB based operation for Rel-19 low band CA via switching.

Proposal 8: The MRTD assumption between the two low bands can be 3us.

Conclusions

In this paper, we give our views on RRM requirements for low band carrier aggregation via switching. And the following proposals are provided.

Proposal 1: RAN4 RRM not to define additional interruption or scheduling restriction.

Proposal 2: If SSB based SCell operation is used, RAN4 to define SDL activated SCell L3 measurement requirement considering SMTC/SSB periodicity and the camping time on SDL SCC according to the switching pattern.

Proposal 3: RAN4 to clarify the scenarios where UE need to switch to deactivated SDL SCC.

Proposal 4: If SDL de-activated SCell L3 measurement requirement is defined, the SMTC/SSB periodicity, measCycleSCell and the camping time on SDL SCC according to the switching pattern need to be considered.

Proposal 5: RAN4 to consider the following issues for SDL SCell activation:

- whether the known condition is needed

- the cell used for receiving SCell activation command

- the ending point of SCell activation

Proposal 6: RAN4 to discuss the impact on BFD, CBD, L1-RSRP/SINR measurement after switching pattern is defined in RAN1.

Proposal 7: RAN4 to start from SSB based operation for Rel-19 low band CA via switching.

Proposal 8: The MRTD assumption between the two low bands can be 3us.

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2500527, Topic summary for [114][213] NR_LBCA_Sw, Apple