3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503363

# Wuhan, China, 7th - 11th, April, 2025

Source: 	Samsung

Title: 	Discussion on UE and NW behavior for non-collocated deployment

Agenda Item:	7.4.1.2

Document for:	Discussion

- Introduction

We continue to discuss the remaining open issues about new BS signalling and associated UE behavior in this paper.

The agreements of last meeting are captured in WF [1]. We try to keep this paper short as companies already have many offline, more details can be found in our paper [2] [3] of previous meeting.

- Discussion/ Conclusion

In previous meeting, we discussed UE behavior from two aspects (i.e., “switching mode” and “initial connection mode”):

- UE initial configuration/behavior on collocated or non-collocated deployment, at the initial connection mode

- UE switching behavior between collocated and non-collocated deployment

We support/propose this because our preference is that for the "switch from Type-4 to Type-1 with 6/8 L", UE first switch from Type-4 to Type-1 with 4L, then do MIMO layer re-config to 6/8 L → This is like a "2-step" approach.

While companies may prefer "1-step" approach, by using maxMIMO-layers=6 or 8 to directly conduct UE to switch, if understand correctly.

By reviewing again Rel-18 RAN2 and RAN4 spec and recalling what had been discussed in Rel-18, we found that there is actually no description on the UE behavior when maxMIMO-layers＝4 in both RAN2 and RAN4. RAN2 spec is excerpted as below for easy reference. For RAN4 spec, we recalled we did have some description in the initial Rel-18 CRs for maxMIMO-layers＝4, but failed to reach agreement thus relevant description were all removed in the end, basically left a hole in RAN4 spec.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

To avoid same debate in Rel-19, we think same handling as in Rel-18 can be adopted in Rel-19, meaning that at least in both Rel-19 RAN2 and RAN4 spec, no description on the UE behavior when maxMIMO-layer＞4 （=6/8）. With that, both "1-step approach" and "2-step approach" are allowed. And adopting one unified solution for both “switching mode” and “connected more” is also acceptable for us.

With above, we propose:

Proposal 1: Confirm the following understanding for the usage of maxMIMO-layers and the new Rel-19 NW signaling:

1）maxMIMO-layers ＞4, it is collocated

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, follow Rel-18 specification →This was already agreed in previous meeting

Note: More specific, “If Rel-19 BS signaling IE is absent” intends for legacy NW or Rel-19 NW without Type-4 feature, “If Rel-19 BS signaling IE is present” intends for Rel-19 NW with Type-4 feature.

After some offline discussion, we found it might still be difficult to reach agreement in RAN4 as companies have discrepancies since Rel-18. If during the meeting, companies still fail to reach any agreements on this aspect, we suggest to send LS to RAN2 to ask them finalize this work as in Rel-18 it is also RAN2 decided it. We see no point to continue this NW signaling discussion in May meeting in RAN4.

Proposal 2: In case it is impossible to reach agreement during this meeting, send LS to RAN2 ASAP asking them to make decision for the Rel-19 signaling design.

- 3. Reference

[1] R4-2502924, WF on UE requirements for non-collocated scenario, KDDI, RAN4#114

[2] R4-2418303, Discussion on UE and NW behavior for non-collocated deployment, Samsung, RAN4#113

[3] R4-2500807, Discussion on UE and NW behavior for non-collocated deployment, Samsung, RAN4#114