3GPP TSG-RAN WG4 Meeting #114bis	R4-2503364

Wuhan, China, 7th– 11th, Apr, 2025

Title:	LS to RAN2 on Type-4 for non-collocated deployment

Response to:

Release:	Rel-19

Work Item:	NonCol_intraB_ENDC_NR_CA_Ph2-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Yuanyuan Zhang

E-mail Address:  tina55.zhang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 discussed the applicable UE requirements, new UE indication and new NW signalling for Type-4a/4b in previous meetings for which the following agreements were reached. RAN4 respectfully ask RAN2 to devise necessary UE capabilities and NW signalling accordingly.

UE capability Type

- Type-4a is not considered for NR-CA

- Both Type-4a and Type-4b are considered for EN-DC

UE requirements:

- Reuse the power imbalance and REFSENS relaxation of Type-2

- Reuse the MRTD and MTTD requirements of Type-2

UE indication:

- Define one capability for NR CA and one capability for EN-DC

- For EN-DC, Type-4a and Type-4b capability are differentiated by legacy capability, such as MIMO-CapabilityDL-r10 on E-UTRA carrier

- If UE reports Type-4 capability, Type-2 capability shall be deemed as support by default, and UE still needs to report Type-2 capability if UE supports it in order to keep backward compatibility (i.e., for legacy BS)

NW signalling:

- When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed

- When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed

- When maxMIMO-layers=4, new NW signalling is needed, and the details are up to RAN2

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to devise necessary UE capabilities and NW signaling.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #115	18 - 24 May, 2025		                   Malta

TSG-RAN WG4 Meeting #116	25 – 29 Aug, 2025		                  Indian