3GPP TSG-RAN WG4 Meeting # 114bis      	R4-2503375

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.12.6

Source: 	CMCC

Title: 			 Discussion on demodulation and CSI requirements for Rel-19 ATG

Document for:	Discussion

- Introduction

The following objectives has been included in the latest WID on Enhancements for Air-to-ground network for NR [1]:

The performance part of the work item includes: - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA. - Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna. - Specify demodulation requirements for support of DL MIMO for UE with limited test cases. - Specify conformance testing and conformance requirements for BS.

According to the work plan [2], RAN4 will start to discuss the demodulation/CSI performance requirements for ATG from this meeting considering the DL CA, DL MIMO and UL MIMO feature. In this contribution, we give our views on test scope and simulation assumption.

- ATG UE DL CA demodulation requirement

  - Test rules

In Rel-16, UE CA demodulation requirement has been introduced with following test setup and test applicability rules:

For UEs supporting different CA configurations and bandwidth combination sets, the tested CA configuration are selected as follows:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C or CA_N | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Besides, RAN4 introduced performance requirement for all bandwidth configuration for FDD 15kHz and TDD 15k/30kHz, independent of CA configurations and bandwidth combination sets. UE pass the test case with following test applicability rule to minimize the test effort:

Table 5.1.1.7.2-1: Applicability and test rules for CA UE demodulation tests

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_N, CA_AX | Table 5.1.1.7.2-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_N, CA_AX | Table 5.1.1.7.2-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table 5.1.1.7.2-2 | Largest aggregated CA bandwidth combination | TDD CC if supported, otherwise FDD CC |
| FDD 15 kHz + TDD 15 kHz(NOTE 2) | CA_AX | Table 5.1.1.7.2-2 | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 15 kHz + TDD 30 kHz (NOTE 3) | CA_AX | Table 5.1.1.7.2-2 | Largest aggregated CA bandwidth combination | 15 kHz CC if supported, otherwise 30 kHz CC |
| NOTE 1: In case CA_AX with different number of X is supported then one or two CA configurations are selected based on procedure from Table 5.1.1.7.2-2. NOTE 2: These scenarios are only tested for UEs which are not verified with Test 3 in Clause 5.2A.2.1 and 5.2A.3.1. NOTE 3: These scenarios are only tested for UEs which are not verified with Test 2 in Clause 5.2A.2.1 and 5.2A.3.1. |  |  |  |  |

In high level, we think the above CA configuration selection rule and test applicability rule can be reused for Rel-19 ATG CA, but some refinement need to be done with the consideration of ATG specific feature, including:

- Only intra-band contiguous CA and inter-band CA are included in WID

- For FDD 15kHz +TDD 30kHz case, (e.g., Example band combination: CA_n3A-n39A), FDD CC is configured as PCC

- Only FDD 15kHz and TDD 30kHz are supported for Rel-18 single carrier requirement

After the refinement, the CA configuration selection rule and test applicability rule for Rel-19 ATG CA shall be:

For UEs supporting different CA configurations and bandwidth combination sets, the tested CA configuration are chose as follows:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

The test applicability rules are defined as follows:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

In addition, RAN4 shall introduce performance requirement for all bandwidth configuration for FDD 15kHz and TDD 30kHz, independent of CA configurations and bandwidth combination sets.

Proposal 1: RAN4 shall introduce performance requirement for all bandwidth configuration for FDD 15kHz and TDD 30kHz.

Proposal 2: Define the CA configuration selection rules as:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Proposal 3: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

  - Test setup

TDD pattern

Since there are still some potential maintenance issues about the new TDD pattern we introduced in Rel-18, We propose to use legacy TDD pattern for both FDD and TDD.

Proposal 4: Use legacy TDD pattern for both FDD and TDD.

MCS&Rank

Since MIMO can be supported in Rel-19, and only conducted test will be applied for ATG UE, which means the UE antenna type is transparent in the test, we think the Rank 2 can be used for defining the CA requirement.

As for the MCS, we propose to use 16QAM MCS 13 and 64QAM MCS 22 for initial simulation.

Proposal 5: For CA requirement, use Rank 2 as the starting point.

Proposal 6: For CA requirement, use 16QAM MCS 13 and 64QAM MCS 22 for initial simulation.

Others

- In addition, we think following test setups agreed in R18 can be reused for the new test cases which introduced in R19.

- Channel model: AWGN+220Hz for FDD 15kHz，AWGN+500Hz for TDD 30kHz

- Antenna configuration: 2T2R and 2T4R

- Test metric:  70% of max TP

- Receiver: MMSE-IRC

- For the other test parameters, we think the configuration in Table 5.2A-1/2/3 in TS 38.101-4 can be reused.

Proposal 7: Following test setups agreed in R18 can be reused:

- Channel model: AWGN+220Hz for FDD 15kHz，AWGN+500Hz for TDD 30kHz

- Antenna configuration: 2T2R and 2T4R

- Test metric:  70% of max TP

- Receiver: MMSE-IRC

Proposal 8: For the other test parameters, the configuration in Table 5.2A-1/2/3 in TS 38.101-4 can be reused.

- DL MIMO related UE demodulation requirement

Antenna configuration

- 2T2R and 2T4R configuration from Rel-18 can be reused.

- In addition, due to AWGN channel model is assumed in ATG scenario, the cross polarized antenna configuration is necessary to achieve 2 MIMO layer. We think the low correlation can be taken as the starting point.

Proposal 9: 2T2R and 2T4R with X-pol low can be the baseline.

TDD pattern

Since there are still some potential maintenance issues about the new TDD pattern we introduced in Rel-18, We propose to use legacy TDD pattern for both FDD and TDD.

Proposal 10: Use legacy TDD pattern for both FDD and TDD.

Rank/MCS

Since the spatial independence comes from antenna polarization, then only Rank 2 can be supported. Regarding MCS, the assumptions in Rel-18 can be the starting point, and can be further refined according to the simulation results, including 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24.

Proposal 11: For Rank, cover Rank 2 for MIMO performance verification.

Proposal 12: For initial simulation, consider 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24.

For other simulation assumptions, we think the agreements from Rel-18 can be reused, including:

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| Propagation channel |  |  | AWGN+220Hz doppler | AWGN+500Hz doppler |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

Proposal 13: For other simulation assumptions, the agreements from Rel-18 can be reused, including:

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| Propagation channel |  |  | AWGN+220Hz doppler | AWGN+500Hz doppler |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

- DL MIMO related UE CSI requirement

PMI requirement

For ATG MIMO, the spatial diversity comes from antenna polarization, instead of channel characteristics. Hence, we think no need to introduce the PMI requirement.

Proposal 14: No need to introduce PMI requirement.

RI requirement

We would like to investigate the feasibility of introducing the RI requirement for ATG MIMO. Except the antenna configuration and propagation channel, we think the other test configuration from legacy can be reused. The SNR and  shall be decided by simulation results.

Proposal 15: Investigate the feasibility of introducing the RI requirement.

Proposal 16: For antenna configuration and propagation channel, reuse the configuration from DL MIMO demodulation test. For other test parameters, reuse the configuration from legacy.

Proposal 17: Decide the SNR and  by simulation results.

- UL MIMO related BS Demodulation requirement

Test scope

In Rel-18, the legacy PUSCH test cases in Clause 8.2.1, 8.2.2 and 8.2.3 with 1Tx test cases are applied for ATG, and incremental PUSCH test cases in Clause 8.2.14 were introduced.

Similar methodology can be reused for R19 UL MIMO test cases, the legacy PUSCH test cases in Clause 8.2.1, 8.2.2 and 8.2.3 with 2Tx 2 layer test cases will be applied for ATG, and incremental PUSCH 2 layer test cases will be introduced.

Proposal 18: The legacy PUSCH test cases in Clause 8.2.1, 8.2.2 and 8.2.3 with 2Tx 2 layer will be applied for ATG, and incremental PUSCH 2 layer test cases will be introduced.

Antenna configuration for incremental test cases

As stated in WID, it is feasible for UE with dual-polarization antenna to support UL MIMO. Therefore, 2T2R, 2T4R, and 2T8R with XP low correlation can be taken as the starting point.

Proposal 19: For incremental test case, take 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point.

Propagation condition for incremental test cases

Same as Rel-18, AWGN+200Hz can be used for FDD, and AWGN+500Hz can be used for TDD.

For the specific channel model, we think the static propagation condition from TS38.101-4 can be taken as the reference.

Proposal 20: For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference.

SCS/CBW for incremental test cases

Same SCS and CBW from Rel-18 can be reused, which are 5MHz for FDD 15kHz and 10MHz for TDD 30kHz.

Proposal 21: For incremental test case, use 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS.

Rank/MCS for incremental test cases

Since the spatial independence comes from antenna polarization, then only Rank 2 can be supported. Regarding MCS, we propose to consider 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation.

Proposal 22: For incremental test case, consider Rank 2, 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation.

DMRS configuration for incremental test cases

Since Rank 2 is considered here, 2 port DMRS shall also be supported.

Proposal 23: For incremental test case, DMRS port 2 {0,1} for UL MIMO.

TPMI index for 2Tx two-layer spatial multiplexing transmission for incremental test cases

TPMI index 0 shall be used for two layer and two ports transmission.

Proposal 24: For incremental test case, TPMI index 0 for UL MIMO.

- Conclusion

Based on the discussion above, the following proposals are concluded.

DL CA

Proposal 1: RAN4 shall introduce performance requirement for all bandwidth configuration for FDD 15kHz and TDD 30kHz.

Proposal 2: Define the CA configuration selection rules as:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Proposal 3: Define the test rules as:

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| FDD 15 kHz + FDD 15 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| TDD 30 kHz + TDD 30 kHz | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| FDD 15 kHz + TDD 30 kHz | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Proposal 4: Use legacy TDD pattern for both FDD and TDD.

Proposal 5: For CA requirement, use Rank 2 as the starting point.

Proposal 6: For CA requirement, use 16QAM MCS 13 and 64QAM MCS 22 for initial simulation.

Proposal 7: Following test setups agreed in R18 can be reused:

- Channel model: AWGN+220Hz for FDD 15kHz，AWGN+500Hz for TDD 30kHz

- Antenna configuration: 2T2R and 2T4R

- Test metric:  70% of max TP

- Receiver: MMSE-IRC

Proposal 8: For the other test parameters, the configuration in Table 5.2A-1/2/3 in TS 38.101-4 can be reused.

DL MIMO

Proposal 9: 2T2R and 2T4R with X-pol low can be the baseline.

Proposal 10: Use legacy TDD pattern for both FDD and TDD.

Proposal 11: For Rank, cover Rank 2 for MIMO performance verification.

Proposal 12: For initial simulation, consider 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24.

Proposal 13: For other simulation assumptions, the agreements from Rel-18 can be reused, including:

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| Propagation channel |  |  | AWGN+220Hz doppler | AWGN+500Hz doppler |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

Proposal 14: No need to introduce PMI requirement.

Proposal 15: Investigate the feasibility of introducing the RI requirement.

Proposal 16: For antenna configuration and propagation channel, reuse the configuration from DL MIMO demodulation test. For other test parameters, reuse the configuration from legacy.

Proposal 17: Decide the SNR and  by simulation results.

UL MIMO

Proposal 18: The legacy PUSCH test cases in Clause 8.2.1, 8.2.2 and 8.2.3 with 2Tx 2 layer will be applied for ATG, and incremental PUSCH 2 layer test cases will be introduced.

Proposal 19: For incremental test case, take 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point.

Proposal 20: For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference.

Proposal 21: For incremental test case, use 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS.

Proposal 22: For incremental test case, consider Rank 2, 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation.

Proposal 23: For incremental test case, DMRS port 2 {0,1} for UL MIMO.

Proposal 24: For incremental test case, TPMI index 0 for UL MIMO.

- Reference

[1] RP-243285, Revised WID on Enhancements for Air-to-ground network for NR, CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom.

[2] R4-2404692, (NR_ATG_enh-Core) Work plan on Rel-19 enhancements for ATG.