3GPP TSG-RAN WG4 Meeting # 114-bis                                R4-2503400

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.26.2.1

Source: 	CMCC

Title: 	Discussion on LP-WUS UE system parameters requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#114, the LP-WUS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about system parameters.

- Discussion

### 2.1 LP-WUR types

Based on the conclusions of RAN4 last meeting the following agreement has been reached.

Agreement: - Define two set of RF requirements as the baseline, considering the following aspects including - Noise figures - Interference performance - Modulation - Consider to merge two sets of requirements into one if the requirements are close to each other

In our opinion, LP-WUS types for both OOK-based and OFDM-based are needed to be defined. The main difference between the two types of LP-WUS is power consumption. Therefore, it is recommended to use NF as the definition to distinguish between the two types of LP-WUS.

Proposal 1: Use the following table as baseline for the definitions of LP-WUR Types.

Table 1. The definitions of LP-WUR Types

| LP-WUR type | Type description |
| --- | --- |
| OOK-based | The LP-WUR is designed for ultra-low power consumption, assuming a noise figure [X] dB higher than the MR. |
| OFDM-based | The LP-WUR is designed for moderate power consumption, assuming a noise figure [Y] dB higher than the MR. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

### 2.2 Channel bandwidth

The following agreement has been reached.

Agreement: [Transmission bandwidth configuration] for LP-WUS/WUR signal is needed to specify.

To facilitate the design of LP-WUR and its RF implementation, it is essential to explicitly specify in the RAN4 specification that the LP-WUR operates on 11 RBs. Therefore, we propose that the RAN4 RF specification should clearly state the number of RBs allocated for LP-WUR operation.

Proposal 2: Transmission bandwidth configuration for LP-WUS/WUR signal is needed to specify.

# 3. Conclusions

Proposal 1: Use the following table as baseline for the definitions of LP-WUR Types.

Table 1. The definitions of LP-WUR Types

| LP-WUR type | Type description |
| --- | --- |
| OOK-based | The LP-WUR is designed for ultra-low power consumption, assuming a noise figure [X] dB higher than the MR. |
| OFDM-based | The LP-WUR is designed for moderate power consumption, assuming a noise figure [Y] dB higher than the MR. |
| NOTE 1: The LP-WUR types are expected to support OOK and/or OFDM-based LP-WUS signals, depending on the indicated UE capability. |  |

Proposal 2: Transmission bandwidth configuration for LP-WUS/WUR signal is needed to specify.

# 4. Reference

- R4-2503028, WF on UE RF requirements for LP-WUS