3GPP TSG-RAN WG4 Meeting # 114-bis                              R4-25xxxxx

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.26.2.3

Source: 	CMCC

Title: 	Discussion on LP-WUS UE RF Rx requirements of IBB, OBB, intermodulation, spurious emissions and others

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In this contribution, we focus on discuss of UE LP-WUS RF requirements including IBB, OBB, intermodulation, spurious emissions and others.

- Discussion

2.1 IBB and OBB requirements

Issue 3-1-1: IBB, OBB requirements Way forward: - Same interference level as MR - FFS at least for one set of Rx requirements. - FFS on wanted signal in blocking test. Issue 3-1-2: detailed table for test parameters of IBB and OBB test case Way forward: - Encourage companies to provide input and analysis on IBB and OBB requirements Issue 3-2-2: Maximum input level requirements for FR1 LP-WUR Way forward: - FFS. Issue 3-2-3: Maximum input level test case for FR1 Way Forward: - Further check and decide whether the max. input power level is specified for all channel bandwidths. Issue 3-2-4: spurious response requirements for FR2 Way Forward - Further check next meeting on whether need to specify spurious response requirements for FR2 LP-WUR.

We have reached the agreement that the same interference level as for the main radio is assumed for LP-WUR. Based on this assumption, for the in-band blocking requirement, the IBB requirement for legacy UE could be reused.

Proposal 1: In-band blocking requirement for legacy UE could be reused.

There are three types of receiver architectures for LP-WUR: architecture with RF envelope detection, heterodyne architecture with IF envelope detection, and homodyne/zero-IF architecture with baseband envelope detection. The Band Pass Filter is all included in the diagram below, so the OBB requirement for legacy UE in TS 38.101-1 could be reused.

Proposal 2: the OBB requirement for legacy UE in TS 38.101-1 could be reused.

2.2 Intermodulation

For intermodulation requirements, considering the difference between signals, it is better to define two sets of requirements for the OFDM signal and OOK signal. For the OFDM signal, the intermodulation requirements for legacy UE could be reused. Given the difference in the structure of the receiver of the OOK signal, new intermodulation requirements need to be specified for the OOK signal.

Proposal 3: specify two sets of requirements for OFDM signal and OOK signal.

- The requirements in TS 38.101-1 for legacy UE could be reused for OFDM signal.

- New intermodulation requirements need to be specified for the OOK signal.

2.3 Maximum input level

We concur with using the legacy -25dBm as the maximum input level. For a 5MHz channel bandwidth with a 30kHz SCS, the LP-WUS occupies all the allocated 11 RBs (fully allocated). Given that RAN4 is tasked with defining minimum requirements, typically based on the worst-case assumption, the existing specification of -25dBm as the maximum mean power received at the UE antenna port, applicable to the entire allocated RBs, is deemed appropriate.

Proposal 4: Define maximum input level as -25dBm for LP-WUS.

# 3. Conclusions

In this contribution, IBB, OBB, intermodulation, and spurious emissions are discussed with following observations and proposals.

Proposal 1: In-band blocking requirement for legacy UE could be reused.

Proposal 2: the OBB requirement for legacy UE in TS 38.101-1 could be reused.

Proposal 3: specify two sets of requirements for OFDM signal and OOK signal.

- The requirements in TS 38.101-1 for legacy UE could be reused for OFDM signal.

- New intermodulation requirements need to be specified for the OOK signal.

Proposal 4: Define maximum input level as -25dBm for LP-WUS.

# 4. Reference

- R4-2503028, WF on UE RF requirements for LP-WUS