3GPP TSG-RAN WG4 Meeting # 114-bis                                R4-2503403

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.23.2.2

Source: 	CMCC

Title: 	Discussion on existing Tx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements.[1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACLR and OBUE

Following capture the agreements from TR 38.858 for ACLR requirements:

For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.

Since whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS is still FFS. that no conclusions were reached in the previous meeting, we have listed the conclusions of the RAN4#112bis meeting for reference.:

WF: - For FR2-1 WA SBFD BS, re-use the current FR2-1 WA BS in-band blocking requirements. - For FR1 WA BS, try to define the in-band blocking requirement, FFS on following options. - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario. - Option A: 20% - Option B: 50% - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. - Option 3: Define in-band blocking requirements based on co-existence simulation results with GS 5%. - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%. - For FR1 MR SBFD BS, FFS.

Our think that the distinction between Option 1 and Option 2 lies in whether to define the RF requirements for the coexistence scenario of SBFD-SBFD in the current version. Given the potential coexistence scenarios of SBFD-SBFD, we think it necessary to establish RF requirements. However, considering the current deployment status of this technology and the fact that SBFD can be further explored as a 6G day one future research topic, we also find Option 1 acceptable.

Proposal 1: Additional requirements for SBFD-SBFD scenario should be defined in current or future WI.

While recognizing the distinction between Option 1 and Option 2, we also note that the wording in Option 1 is "if regulator make a decision." However, in practice, before a regulator deploys a certain technology, they typically request operators to provide RF requirements as a reference. Without reference to RF requirements of SBFD-SBFD scenario, regulatory is highly unlikely to consider the deployment of new technology. Therefore, we suggest modifying Option 1 to "If operators make a decision."

Proposal 2: Option 1 should be modified to:

Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If operators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

Given the difficulty in defining the co-location ACLR requirement, it is advisable to initially focus on discussing scenarios where co-location is not a factor.

Proposal 3: Prioritize discussing the scenarios under non-colocation case first.

Proposal 4: The legacy BS requirement could be considered as the starting point.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Additional requirements for SBFD-SBFD scenario should be defined in current or future WI.

Proposal 2: Option 1 should be modified to:

Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If operators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

Proposal 3: Prioritize discussing the scenarios under non-colocation case first.

Proposal 4: The legacy BS requirement could be considered as the starting point.

# 4. Reference

[1] R4-2502310, Way Forward for [114][306] NR_duplex_evo_General