3GPP RAN WG4 Meeting #114bis	R4-2503410

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on testing issues for CSI compression two-sided models

Source: 	CATT

Agenda item:	7.20.2

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 discussed testability and interoperability issues for two-sided model in Rel-19 AI/ML for NR air interface and agreements were captured in [1]. But some issues need further investigating and discussing. In this contribution, we will continue to discuss those issues based on our simulation results and observations.

Discussion

For testability study of two-sided model for CSI compression, RAN4 now have four candidate solutions which are

- Option 3 track 1: Fixed test decoder trained with own dataset

- Option 3 track 2: Fixed test decoder trained with mixed dataset

- Option 4a: dataset based solution

- Option 4b: reference based solution

After RAN4#114, email discussions in reflector provided more detailed guidance for further study. In this contribution, we will share some simulation results and discuss the feasibility of each solution above.

Option 3 track 1

In way forward [1], the guidance for option 3 track 1 is:

How to continue with track 1 until RAN4#114bis Companies should provide quantized and unquantized models according to the advice above, in particular Samsung, CATT and Huawei. Each company is encouraged to train own encoder using the fixed decoder and their own dataset, and then test with every companies datasets, in order to check for consistency in results

We followed this guidance and trained own encoder with one of the fixed decoders, CATT’s decoder. Then the trained own encoder was tested with other companies’ datasets. The results are provided in Figure 1:

Figure 1-SGCS of Option 3 track 1

In Figure 1, the baseline performance is the SGCS of trained own encoder and fixed test decoder pair on the test dataset. We observe that our own encoder-fixed decoder pair has a low performance on MTK dataset, while it has comparable performances on other four companies’ datasets within a range of 3.78% for un-quantized model and of 4.56% for the quantized model.

Observation 1: In Option 3 track 1, SGCS spans from 0.6868 to 0.7246 for un-quantized model and from 0.6557 to 0.7013 for quantized model.

Observation 2: Option 3 track 1 is feasible when characteristic of training datasets and test datasets are aligned.

Option 3 track 2

How to continue with track 2 until RAN4#114bis: Once the frozen decoder is selected, companies are invited to train their own encoders using the mixed dataset or/and private dataset and test using the mixed test dataset

For Option 3 track 2, Samsung decoder was chosen as the test decoder, and we trained our own encoder with the chosen decoder and the mixed dataset. Afterwards, this encoder was tested with the frozen test decoder using mixed dataset. In addition, we also simulated the performance changes when encoders have different structures, e.g., different number of ResNet blocks. The simulation results are given in Figure 2:

Figure 2-SGCS of Option 3 track 2

Observation 3: In Option 3 track 2, SGCS is 0.7206 for un-quantized model and 0.713 for quantized model.

Observation 4: CNN encoders with different number of ResNet blocks have slightly different SGCS.

Option 4a/4b

Regarding the working procedures of Option 4a and 4b, RAN4 have provided detailed steps as guidance:

Option 4a(Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - If multiple datasets are selected (as was the case of Option 3 track 1), the subsequent steps of the feasibility procedure will be applied on each one of the selected datasets separately - Step 5: Label selected dataset with encoder input/output using encoder corresponding to the selected test decoder from option 3. - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc.). - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - FFS what is the dataset assumed for the training - Step 6: Company trains “own encoder(s)” with “own decoder(s)” from step 5 - FFS what is the dataset assumed for the training - Step 7: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc.). - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

In Option 4a, Samsung encoder is used to label the mixed dataset. But the dataset used for training own encoder is not specified, therefore either mixed dataset or own dataset can be used.

In Option 4b, it is okay to use Samsung encoder. And similar to the situation in Option 4a, the dataset used for training own decoder and own encoder is not specified, therefore either mixed dataset or own dataset can be used.

In Figure 3 and Figure 4, we illustrate the steps of Option 4a/4b in a high-level way to clearly show their similarities and differences and how to conclude their feasibility respectively:

Figure 3-Illustation of Steps of Option 4a/4b

Figure 4-Illustation of feasibility check of Option 4a/4b

Figure 3 clearly shows that the option 4a and option 4b will be essentially the same, i.e., equivalent to Option 4a, if mixed dataset is used to train own encoder in both options and own decoder in Option 4b. Furthermore, in this case both encoder and dataset, i.e., a common dataset, need specifying in spec, which is not aligned with the initial motivation of Option 4b.

Observation 5: Option 4a and 4b will be essentially the same, i.e., equivalent to Option 4a, if mixed dataset is used to train own decoder in Option 4b and own encoder in both options.

Based on the above observation, we firstly simulated the Option 4a with only the mixed dataset used to train our own encoder. In addition, we also tried to train our own encoder with own dataset in step 6-2 of Option 4a, which allows UE vendors to use diverse datasets that help generalize their encoders as much as possible. The simulation results of Option 4a with both mixed dataset and own dataset used to train own encoder are shown in Figure 5:

Figure 5- SGCS of Option 4a with mixed dataset and own dataset for training own encoder

The baseline in Figure 5 is the SGCS of Samsung encoder and own decoder pair tested with the mixed dataset.

Observation 6: In Option 4a, SGCS is 0.7411 for un-quantized model and 0.6939 for quantized model when the mixed dataset is used; 0.7265 and 0.6722 respectively when own dataset is used.

Observation 7: Compared with SGCS with the mixed dataset used to own encoder, a performance degradation, around 2%, is observed when own dataset is used.

Next we would like to discuss how to process with Option 4b when it is different from Option 4a. The idea of Option 4b is to capture an encoder in the spec without extra information included, and UE vendors and TE vendors can design and train their own encoder-decoder pair with this specified encoder independently. Then the expected situation is that UE encoder and TE decoder pair can work properly.

As shown in Figure 3, if RAN4 use the mixed dataset to perform the feasibility study and it is concluded this option is feasible, then a common dataset will be needed in spec. Thus, we suggest using own dataset to train own decoder and own encoder and if the performance check against feasibility criteria passes, then RAN4 can conclude that Option 4b is feasible without dataset needed in spec. In Figure 6, we provide our simulation results of Option 4b with own dataset used to train own decoder and own encoder.

Figure 6- SGCS of Option 4b with own dataset to train own encoder and own decoder

In Figure 6, the baseline is the SGCS of Samsung encoder and own decoder pair tested with own dataset.

Observation 8: In Option 4b, SGCS is 0.749 for un-quantized model and 0.6841 for quantized model when own dataset is used.

Proposal 1: Use own dataset to train own decoder and own encoder in Option 4b.

What would be standardized for option 3

There was discussion on what would be standardized for option 3 in last meeting and RAN4 had the below agreements:

By definition, a test decoder is standardized for option 3. The need to define a reference encoder was discussed and the following agreement reached Agreement: - To verify the performance of CSI compression test decoder for calibration of test equipment, agree to define the reference encoder including encoder structure and parameters in RAN4 - FFS on whether to specify it in TS or TR, and capture the conclusion in the TR conclusion part - FFS on criterion for selection of reference encoder - FFS on decision whether RAN4 or RAN5 will specify it Way forward: There was also a discussion on whether to capture a training dataset, but no agreements. Some open questions from the discussion (not agreements): - Whether it is useful to capture a training dataset - Where and how the training dataset would be captured (TR, TS, some other public place…)

As the simulation results shown in Figure 3, 5, and 6, CNN encoders with different number of ResNet blocks have a slight different SGCS performance. And a larger SGCS performance difference can be expected if UE vendors use an encoder with backbone different from test decoder. Therefore it would be beneficial to capture a reference encoder. Regarding the document to capture this reference encoder, we suggest ruling out TS and considering this issue when defining performance requirements in WI phase, if we will have one.

On how to select reference encoder, we think it would be quite straightforward. Since if a test decoder will be specified, the corresponding encoder can be selected as the reference encoder.

Proposal 2: In Option 3, the encoder corresponding to the test decoder is selected as reference encoder.

Proposal 3: FFS where to capture the reference encoder when defining performance requirements in WI, if exists.

Another aspect is whether to capture a training dataset. Currently RAN4 have two tracks for Option 3:

- Option 3 track 1: Fixed test decoder trained with own dataset

- Option 3 track 2: Fixed test decoder trained with mixed dataset

In track 1, test decoder is trained with own dataset. As Figure 1 shows, even RAN4 tried to align datasets in multiple meetings, including simulation assumptions and pre-processing alignments, it is very challenging to completely align datasets provided by companies. And when datasets have non-negligible differences, SGCS performance may degrade significantly.

Similarly in track 2, even the specified test decoder is trained with a mixed dataset, if a UE vendor could not generate a dataset that has a very similar characteristic to that of the mixed dataset, and its encoder may not work well with the test decoder.

Thus, for both track 1 and 2, it would be helpful to capture the training dataset. Considering the size and type of dataset, storing it in a dedicated folder can be considered.

Observation 9: For both Option 3 track 1 and 2, it would be helpful to capture the training dataset.

Proposal 4: Storing the training dataset in a dedicated folder can be considered for both Option 3 track 1 and 2.

Conclusions

This contribution discussed some testability and interoperability issues for AI/ML enabled CSI compression and some observations and proposals are provided:

Observation 1: In Option 3 track 1, SGCS spans from 0.6868 to 0.7246 for un-quantized model and from 0.6557 to 0.7013 for quantized model.

Observation 2: Option 3 track 1 is feasible when characteristic of training datasets and test datasets are aligned.

Observation 3: In Option 3 track 2, SGCS is 0.7206 for un-quantized model and 0.713 for quantized model.

Observation 4: CNN encoders with different number of ResNet blocks have slightly different SGCS.

Observation 5: Option 4a and 4b will be essentially the same, i.e., equivalent to Option 4a, if mixed dataset is used to train own decoder in Option 4b and own encoder in both options.

Observation 6: In Option 4a, SGCS is 0.7411 for un-quantized model and 0.6939 for quantized model when the mixed dataset is used; 0.7265 and 0.6722 respectively when own dataset is used.

Observation 7: Compared with SGCS with the mixed dataset used to own encoder, a performance degradation, around 2%, is observed when own dataset is used.

Observation 8: In Option 4b, SGCS is 0.749 for un-quantized model and 0.6841 for quantized model when own dataset is used.

Proposal 1: Use own dataset to train own decoder and own encoder in Option 4b.

Proposal 2: In Option 3, the encoder corresponding to the test decoder is selected as reference encoder.

Proposal 3: FFS where to capture the reference encoder when defining performance requirements in WI, if exists.

Observation 9: For both Option 3 track 1 and 2, it would be helpful to capture the training dataset.

Proposal 4: Storing the training dataset in a dedicated folder can be considered for both Option 3 track 1 and 2.

References

[1] R4-2503953, WF on AI/ML air interface study, Ericsson, RAN4#114.