3GPP RAN WG4 Meeting #114bis	R4-2503411

Wuhan, CN, April 7th – April 11th, 2025

Title: 	Discussion on impacts of AIML RRM measurement prediction on RRM requirements

Source: 	CATT

Agenda item:	7.21.2.1

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 discussed the RRM impacts of AI/ML based mobility and the agreements were captured in the WF [1]. In this contribution, we will further discuss the open issues listed in [2] and present our views.

Discussion

General and scope

Radio Link Failure (RFL) prediction use case

According to RAN2 agreements [3], AI/ML based radio link failure prediction will not be studied in Rel-19, and therefore RAN4 do not need to study this use case either.

Agreements on priority 1. RLF prediction will not be studied in Rel-19.

Proposal 1: RAN4 do not study radio link failure use case in Rel-19.

In the rest of contribution, we will focus on RRM measurement prediction use case.

Relative RSRP Prediction Accuracy

In last meeting, RAN4 discussed the definition of relative RSRP prediction accuracy and the following agreement was achieved:

RAN4#114 Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

Regarding the 1st sub-bullet, we do not think such limitations are needed for a definition. And considering the current scenarios and sub-use cases, this definition can apply when beams come from the same cell or different cells. For example, beams can come from different cells in scenario 3 sub-use case 1.

Proposal 2: Beams in the definition of relative RSRP prediction accuracy can come from the same or different cells.

The 2nd sub-bullet is about the ground truth definition for RSRP accuracy. In last meeting, RAN4 spent a lot of time on this issue and some agreements were achieved:

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2. Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

RAN4 agreed to discuss ground truth for FR1 and FR2 separately. And for FR2, the ground truth is the reported L3 RSRP measurement result under sufficient high SNR in FR2, which is also used in AI/ML based beam management use case. Thus, there is no need to repeat the same discussion on this issue in this study item. For FR1, two alternatives are given and further discussions are required, taking the above listed aspects into account.

RAN4 already agreed that conducted testing will be adopted in FR1, where TE is able to have a full knowledge of ideal received power at UE for each corresponding beam considering that channel is simulated by TE. Specifically, with beam consolidation/selection method and L3 filtering coefficients, which are configured by TE, TE can calculate ideal L3 cell-level measurement results based on the mentioned aspects. Hence, ground truth definition for RSRP accuracy can base on reception power which is known to TE.

Some argued that TE does not know the exact timing when UE performs measurement or prediction and channel condition constantly changes, which makes it unreasonable to use TE transmitted power or UE received power as ground truth. In our understanding, RAN4 will configure reference signals, e.g., SSB, in tests, and SSB occasions occupy specific time/frequency resources. Though TE cannot know the exact timing instance where UE performs measurement or prediction, TE can still approximately derive the ground truth by using the SSB transmitted power and channel information. Therefore we support Alt 1.

On Alt2, a main concern is that UE may cheat in tests since ground truth is the value reported by UE. And with a same test configuration, UEs with different capabilities may provide different ground truths, and possibly they all can pass RAN4 tests. Apparently, in this case RAN4 test cannot fulfil its purpose anymore.

Proposal 3: For FR1, ground truth is UE received power derived by TE. For FR2, ground truth is reported value and RAN4 wait for progress on AI/ML BM and reuse the agreement.

For the 3rd sub-bullet, at least the reported RSRP of cell2/beam2 can be predicted results, and the necessity of a combination of predicted and measured samples needs clarifying. And as we stated at the beginning of this issue, we do not think limitations are needed for this definition. Any applicability conditions can be discussed for specific use cases.

Proposal 4: The reported RSRP of cell2/beam2 can be predicted results and proponents to clarify the necessity of a combination of predicted and measured samples.

Proposal 5: It is not necessary to add limitations for the definition of relative RSRP prediction accuracy. Applicability conditions can be discussed for specific use cases.

Strong Cell Prediction Accuracy

One proposal was to define prediction accuracy requirement for strong cell prediction and candidate metrics were provided:

- Proposal 1: RAN4 to consider the following as one of the performance metrics for AI/ML-based L3-RSRP prediction: - Top-1 (%) : The percentage of ‘the Top-1 strongest cell is Top-1 predicted cell’ - Top-K/1 (%) : The percentage of ‘the Top-1 strongest cell is one of the Top-K predicted cells’ - Top-1/K (%) : The percentage of ‘the Top-1 predicted cell is one of the Top-K strongest cells’ - Top-K (%): The percentage of the maximum ground truth L3-RSRP among the Top-K predicted cells being larger than ‘the ground truth L3-RSRP of the strongest cell’ - X dB - Note: Top-1 strongest cell and Top-K strongest cells are determined by the metric of ground truth L3-RSRP.

In our understanding, before discussing the candidate metrics, RAN4 need to discuss necessity of this requirement. Currently, RAN4 have been discussing RSRP prediction accuracy requirement, and a UE that can achieve a good prediction performance is able to predict strong cell with a high successful rate. In addition, these candidate metrics are quoted with some updates and they were for AI/ML beam management use case where AI/ML models have different types of outputs: type 1: beam ID (i.e., reference signal index) or type 2: beam ID and corresponding RSRP. And these listed metrics are for beam ID prediction accuracy. However, as far as we know, all AI/ML models in RRM measurement prediction use case output RSRP only. Therefore we do not think it is not needed to define strong cell prediction accuracy requirement at current stage.

Proposal 6: RAN4 not to define strong cell prediction accuracy requirement.

Impacts on beam/cell level measurements

In RAN4#112bis, the following agreement was achieved for further discussion:

RAN4#112bis: Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

For 2nd and 3rd bullets, considering that RAN2 still deprioritizes L3 filtered beam-level prediction sub-use cases (E) and they focuses on FR2 intra-frequency temporal domain case A, i.e., scenario 4, at the current stage, RAN4 can study the impacts of L1 beam-level (A1) on RRM requirements only.

TR 38.744: 3 sub-use cases are considered for beam-level RRM measurement prediction: Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and should focus on FR2 intra-frequency temporal domain case A.

Proposal 7: RAN4 to deprioritize the study on impact of L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

Regarding the impact of L3 cell level RSRP (Point C) prediction, a new performance requirement will be needed, since current SS-RSRP performance requirements do not consider beam consolidation/selection and layer 3 filtering. These aspects should be considered during simulations for defining L3 cell level RSRP performance requirements.

Proposal 8: Consolidation/selection and layer 3 filtering should be included if RAN4 agree to define L3 cell level RSRP performance requirements.

LCM related requirements

Regarding the LCM related requirements, some proposals were brought up and recommended WFs were provided in last meeting:

RAN4#113: - Recommended WF Candidate options / tentative agreements: - Option 1: RAN4 should focus first on performance metrics and relevant requirements and post-pone the discussion on LCM related issues. - Option 2: RAN4 should start discussion LCM related issues in parallel of discussion on performance metrics.

From our perspective it is pre-mature to discuss LCM related requirements at current stage considering this is a study item and actually more RAN2 progress is needed. In addition, the outcomes in Rel-19 AI/ML Air Interface WI, which is not closed yet, can also be reused. Hence, there is no need to discuss LCM requirements in two AI/ML items in parallel. We suggest agreeing the recommended WF Option 1.

Proposal 9: RAN4 should focus on performance metrics and relevant requirements and postpone the discussion on LCM related issues.

Conclusions

This contribution discussed the impacts of AI/ML RRM measurement prediction on RRM requirements, and some observations and proposals are provided:

Proposal 1: RAN4 do not study radio link failure use case in Rel-19.

Proposal 2: Beams in the definition of relative RSRP prediction accuracy can come from the same or different cells.

Proposal 3: For FR1, ground truth is UE received power derived by TE. For FR2, ground truth is reported value and RAN4 wait for progress on AI/ML BM and reuse the agreement.

Proposal 4: The reported RSRP of cell2/beam2 can be predicted results and proponents to clarify the necessity of a combination of predicted and measured samples.

Proposal 5: It is not necessary to add limitations for the definition of relative RSRP prediction accuracy. Applicability conditions can be discussed for specific use cases.

Proposal 6: RAN4 not to define strong cell prediction accuracy requirement.

Proposal 7: RAN4 to deprioritize the study on impact of L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

Proposal 8: Consolidation/selection and layer 3 filtering should be included if RAN4 agree to define L3 cell level RSRP performance requirements.

Proposal 9: RAN4 should focus on performance metrics and relevant requirements and postpone the discussion on LCM related issues.

References

[1] R4-2502594, WF on AI/ML Mobility, Nokia, RAN4#114.

[2] R4-2500533, Topic summary for [114][219] FS_NR_AIML_Mob, Nokia, RAN4#114.

[3] Report of 3GPP TSG RAN WG2 meeting #129, RAN2#129.