3GPP TSG-RAN WG4 Meeting #114bis				                                           R4-2503417

Wuhan, CN, April 7th – April 11th, 2025

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Source:	CATT

Agenda Item:	7.17.2.2

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC was discussed and the conclusions were captured in the approved WF [1].In this paper, we will continue the discussion on open issues of FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC [2] and give our views.

2 Discussion

Issue 1-2: Applicability requirement of L3 measurement delay reduction by optimizing CSSF - Option 1 (Nokia, vivo): To focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards. - Option 2 (ZTE, CMCC, Ericsson): The CSSF optimization is applicable to all CA, NR-DC, EN-DC and NE-DC scenarios.

According to the latest WID [3], the solution(s) for L3 measurement delay reduction by optimizing CSSF outside gap is applicable for CA/DC scenarios. Therefore all CA, NR-DC, EN-DC and NE-DC scenarios should be covered by the solution(s), if no explicit clarification for specific solution(s).

Proposal 1: The CSSF optimization is applicable to all CA, NR-DC, EN-DC and NE-DC scenarios, if no explicit clarification.

For 3-searcher based solution 3, the following agreement was achieved in last meeting:

Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

Whether to consider NR-DC needs further discussion. As we discussed above, all solutions for CSSF enhancement apply for all CA/DC scenarios should be covered by the solution(s), if no explicit clarification. Therefore NR-DC scenario can be considered for this 3-searcher based solution.

Proposal 2: NR-DC scenario is included for 3-searcher based solution.

Since the solutions for L3 measurement delay reduction by optimizing CSSF outside gap are almost stable and we have only 3 meetings left before the core part ends, we would like to discuss the UE capabilities in this contribution. An initial summary of UE capabilities that need defining for this objective is provided below:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xx. NR_RRM_ph5 | xx-1 | L3 measurement delay reduction by measuring one serving CC per band | Support of L3 measurement delay reduction by measuring one serving CC per band |  | Yes | N/A | UE does not support L3 measurement delay reduction by measuring one serving CC per band | Per UE | No | N/A | N/A |  | Optional |
| xx. NR_RRM_ph5 | xx-2 | L3 measurement delay reduction by supporting 3 searchers in FR1 only scenarios | Support of L3 measurement delay reduction by supporting 3 searchers in FR1 only scenarios |  | Yes | N/A | UE does not support L3 measurement delay reduction by supporting 3 searchers in FR1 only scenarios | Per UE | No | FR1 only | N/A |  | Optional |
| xx. NR_RRM_ph5 | xx-3 | L3 measurement delay reduction by supporting 3 searchers in FR1+FR2 scenarios | Support of L3 measurement delay reduction by supporting 3 searchers in FR1+FR2 scenarios |  | Yes | N/A | UE does not support L3 measurement delay reduction by supporting 3 searchers in FR1+FR2 scenarios | Per UE | No | N/A | N/A |  | Optional |

Proposal 3: RAN4 to discuss UE capabilities for this objective based on the following table:

| Features | Index | Feature group | Components | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xx. NR_RRM_ph5 | xx-1 | L3 measurement delay reduction by measuring one serving CC per band | Support of L3 measurement delay reduction by measuring one serving CC per band | Yes | UE does not support L3 measurement delay reduction by measuring one serving CC per band | Per UE | No | N/A | Optional |
| xx. NR_RRM_ph5 | xx-2 | L3 measurement delay reduction by supporting 3 searchers in FR1 only scenarios | Support of L3 measurement delay reduction by supporting 3 searchers in FR1 only scenarios | Yes | UE does not support L3 measurement delay reduction by supporting 3 searchers in FR1 only scenarios | Per UE | No | FR1 only | Optional |
| xx. NR_RRM_ph5 | xx-3 | L3 measurement delay reduction by supporting 3 searchers in FR1+FR2 scenarios | Support of L3 measurement delay reduction by supporting 3 searchers in FR1+FR2 scenarios | Yes | UE does not support L3 measurement delay reduction by supporting 3 searchers in FR1+FR2 scenarios | Per UE | No | N/A | Optional |

3 Conclusion

In this paper, we have some discussions on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the following proposals are given:

Proposal 1: The CSSF optimization is applicable to all CA, NR-DC, EN-DC and NE-DC scenarios, if no explicit clarification.

Proposal 2: NR-DC scenario is included for 3-searcher based solution.

Proposal 3: RAN4 to discuss UE capabilities for this objective based on the following table:

| Features | Index | Feature group | Components | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xx. NR_RRM_ph5 | xx-1 | L3 measurement delay reduction by measuring one serving CC per band | Support of L3 measurement delay reduction by measuring one serving CC per band | Yes | UE does not support L3 measurement delay reduction by measuring one serving CC per band | Per UE | No | N/A | Optional |
| xx. NR_RRM_ph5 | xx-2 | L3 measurement delay reduction by supporting 3 searchers in FR1 only scenarios | Support of L3 measurement delay reduction by supporting 3 searchers in FR1 only scenarios | Yes | UE does not support L3 measurement delay reduction by supporting 3 searchers in FR1 only scenarios | Per UE | No | FR1 only | Optional |
| xx. NR_RRM_ph5 | xx-3 | L3 measurement delay reduction by supporting 3 searchers in FR1+FR2 scenarios | Support of L3 measurement delay reduction by supporting 3 searchers in FR1+FR2 scenarios | Yes | UE does not support L3 measurement delay reduction by supporting 3 searchers in FR1+FR2 scenarios | Per UE | No | N/A | Optional |

4 References

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#114.

- R4-2500531, Topic summary for [114][217] NR_RRM_Ph5_Part1, Apple, RAN4#114.

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RAN#105.