3GPP TSG-RAN WG4 Meeting #114bis				                                           R4-2503419

Wuhan, CN, April 7th – April 11th, 2025

Title:	Discussion on performance requirements of fast SCell activation for UE supporting R18 EMR

Source:	CATT

Agenda Item:	7.17.5

Document for:	Discussion

1 Introduction

According to the time budget for this work item, RAN4 will start discussion on performance requirements of fast SCell activation for UE supporting R18 EMR from April meeting. Therefore we will present our views on how to proceed.

2 Discussion

General principle for defining test case

The objective of fast SCell activation is to shorten the SCell activation delay for unknown cells when UE supports R-18 eEMR measurements and UE has a valid eEMR result for the target SCell. In core part, the enhanced SCell activation delay requirement was discussed and stabilized which can be found in [1]. Since the goal is to shorten delay, therefore no any accuracy requirements will be impacted and RAN4 can directly discuss test cases.

Observation 1: No accuracy requirements will be impacted and RAN4 can directly discuss test cases.

Regarding the test cases for this new feature, considering that RAN4 have defined test cases for Rel-18 eEMR measurement and SCell activation separately in previous releases, RAN4 can use those existing test cases as a starting point. For example, the test case in A.7.6.23.3 for Idle mode fast CA/DC cell reselection measurement for FR2 with valid reporting and the test case in A.7.5.3.19 for SCell activation and deactivation of FR2 unknown SCell with FR1 PCell in non-DRX with L3 reporting during activation.

Next we will discuss how to combine the test cases for Rel-18 eEMR and SCell activation.

Firstly, before UE is indicated to activate target SCell, UE needs to perform eEMR measurement on the target SCell and report a valid result, which, in our understanding, is exactly the same as that in the test case where valid reporting is included. RAN4 can discuss which one can be reused.

Observation 2: The contents in the legacy Rel-18 eEMR test case with valid reporting can be reused.

After UE reports a valid EMR result of target SCell during RRC connection setup as requested by network, UE can be signaled to activate the target SCell. But note that the goal of this feature to shorten the SCell activation delay for unknown SCell, i.e., the target SCell shall be unknown, which means this target SCell needs to meet the unknown SCell conditions. In TS 38.133 clause 8.3.2, the known conditions are defined as follows [2]:

FR1 SCell known/unknown conditions: SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. FR2 SCell known/unknown conditions: For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown.

In order to make the target SCell unknown, a simple way is to extend the time gap between early measurement report transmission and SCell activation command reception. Specifically, the time gap needs to be larger than max(5*measCycleSCell,  5*DRX cycles) for FR1 SCell activation case, and 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4 for FR2 SCell activation case.

Observation 3: Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions, which ensures the target SCell is unknown.

After UE receives SCell activation command from TE, it shall start to activate the target SCell and send the first CSI report in the first available UL resource after at least one CSI-RS occasion and satisfy the activation delay requirement defined in clause 8.3.2A. Some contents in the legacy SCell activation delay test case can be reused. RAN4 can discuss which one can be reused. In Figure 1, we provide an example of high-level test procedure to demonstrate the whole idea.

Figure 1-Illustration of test procedure for fast SCell activation for UE supporting Rel-18 eEMR

Proposal 1: Reuse the contents in the legacy test cases for Rel-18 eEMR measurement with valid result reporting and SCell activation.

Proposal 2: Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions:

-	For target SCell in FR1, time gap needs to be larger than max(5*measCycleSCell,  5*DRX cycles);

-	For target SCell in FR2, time gap needs to be larger than4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4.

Number of test cases

Regarding the number of test cases that RAN4 need to define, RAN4 discussion can consider the following aspects:

1. Target SCell can be in FR1 or FR2 band.

2. Applicable scenarios: EN-DC, SA CA, NE-DC, NR-DC.

Note that the following scenarios are not in the scope as agreed in [3].

  - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA)

  - SCell being in the same band as PCell in FR2 (i.e. intra-band CA)

3. Difference UE capabilities and configurations, i.e., applicability of the requirements [1]:

Requirements in this clause apply if the SCell is unknown, provided: - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or, - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or, - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode, The SCell is unknown if the corresponding known conditions specified in clause 8.3.2 are not met.

4. Type of CSI-RS resources for CQI reporting, i.e., semi-persistent and periodic CSI-RS.

If RAN4 consider all combinations of above aspects, there might be dozens of test cases that need to be defined. In our understanding, the purpose of test case is to verify whether UE is capable of shortening the SCell activation delay by firstly performing Rel-18 EMR measurement and report valid results of target SCell, so it is not necessary to verify each aspect mentioned above. Thus, RAN4 can choose a subset of all combinations which can be discussed during meeting.

Proposal 3: RAN4 consider the following aspects when defining test cases:

1. Target SCell can be in FR1 or FR2 band.

2. Applicable scenarios: EN-DC, SA CA, NE-DC, NR-DC, excluding the following scenarios:

  - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA)

  - SCell being in the same band as PCell in FR2 (i.e. intra-band CA)

3. Difference UE capabilities and configurations.

4. Type of CSI-RS resources for CQI reporting.

Proposal 4: RAN4 to choose a subset of all combinations to define test cases. The list can be further discussed.

3 Conclusion

In this paper, we discussed performance requirements of fast SCell activation for UE supporting R18 EMR, and the following proposals are given:

Observation 1: No accuracy requirements will be impacted and RAN4 can directly discuss test cases.

Observation 2: The contents in the legacy Rel-18 eEMR test case with valid reporting can be reused.

Observation 3: Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions, which ensures the target SCell is unknown.

Proposal 1: Reuse the contents in the legacy test cases for Rel-18 eEMR measurement with valid result reporting and SCell activation.

Proposal 2: Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions:

-	For target SCell in FR1, time gap needs to be larger than max(5*measCycleSCell,  5*DRX cycles);

-	For target SCell in FR2, time gap needs to be larger than4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4.

Proposal 3: RAN4 consider the following aspects when defining test cases:

1. Target SCell can be in FR1 or FR2 band.

2. Applicable scenarios: EN-DC, SA CA, NE-DC, NR-DC, excluding the following scenarios:

  - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA)

  - SCell being in the same band as PCell in FR2 (i.e. intra-band CA)

3. Difference UE capabilities and configurations.

4. Type of CSI-RS resources for CQI reporting.

Proposal 4: RAN4 to choose a subset of all combinations to define test cases. The list can be further discussed.

4 References

- R4-2502685, draft CR for fast SCell activation for UE supporting Rel-18 EMR, Apple, CATT, RAN4#114.

- 3GPP TS 38.133, NR; Requirements for support of radio resource management, V18.8.0.

- R4-2420094, WF on RRM requirements for NR_RRM_Ph5_Part2, RAN4#113.