3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503428

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Discussion on enhancements for Rel-19 ATG RRM performance requirements

Source: 	CATT

Agenda item:	7.12.5

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the core RRM requirements and CRs split work has been done [1]. In this contribution, we discuss and provide our initial views on the performance requirements based on the agreed core requirements.

- Discussion

  - 2.1 Test configurations

For R19 ATG performance part, many configuration and assumption defined in R18 ATG can be reused, which at least including  the following:

- TDD pattern

- Channel model

- GNSS setup

- UE mobility/location assumption

- BS location

- RMC

- OCNG

Proposal 1: RAN4 to reuse the following configurations and assumptions for R19 ATG CA.

- TDD pattern

- Channel model

- GNSS setup

- UE mobility/location assumption

- BS location

- RMC

- OCNG

- General test configuration

For general test configuration, RAN4 need to discuss whether to add new test configuration to include more UE types defined in R19 ATG.

Proposal 2: RAN4 need to discuss whether to add new test configuration to include more UE types defined in R19 ATG core part.

- Measurement accuracy

In R18 ATG, RAN4 to reuse the L3 and L1 accuracy requirement of legacy FR1 TN. For R19 ATG CA, we think that RAN4 to reuse L1/L3 measurement accuracy requirements of legacy FR1 for TN CA.

Proposal 3: RAN4 to reuse L1/L3 measurement accuracy requirements of legacy FR1 for TN CA.

- Test method and scaling factor for UE with antenna array

For test method and scaling factor for UE with antenna array, since the UE types defined in R19 ATG is more complex, RAN4 need to discuss whether to reuse the legacy test method and scaling factor for all types of UE.

Proposal 4: RAN4 need to discuss whether to reuse the legacy test method and scaling factor for all types of UE.

- SCell impact for test cases

In Rel-19 ATG, intra-band co-located and inter-band co-located DL CA are supported. Thus, RAN4 need to define any test cases related to the SCC.

Proposal 5: RAN4 to define test cases for SCC for R19 ATG.

  - 2.2 Test scope

In Rel-19 ATG, the types of UE are different with R18 ATG, and the following types of UE are considered in the core part:

- For intra-band contiguous CA, same antenna type should be applied on each carrier.

- For inter-band CA, either different or same antenna type can be applied on different carriers, RAN4 to consider the following cases for RRM core requirements:

- 1. ATG UE with omnidirectional antennas on both PCell and SCell.

- 2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell.

- 3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell.

- 4. ATG UE with an antenna array on both PCell and SCell with only one antenna panel.

- 5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels.

Therefore, we propose follow this assumption when define the RRM test cases:

Proposal 6: Define test cases and test requirements for the agreed UE types in core part for TDD and FDD.

For the exact test cases, based on the core requirements, RAN4 has introduced some new clauses for R19 ATG, so we believe that the folowing new test cases also should be added for R19 ATG compared to R18 ATG test cases:

- TC for SCell Activation and Deactivation Delay for ATG

- FFS: TC for Interruption for ATG

Proposal 7: The folowing new test cases should be added for R19 ATG compared to R18 ATG test cases:

- TC for SCell Activation and Deactivation Delay for ATG

- FFS: TC for Interruption for ATG

For active BWP switch, since RAN4 has agreed to introduce the following requirements and the legacy can be reused as baseline.

- Simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous Timer based BWP switch delay on multiple CCs

- non-simultaneous Timer based BWP switch delay on multiple CCs

So, RAN4 also need to introduce the test cases as follows:

- TC for Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

- TC for Simultaneous RRC-based Active BWP Switch on multiple CCs

Proposal 8: For active BWP switch, RAN4 need to introduce the following test cases for R19 ATG:

- TC for Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

- TC for Simultaneous RRC-based Active BWP Switch on multiple CCs

- Conclusion

This document discussed the issues for R19 ATG RRM performance requirements and presented following proposals:

Proposal 1: RAN4 to reuse the following configurations and assumptions for R19 ATG CA.

- TDD pattern

- Channel model

- GNSS setup

- UE mobility/location assumption

- BS location

- RMC

- OCNG

Proposal 2: RAN4 need to discuss whether to add new test configuration to include more UE types defined in R19 ATG core part.

Proposal 3: RAN4 to reuse L1/L3 measurement accuracy requirements of legacy FR1 for TN CA.

Proposal 4: RAN4 need to discuss whether to reuse the legacy test method and scaling factor for all types of UE.

Proposal 5: RAN4 to define test cases for SCC for R19 ATG.

Proposal 6: Define test cases and test requirements for the agreed UE types in core part for TDD and FDD.

Proposal 7: The folowing new test cases should be added for R19 ATG compared to R18 ATG test cases:

- TC for SCell Activation and Deactivation Delay for ATG

- FFS: TC for Interruption for ATG

Proposal 8: For active BWP switch, RAN4 need to introduce the following test cases for R19 ATG:

- TC for Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

- TC for Simultaneous RRC-based Active BWP Switch on multiple CCs

- Reference

[1]	R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC