3GPP TSG-RAN WG4 Meeting #114-bis	                                R4-2503432

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Discussion on RRM requirements for LP-WUS/WUR

Source: 	CATT

Agenda Item:	7.26.4.2

Document for:	Discussion

1. Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 LP-WUS/WUR were further discussed. The conclusions were captured in the approved WF [1] and the detailed issue list can be found in the summary [2].

In this contribution, we have some further discussions on RRM requirements for LP-WUS/WUR and give our proposals.

2. Discussion

2.1 General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

In the last meeting, RAN4 didn’t discuss Cases/states to be considered for RRM relaxation and serving cell measurement offloading. The controversy is mainly about Case 2, and the RRM measurement Case 2 is clarified as follows [2]:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

In our view, we don’t see the necessity of MR serving cell relaxation measurement when MR neighbouring cell measurement is off and LR measurement is on. We prefer not to consider Case 2 and RAN4 can revisit if RAN1/2 has any conclusion conflicting with RAN4 assumptions.

Proposal 1: Prefer not to consider Case 2 and RAN4 can revisit if RAN1/2 has any conclusion conflicting with RAN4 assumptions.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

The discussion of criteria of entry/exit conditions includes the conditions for LP-WUS monitoring, LP-WUR measurement and MR RRM measurement relaxation. Firstly, RAN4 need to align the understanding on the three procedures and our understandings on the entry of three procedures are as below:

- LP-WUR measurement: LR starts serving cell measurement and meets the measurement requirements regardless MR status.

- MR measurement relaxation: MR performs relaxed measurement and meets relaxed requirements when LR is turned on, i.e., the case #3 above.

- LP-WUS monitoring: LR starts monitoring LP-WUS. MR may stop the legacy PO monitoring before LR receives LP-WUS indicating wake-up.

For the entry/exit condition of LP-WUS monitoring, RAN1 and RAN2 had the following conclusions:

RAN1#116bis meeting: Working Assumption - From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. - RAN2#126 meeting: - Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. - RAN2#127 meeting: - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ. RAN2#127bis meeting: - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

And for RRM relaxation and offloading in RRC_IDLE/INACTIVE, RAN2 had the following agreements in RAN2#126 and RAN2#127 meeting:

- RAN2#126 meeting: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbour cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. - RAN2#127 meeting: - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

We can see that RAN1/2 already had initial agreements on the criteria for LP-WUS monitoring, RRM measurement relaxation and fully offloading. And from the previous experience, the criteria discussion is usually within the scope of RAN2. So we think that RAN4 can follow the RAN1/2 conclusions and focus on the discussion of requirements, and any further discussion can be triggerred by RAN2 LS.

For the entry/exit conditions for LP-WUR measurement which is not discussed in RAN1/2, we think when to turn on the LR and start LP-WUR measurement is UE implementation, and what we should consider is when UE can relax or stop the MR measurements. And even when the measurement is performed by MR, UE is not mandatory to turn off LR and stop LR measurement. Based on the above understanding, we think that RAN4 does not need to define the entry/exit condition for LP-WUR measurement.

Proposal 2: RAN4 not to define entry/exit conditions for LP-WUR measurement.

Proposal 3: The discussion for criteria of LP-WUS monitoring, RRM measurement relaxation and fully offloading can be left to RAN2, and RAN4 to focus on the LP-WUR measurement requirements and MR measurement relaxation requirements.

2.2 LP-WUR measurement requirements in RRC_IDLE/INACTIVE state

Issue 1-2-2: Periodicity for LP-WUR RRM requirements

Besides the above requirements, we need to define measurement period requirements for LR. In the previous meeting, it was agreed both LP-SS and SSS based serving cell measurement requirements need to be defined. For LP-SS based and SSB based measurement, the following agreements were reached in the previous meeting:

RAN4#112-bis： Issue 1-2-6: LP-SS based RRM requirements periodicity and filtering Agreement: Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity. RAN4#113： Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

For the SSB based LP-WUR measurement delay requirements, RAN4 has agreed the periodicity when LP-SS is not configured, and RAN4 will apply the same agreement as for when LP-SS is not configured. However, some company believed that LP-SS and DRX are both designed for the same power saving, if both are configured, there is no need for UE to measure with a smaller cycle, so the upper bound for SSB based LP-WUR measurement delay requirements is not necessary. When LP-SS is configured, if LO periodicity is smaller than LP-SS periodicity, it is reasonable for UE to use the LP-SS periodicity for measurement, but for the consistency of the design, we can also agree that there is no requirement.

Proposal 4: No need to define upper bound when LP-SS is not configured for SSB based LP-WUR measurement requirements.

Proposal 5: For periodicity for SSB based LP-WUR measurement delay requirements, when LP-SS is configured, there is no requirement if LO periodicity is smaller than LP-SS periodicity.

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

According to the existing cell reselection requirements, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell within a certain cycle. But for fully offloading case, the MR measurements are stopped and no neighbour cell measurement is performed. In such case, whether the evaluation of cell selection criterion is still needed should be discussed. In our understanding, although only LR measurement is performed in such case, the serving cell measurement should be still guaranteed. However, considering that the typical case is the S criterion threshold is lower than the MR offloading threshold and neighbor cell measurement triggering threshold, so we can compromise to not define the LR-WUR based cell selection criterion evaluation in case 1.

Proposal 6: RAN4 to not define LR-WUR based cell selection criterion evaluation for fully offloading case.

In last meeting, there was also proposal to define interruption requirements due to the time gap between LP-WUS reception and MR to start PDCCH monitoring. For this time gap, RAN1 has the following agreements:

- RAN1#118bis - Agreement - From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if - it does not detect a LP-WUS on the monitored LO - or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup - [RAN1#118]: - Agreement - For the wake-up delay, consider the following options: - Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 - Above applies for IDLE/INACTIVE mode. - Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring - [RAN1#118bis]: - Agreement - Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

According to the agreements above, the time gap is based on UE report but the value is still FFS. As to whether to define interruption requirements due to the time gap, we prefer to wait for RAN1 progress on the candidate values and the possible components.

Proposal 7: Regarding whether to define interruption requirements due to the time gap between LP-WUS reception and MR to start PDCCH monitoring, wait for RAN1 further progress on the candidate values and the possible components.

2.3 LP-WUS operation in RRC_CONNECTED state

Issue 1-4-1: LP-WUR at CONNECTED mode

Based on the WID as copied below, no LP-WUR measurement is requested and the UE RRM/RLM/BFD/CSI measurements are performed by MR in for CONNECTED mode. So no LP-WUR measurement requirements are needed and no RAN4 impact is expected.

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

RAN1 has discussed LP-WUS operation in RRC_CONNECTED state, and the only aspect related to RAN4 is the time gap between LP-WUS reception and PDCCH monitoring. And it has been defined as UE capability in RAN1, so RAN1 will decide the candidate values and there is no RAN4 impact is expected so far.

Proposal 8: For LP-WUS operation in RRC_CONNECTED state, no RAN4 impact is expected so far.

3. Conclusion

In this paper, we provide our views on RRM core requirements for R19 LP-WUS/WUR. From this discussion we have derived the following proposals:

Proposal 1: Prefer not to consider Case 2 and RAN4 can revisit if RAN1/2 has any conclusion conflicting with RAN4 assumptions.

Proposal 2: RAN4 not to define entry/exit conditions for LP-WUR measurement.

Proposal 3: The discussion for criteria of LP-WUS monitoring, RRM measurement relaxation and fully offloading can be left to RAN2, and RAN4 to focus on the LP-WUR measurement requirements and MR measurement relaxation requirements.

Proposal 4: No need to define upper bound when LP-SS is not configured for SSB based LP-WUR measurement requirements.

Proposal 5: For periodicity for SSB based LP-WUR measurement delay requirements, when LP-SS is configured, there is no requirement if LO periodicity is smaller than LP-SS periodicity.

Proposal 6: RAN4 to not define LR-WUR based cell selection criterion evaluation for fully offloading case.

Proposal 7: Regarding whether to define interruption requirements due to the time gap between LP-WUS reception and MR to start PDCCH monitoring, wait for RAN1 further progress on the candidate values and the possible components.

Proposal 8: For LP-WUS operation in RRC_CONNECTED state, no RAN4 impact is expected so far.

4. Reference

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo, RAN4#114

- R4-2500539, Topic summary for [114][225] NR_LPWUS, vivo, RAN4#114