3GPP TSG-RAN WG4 Meeting #114-bis	                                                                                       R4-2503438

Wuhan, CN, 7 Apr. – 11 Apr, 2025

Source: 	CATT

Title: 	Discussion on Ambient IoT device 1 RF requirements

Agenda item:	7.24.2.2

Document for:	Approval

- Introduction

In RAN4 #114 meeting, a WF [1] on RF requirements for A-IoT was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

This contribution provides our analysis for device 1 RF requirements based on the discussion in previous meeting.

- Discussion

  - Tx requirements

The open issues on RF requirements for A-IoT were discussed during the previous meetings, as shown below:

#### Topic 3-2: Tx Issue 3-2-1: Transmit output power Agreement: - Measurement configuration including input CW power and potential other conditions need to be defined. Issue 3-2-2: Modulation quality requirements Agreement: - Further discuss whether and how to define the modulation signal quality requirements Issue 3-2-3: Occupied bandwidth Agreement: - Further discuss whether to define the occupied bandwidth for device 1. Issue 3-2-4: Emission requirements Agreement: - Consider one composite emission requirements covering the emission frequency range of SEM and spurious emission as starting point. - Further discuss whether to define TRP or EIRP requirement for unwanted emission.

Since A-IoT Device 1 will operate in backscatter mode, the term “transmit output power” does not accurately describe the Tx output power requirements. Considering that there is no transmitter in Device 1, the term “transmit” is not appropriate, and for aligning with the 3GPP style, we believe that the term Radiated power could be a good choice.

Proposal 1: The terminology of Device 1 Tx output power requirements could be defined as Radiated power.

The detail of Tx output power requirements should be discussed as well. The output power requirement is used to determine the maximum output power of the device, for device 1 the output signal is generated by backscattering the signal from the CW. Therefore, the Device 1 radiated power requirement should be defined as the maximum backscattered power of Device 1 at the maximum CW Tx power and at a designated distance.

Proposal 2: Device 1 radiated power requirement should be defined as the maximum backscattered power of Device 1 at the maximum CW Tx power and at a designated distance.

For modulation quality requirements, according to RAN1#120 meeting agreement, both OOK and BPSK for baseband modulation will be supported modulation scheme for D2R devices, as shown below:

Agreement For a D2R transmission, it is up to device implementation to use OOK or BPSK modulation. It is assumed that a device operates consistently with the same D2R modulation in the same inventory/command round. Agreement For D2R BPSK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {-1} in the baseband. Agreement For D2R OOK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {0} in baseband.

Since the modulation quality requirements are mainly used to limit the amplitude and phase downgrade of a signal, we believed that the modulation quality requirements are still required for Device 1.

However, the modulation quality requirements for Device 1 might be slightly different from NR, depending on the modulation scheme. For OOK modulation, EVM measurements can only be conducted when the transmitter is switched ON (no signal is transmitted when the transmitter is switched OFF), so how to apply the EVM requirements to Device 1 may still need to be investigated. Meanwhile, the above problems do not exist for BPSK modulation, so the current NR EVM requirements can be considered as a good starting point. In addition, considering the capability and cost of device 1, the EVM requirements can be appropriately relaxed.

Therefore, we believe that a less stringent EVM requirements than BPSK EVM requirement could be defined for Device 1 (only applies for ON chip).

Proposal 3: For modulation quality, a less stringent EVM requirements than BPSK EVM requirement could be defined for Device 1 (only applies for ON chip).

For Unwanted emissions or SEM and spurious emissions requirements, since Device 1 is considered as a passive device, the transmitted signal of a passive device is generated by reflection through impedance matching, and the very simple implementation makes its performance also less varied and relatively easy to control. Due to the relatively simple structure, the unwanted emissions of Device 1 may not be too complex. In addition, considering that Device 1 has a structure similar to RFID tags, we believed that the ETSI RFID study could be considered as a good reference and starting point.

Referring to the ETSI EN 302 208, the emissions requirements are captured below (aka spectrum mask):

The ETSI RFID emission requirements includes spectrum masks (indicated the emission range of SEM) and spurious emissions, hence we believed that the emission requirements for Device 1 could consider it as a reference as well.

Proposal 4: Emission requirements should be defined for Device 1. Meanwhile, the requirement should cover the emission range of SEM and spurious emissions.

  - Rx requirements

For maximum input power, this requirement might be related to the maximum coupling loss (MCL). The maximum input power level for device could be calculated by maximum A-IoT BS Tx power minus the MCL, so if the Maximum Input Power requirements need to be discussed, the MCL needs to be determined first.

We noticed that the MCL has been discussed during the coexistence study of ambient IoT SI phase and was captured in TR 38.769. Therefore, we believed that the MCL value in TR 38.769 could be considered as a good reference and starting point.

Proposal 5: The MCL should be discussed first to check if maximum input power requirement is needed. If the MCL needs to be determined, the value of MCL in TR 38.769 could be considered.

- Conclusion

This contribution provides our initial views on A-IoT Device 1 requirements. The following proposals are concluded as follows:

Proposal 1: The terminology of Device 1 Tx output power requirements could be defined as Radiated power.

Proposal 2: Device 1 radiated power requirement should be defined as the maximum backscattered power of Device 1 at the maximum CW Tx power and at a designated distance.

Proposal 3: For modulation quality, a less stringent EVM requirements than BPSK EVM requirement could be defined for Device 1 (only applies for ON chip).

Proposal 4: Emission requirements should be defined for Device 1. Meanwhile, the requirement should cover the emission range of SEM and spurious emissions.

Proposal 5: The MCL should be discussed first to check if maximum input power requirement is needed. If the MCL needs to be determined, the value of MCL in TR 38.769 could be considered.

- References

[1] R4-2502858, “WF on requirements for system parameters and A-IOT device”, RAN4#114, CMCC