3GPP TSG-RAN WG4 Meeting #114bis                                                                                                      R4-2503457

Wuhan, Hubei, China, 07th –11st April, 2025

Source: 	CATT

Title: 	Discussion on UE RF requirements for RedCap PC2 FDD

Agenda item:	7.6.1

Document for:	Approval

- Introduction

In RAN4# 114 meeting, A WF [1] on NR PC2 RedCap UE in FDD bands was agreed and some remaining issues in WF need further discussion.

So, in this contribution, we provide our views on UE RF requirements for RedCap PC2 UEs for FDD bands.

- Discussion

Baseline for PC2 RedCap UE with 1Rx and 2Rx for FDD bands

In current TS 38.101, For FDD bands, for non-RedCap UE, the Table 7.3.2-1a was used as baseline for PC3 2Rx and PC2 2Rx, and for RedCap UE, Table 7.3.2-1a was also used as baseline for PC3 1Rx and PC3 1Rx. The Table 7.3.2-1a is defined as “Table 7.3.2-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands”, which is for PC3 non-RedCap UE with 2Rx.  So for PC2 RedCap UE for FDD bands operating FD-FDD mode, it is reasonable to use reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a as baseline for 2 Rx antenna ports and 1 Rx antenna ports, since REFSENS value in the Table 7.3.2-1a is more clearer.

For example for FDD bands, for a RedCap UE supporting power class 2 equipped with 2 Rx antenna ports, the requirement can be defined as below:

For FDD bands, for a RedCap UE supporting power class 2 equipped with 2 Rx antenna ports, certain degradation of the reference sensitivity for FDD bands in Table 7.3.2-1a is allowed. The maximum amount of degradation is specified in Table 7.3.2-1c for channel BWs up to 20MHz for a UE that indicates txDiversity-r16 or txDiversity2Tx-r18 [15].

Proposal 1: Use reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a as baseline for 2 Rx antenna ports and 1 Rx antenna ports for PC2 RedCap UE for FDD bands operating FD-FDD mode.

REFSENS for 1Rx in FD-FDD for PC2 RedCap UE

As per the WF [1], the remaining open issue concerning REFSENS for 1RX in FD-TDD for PC2 RedCap UE is shown as below:

Issue 1-2-3: REFSENS for 1Rx in FD-FDD Agreement: - Further discuss the following formats for further evaluations - Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR - Evaluate the values of ΔR band by band - Note: Other options are not precluded

As above discussion and proposal, it is better to use reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a as baseline for 1 Rx antenna ports for PC2 RedCap UE for FDD bands operating FD-FDD mode. The REFSENS (1Tx (23dBm) /1Rx) in Option 1 is not directly defined as in Table, which is also needed to use reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a as baseline, so Option 1 is not good choice. For Option 2, REFSENS (1Tx (26dBm) /2Rx) in Option 2 can defined as reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a,  the ΔR in Option 2 can be defined as maximum amount of degradation specified in Table 7.3.2-1c for channel BWs up to 20MHz  plus the amount given in ΔR1R in Table 7.3I.2-1 as starting point. So we think the REFSENS (1Tx (26dBm) /2Rx) should be reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a.

So, we propose option 2, and REFSENS (1Tx (26dBm) /2Rx) in Option 2 should be reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a, the ΔR in Option 2 can be define as maximum amount of degradation specified in Table 7.3.2-1c for channel BWs up to 20MHz plus the amount given in ΔR1R in Table 7.3I.2-1 as starting point.

Proposal 2: Adopt Option 2 for REFSENS for 1Rx in FD-FDD as below:

  - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR

    - REFSENS (1Tx (26dBm) /2Rx) in Option 2 should be reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a

    - The ΔR in Option 2 can be defined as maximum amount of degradation specified in Table 7.3.2-1c for channel BWs up to 20MHz plus the amount given in ΔR1R in Table 7.3I.2-1 as starting point.

Base on above proposals, a draft CR on power class 2 RedCap for TDD bands and FDD bands for TS 38.101-1 is provided for endorsement.

- Conclusion

This contribution provides analysis on UE RF requirements for RedCap PC2 UEs for FDD bands, and the following proposal is concluded as follows:

Proposal 1: Use reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a as baseline for 2 Rx antenna ports and 1 Rx antenna ports for PC2 RedCap UE for FDD bands operating FD-FDD mode.

Proposal 2: Adopt Option 2 for REFSENS for 1Rx in FD-FDD as below:

  - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR

    - REFSENS (1Tx (26dBm) /2Rx) in Option 2 should be reference sensitivity for 2Rx antenna ports for FDD bands in Table 7.3.2-1a

    - The ΔR in Option 2 can be defined as maximum amount of degradation specified in Table 7.3.2-1c for channel BWs up to 20MHz plus the amount given in ΔR1R in Table 7.3I.2-1 as starting point.

Base on above proposals, a draft CR[3] on on power class 2 RedCap for TDD bands and FDD bands for TS 38.101-1 is provided for endorsement.

- References

- R4-2502874, WF on NR PC2 RedCap UE in FDD bands, China Telecom, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple, RAN4#114

- RP‑243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, RAN#106

- R4-2420357, (NR_PC2_RedCap_UE) CR to 38.101-1: PC2 Redcap support for TDD bands, Ericsson, RAN4#113

- R4-2503458, Draft CR for 38.101-1: Introduction on power class 2 RedCap for TDD bands and FDD bands, CATT, RAN4#114bis.