3GPP TSG RAN WG4 Meeting #114bis		R4-2503492

Wuhan, Hubei, China, April 07th – 11th , 2025

Title: 	Further discussion on power domain enhancements for single carrier

Agenda Item:	7.1.1.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

RAN4#114 has made a decent progress on power domain enhancements for single carrier [1], and there are still some open issues to be decided:

#### Issue 1-1-3: A-MPR handling - Proposals - Option 1: - If A-MPR is specified for a given RB allocation X, then the UE performs max(MPRenh(X), A-MPR(X)) - If A-MPR is not specified for a given RB allocation, then the UE performs max(MPR(X), A-MPR(X)) - Option 2: - For a given additional emissions requirement and corresponding A-MPR table, if it can be confirmed that A-MPR is specified for all RB allocations X for which A-MPR(X)≥minX∈SMPR(X) where S is the set of all possible RB allocations for the given carrier, then the UE performs max(MPRenh(X), A-MPR(X)) - Option 3: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. In many cases A-MPR is defined for the channel located closest to the additional emission requirement while channels with certain guard gap default to MPR. Those channels could benefit from the power enhancement scheme while minimising the risk for implementation. - Others - WF - FFS, considering that additional emission in most cases are applied on one side of the UE CBW #### Issue 1-2-1: Width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed 0.5* UE CBW is considered - Symmetrical extension is 0.5 * UE CBW on each side. - Asymmetric extension is 0.5 * UE CBW on one side. - Option 2: ≤ 0.5* UE CBW, namely flexible size at each side pending on deployment scenario, and offset of the edges between UE CBW and BS CBW, etc. - Option 3: In scenario 2 no limitations are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion. - Option 4: The CBW extension shall be aligned with existing channel bandwidth sizes. The extension should always result into a total number of RBs (“original UE channel RBs” + “RB extension”) being an element of Table 5.3.2-1 from TS 38.101-1. - Option 5: Extension is performed by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth - WF - FFS in next meeting - The max size of the extension at each side of UE CBW - Whether extended CBW is fixed or could be flexible #### Issue 1-2-2: How to get the width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed value, i.e., 0.5* UE CBW is stipulated in the spec - Option 2: Flexible value signalled by the NW, the extended CBW at each side ≤ 0.5* UE CBW but not necessarily the one in Table 5.3.2-1 in TS 38.101-1 - Option 2a: Flexible value signalled by the NW, the extended CBW at each side should be the CBW specified in Table 5.3.2-1 in TS 38.101-1 - Option 3: Based on SIB1 cell specific CBW - WF - FFS in next meeting #### Issue 1-2-4: Approaches of deriving the new inner RB region with RBs in extended CBW - Options - Option 1: Fixed value case with extended CBW - If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region. If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart_Low ≤ RBStart ≤ NRB – LCRB, and LCRB ≤ ceil(2/3 *NRB) If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart ≤ RBStart,High, and LCRB ≤ ceil(2/3 *NRB) - Option 2: Flexible value case with extended CBW at each side - Variety expressions from companies, but essentially are based on the new NRB to derive the new inner region, e.g. - N'RB=Nextended_low+NRB+Nextended_high - Option 3: Using cell specific CBW to derive the new inner region - WF - FFS in next meeting

In this contribution, we present our views on these open issues.

# 2 Discussion

A-MPR handling

The purpose of A-MPR is to ensure a certain additional spurious emission requirements, and in 5G NR the power back-off is determined by max(MPR, A-MPR), thus there may be two cases:

(1) Case #1: MPR > A-MPR. Because power enhancement leads to a lower MPR value, it may not be able to meet the additional spurious emission requirements, therefore, no power enhancement should be applied to this case.

(2) Case #2: MPR <= A-MPR. A enhanced lower MPR value is still less than A-MPR, hence power enhancement can be applied to this case while still meeting the additional emission requirements.

Proposal 1: Power enhancement is only applied to the case where MPR is no larger than A-MPR.

Width/RB numbers of extended CBW at each side of UE CBW and how to get the width/RB numbers of extended CBW at each side of UE CBW

Fig. 1, Symmetric extension

Since RAN4 has agreed that the OOB boundary is UE CBW + 5MHz from the edge of the extended UE CBW, the new OOB boundary should still meet ITU recommendation no matter what is the extension of UE CBW. Fig. 1 illustrates a symmetric extension based on this agreement, where the extended UE CBW is 2αB with B as the UE CBW. From Fig. 1, it is clear that αB+B+5 <= 2.5B, i.e.,

α≤1.5-5B                                (1)

If the minimum allowed UE CBW is 10MHz for the extension operation, then the maximum value of α should be:

α≤1.5-510=1

That means, the total extended UE CBW is 2αB, i.e., 0.5B at each side.

Proposal 2: If the minimum allowed UE CBW for the extension is 10 MHz, the extended UE CBW should not exceed half of the UE CBW on each side, resulting in a total extended UE CBW that is twice the original UE CBW.

However, the extended UE CBW should still be confined within BS CBW as agreed in [2]. Therefore, the final effective extended UE CBW is the overlapping portion between BS CBW and the extension, which adds half of the UE CBW on each side.

Proposal 3: The final effective extended UE CBW is the overlapping portion between BS CBW and the extension, which adds half of the UE CBW on each side.

Hence, it becomes clear that UE can derive the RB numbers of extended UE CBW at each side based on the final effective extended UE CBW and SIB1 carrier bandwidth.

Proposal 4: UE can derive the RB numbers of extended UE CBW at each side based on the final effective extended UE CBW and SIB1 carrier bandwidth.

Approaches of deriving the new inner RB region with RBs in extended CBW

The purpose of introducing the concept of extended UE CBW is to apply the same principle or calculation of different regions including inner region, where NRB is replaced by the RB number of extended UE CBW.

Proposal 5: The same calculation of different regions including inner region applies by replacing NRB with the RB number of extended UE CBW.

# 3 Conclusion

In this contribution, we have the following proposals on power domain enhancements for single carrier:

Proposal 1: Power enhancement is only applied to the case where MPR is no larger than A-MPR.

Proposal 2: If the minimum allowed UE CBW for the extension is 10 MHz, the extended UE CBW should not exceed half of the UE CBW on each side, resulting in a total extended UE CBW that is twice the original UE CBW.

Proposal 3: The final effective extended UE CBW is the overlapping portion between BS CBW and the extension, which adds half of the UE CBW on each side.

Proposal 4: UE can derive the RB numbers of extended UE CBW at each side based on the final effective extended UE CBW and SIB1 carrier bandwidth.

Proposal 5: The same calculation of different regions including inner region applies by replacing NRB with the RB number of extended UE CBW.

# Reference

[1] R4-2503013, “WF on Power domain enhancement”, Huawei, HiSilicon

[2] R4-2414441, “WF on power domain enhancements”, Huawei, HiSilicon