3GPP TSG-RAN WG4 Meeting #114                                                          R4-2503522

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Study of RAN4 impacts for RRM measurement prediction for AIML Mobility in NR

Source:	Apple

Agenda item:	7.21.2.1

Document for:	Discussion

Introduction

RAN4 discussion on the AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR (FS_NR_AIML_Mob study item) starts from RAN4#112bis. The recent RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

During this meeting, it is expected the following aspects to be discussed.

- Work Plan for RAN4 Discussions on AI/ML for Mobility in NR

- AI/ML Mobility Scenarios Evaluation and RRM Requirements

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution, we provide our views and discuss different aspects of AI/ML Mobility Scenarios, Evaluation and RRM Requirements based on the RAN2 framework.

Discussion

During RAN4#112bis meeting the following agreements were captured:

Issue 1-1-2: Scenario/sub-use-case priorities

Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios.

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Agreement:

  - Scenario 2, 3 and 4, the 3 cases can be discussed:

    - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

    - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

    - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

    - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case

Agreement:

- RAN4 to first study the impacts on requirements for the following case:

  - L3 cell level RSRP (Point C) prediction

  - FFS L1 beam-level (Point A1)

  - FFS L3 beam-level (Point E)

  - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Figure 1: 3GPP TS 38.300 Measurement Model

Based on the current RAN2 agreements, following scenarios and use cases have been initially considered for AI/ML in mobility

- Measurement Prediction,

- Handover and Radio Link Failure Prediction, and

- Measurement Event Prediction.

During last meeting RAN4#112bis the following agreement was reached:

Issue 1-1-1: Priorities of different use cases

Agreement:

  - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting.

Agreement:

  - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2.

  - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025).

  - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

RRM Measurement Prediction

In RAN2#125bis the following sub-use cases were agreed for RRM measurement prediction:

- Sub-case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated

- Sub-case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Sub-case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In the draft of TR 38.744, the two main identified study goals are:

- First Goal; Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency, reduces the required signalling, and network overhead of transmitting reference signals (RSs).

- Second Goal; Mobility optimization. Predict future UE radio measurements and use them to optimize HO performance and mobility decisions.

Three domains (or a combination them) have been identified for mobility predictions:

- Spatial: when UE is not measuring every cell and/or beam

- Frequency domain: The UE is not measuring every frequency carrier/bands

- Temporal: when UE may skip and predict some of the measurement time steps

The primary focus of the spatial domain prediction is deciding about a beam/cell based on the measurement of another cell/beam. Based on the current RAN2 framework, spatial domain prediction is used for FR2 intra-cell intra-frequency scenario targeting to reduce the measurement effort.

In the case of frequency domain prediction, the UE measures a specific frequency carrier or band and predicts the measurements for another carrier or band. This approach helps minimize the number of measurements and reduces the required measurement gap, thereby decreasing communication interruptions.

For temporal domain measurement prediction,

In RAN2#125bis meeting, it has been agreed as:

- For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predication.

Intra-frequency temporal domain case A: continuous measurement results in prediction window are predicted by continuous historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either one or more sampling periods (with sliding L1/L3 filtering option) or one or more measurement periods (with non-sliding L1/L3 filtering option), where measurement result(s) are measured before sliding. In RRM temporal prediction case A, there is no measurement reduction within observation window (OW). When sliding forward, to produce new predictions, the UE measures at least part of the samples of the previous predicted window (PW). There is no measurement samples reduction in Case A and it is used for achieving the second goal (i.e., mobility optimization such as handover prediction).

Figure 2: Illustration of temporal domain prediction – case A

Intra-frequency temporal domain case B: measurement results in prediction window are predicted by historical measurement result(s) in observation window. Then observation window and prediction window slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous prediction window is/are skipped during window sliding. Case B design target is for the first goal of measured reduction, where the measurement of the previously predicted samples in previous PW is skipped (either by sliding window or applying sample and hold method) for the new predicted window, enabling measurement reduction (e.g., 50% Figure 3).

Figure 3: Illustration of temporal domain prediction – case B

It is expected that the performance of different approaches (spatial, temporal case A, temporal case B) to be dependent on several factors such as input samples, reductions patterns, predicted window length, etc. Some of the factors need standardization and some need RAN2 signaling for coordination between UE and the network.

Regarding scenario prioritization, the following prioritization was agreed upon during RAN2#126, as outlined in Table 1:

Table 1: Prioritization of evaluation scenarios (Table 5.2.1-1 in TR 38.744 draft)

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

AI/ML Measurement Prediction with Measurement Reduction (UE-Side, first goal)

Measurement reduction in Scenario #2:

(i.e. FR1 to FR1 intra-frequency temporal domain case B), the goal is to reduce measurement overhead by reducing the number of measured samples. To avoid additional complexity, i.e., in the design of new reporting mechanisms, in the 1st goal of measurement reduction, the network is expected to use the predicted measurements reported by a UE, or measurements derived from predicted and measured values, as it uses the current measurements reports. The reported predicted measurements or measurements derived from predictions may include:

- Beam-level layer-1 filtered measurements (A1)

- Cell-level layer-3 filtered measurements (C)

- Beam-level layer-3 filtered measurements (E)

For predicted measurement, one of the key issues which needs to be investigated is predicted measurement accuracy. Based on the current RAN2 progress on AI/ML for Mobility, for measurement prediction accuracy Mean Absolute Error (MAE) which is the average difference between the predicted values and the actual values (e.g., predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement) has been agreed to be used as the performance metric.

However, in RAN4 AI/ML for Air Interface, single or a combination of options listed in Table 3 are under investigation for accuracy. [3]

Table 3: RAN4 metric options for beam management in AI/ML Air Interface WI

- Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

For scenario 2 we have the following sub-use cases:

Agreement:

  - Scenario 2, 3 and 4, the 3 cases can be discussed:

    - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

    - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

    - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

    - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

Sub-use case 1: To predict beam level results, then generate cell level results based on the predicted beam results

Sub-case 1 can be conceptually modeled as a concatenation of an AI/ML model for beam management and the consolidation of beam prediction results. Optionally a single AI/ML model can generate cell-level predictions. However, the sub-case may be transparent to the NW. Figure 4 shows in more detail the AI/ML role/functionality inside the measurement model.

Figure 4: Scenario #2 and Sub-use case 1

The beam level Layer 1 is similar to Rel-18 AI/ML L1-RSRP for beam temporal prediction with exception that the subsequent filtering processing is needed, while AI model is not impacted. Therefore,

- The UE side AI/ML model may make use of a Rel-18 AI/ML model for beam measurement predictions to obtain the predicted L1-RSRP of the same cell (serving cell-serving cell and neighbouring cell-neighbouring cell)

- The framework/simulation platform for AI-PHY beam management can be utilized as much as possible

However, in TR 38.843 it is mentioned that all the following cases are considered for BM-Case2:

- Set B is a subset of Set A

- Set B is not a subset of Set A

- Set B is the same as Set A

It is our understanding that the AI/ML model in Fig 4 performs only temporal prediction and not spatial. That is, scenario #2 for beam level prediction is identical to AI/ML BM-Case when set A= set B.

Observation 1: Scenario #2 for AI/ML in Release-19 Mobility is similar to Release-18 AI/ML beam prediction (Case-2), except that BM Case-2 allows set B to differ from set A. They are identical when set B = set A

Regarding the report of the predictions and the prediction requirements the following agreement was reached:

Agreement:

- RAN4 to first study the impacts on requirements for the following case:

  - L3 cell level RSRP (Point C) prediction

  - FFS L1 beam-level (Point A1)

  - FFS L3 beam-level (Point E)

  - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Beam management/measurement is a L1 procedure and only for serving cell, so network configures L1 measurement and report to UE and therefore UE reports L1 measurement result (beam level) as in Rel-18 AI/ML L1-RSRP beam prediction.  In legacy case, L3 is for mobility and RRM purpose, and network configures L3 measurement/report to UE, and UE reports L3 beam or cell result. If LTM is considered, then neighbour cell measurement can be L1 and then we don’t need to follow this diagram in TS38.300. Legacy L3 framework doesn’t support L1 report (beam level point A1) during the L3 procedure. If network configures to let UE report all the beams in point E, i.e., Point F and point E report are both technically feasible.

In TS 38.133, RAN4 has defined measurement accuracy as a fixed limit (e.g. +/-X dB). As an example, absolute SS-RSRP, and relative SS-RSRP accuracy requirements for intra-frequency FR1 band in some scenarios are expressed as a fixed limit (e.g. +/-X dB), during the tests assuming a Gaussian distribution of the error, the limits will have to be set at +/-3.29 if the probability of failing a "good device under test" in a single test is to be kept at 0.1% (i.e., 99.9% of the measurements based on the assumed Gaussian distribution of errors) will fall within the specified range.

Requirements for the accuracy of predicted measurements may follow the requirements for baseline measurements, but RAN4 should study if there are potentially new or updated requirements when AI/ML-based predicted measurements are reported for measurement reduction purpose. [3]

RAN4 to study accuracy requirements for the AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose (case B). FFS on predicted beam level L3-filtered measurements (point E)

Proposal 2: RAN4 to investigate the impact of measurement errors (RF and baseband) in AI/ML model input data on the accuracy of predicted measurements. Additionally, RAN4 should investigate whether RAN2 simulation results and conclusions regarding achievable prediction accuracy remain valid under such measurement errors for scenario 2.

Proposal 3: For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C)

Additionally, as discussed, RAN4 may have different requirement KPIs (metrics) than RAN2, e.g., because of the availability of the ground truth/reference values and their accuracy in the tests. Evaluation of new RAN4 requirements can be associated with a need to perform new simulations, such as errors/accuracy of the inputs/measurements, latencies of the procedures, new metrics, etc. RAN2 has progressed in defining simulation assumptions and setup for the RRM measurement prediction and RLF prediction. To ensure that these simulations accurately reflect RAN4 RRM requirements and KPIs, it is important to cross-check them with the RAN4 requirements.

Proposal 4: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR.

Regarding Sub-use case 2: To directly predict cell level results based on cell level results, the AI/ML model role is shown in Fig 5.

Figure 5:  Scenario #2 and Sub-use case 2

RAN2 agreements define measurement prediction accuracy for cell-level RRM measurement prediction as the average L3 RSRP difference between the predicted and actual L3-filtered cell-level measurement results for the same cell. For FR2, the RX beamforming gain at the UE is implementation-dependent and thus unknown, therefore more clarification is needed.  For AI/ML-based beam management, one candidate test metric is RSRP prediction accuracy which was concluded to be defined as= predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. The index i may be any beam index in top-K beams based on ground truth L1-RSRP.

As for the ideal measured L1-RSRP for FR2, it was agreed that the ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam. To solve the uncertainty due to RX beamforming gain, it is agreed in RAN4 that the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam.

Considering the above, cell-level RSRP prediction accuracy can be defined as the difference between the predicted RSRP and the ideal measured RSRP. For FR1, the ideal measured RSRP is the transmitted RSRP (assuming conducted test and AWGN channel during the test). For FR2, the ideal RSRP approximates the reported RSRP measurement when the SNR is sufficiently high to ensure accurate reporting

During RAN4#113 the following agreement was reached:

Issue 2-1-9a: L3 RSRP prediction accuracy definition

Agreement:

- For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP

  - FFS Ground truth definition of L3-RSRP for FR1 and FR2

Here is our proposal:

Proposal 5: The proposed cell-level RSRP prediction accuracy is defined as follows:

- Absolute RSRP Accuracy for Cell-Level = predicted L3-RSRP of cell i - ideal received L3-RSRP of cell i

  - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy.

  - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test)

The role of AI/ML model in the measurement model is shown in Fig 6 for sub-use case 3.

Figure 6: Scenario #2 and Sub-use case 3

Measurement reduction in Scenario #3:

Scenario#3 (i.e., FR1 to FR1 inter-frequency in frequency domain) focuses on an additional use case in the inter-cell mobility measurement reduction which is inter-frequency measurement prediction. It can be used to reduce the required inter-frequency measurements a UE needs to take, which in turn can reduce the number of measurement gaps that the UE needs to be configured with. Reducing measurements gaps can improve the data throughput and reduce the latency. The required accuracy of the inter-frequency measurement prediction needs to be studied based on how the predictions are used. Additionally, RAN4 to study any impacts due to coordinating the measurement gaps between the network and the UE.

In Scenario 3, which involves inter-frequency inter-cell prediction from FR1 to FR1 (frequency domain), RAN2 concentrates on predicting a single cell within one inter-frequency layer. The advantage of this scenario lies in the ability to utilize a much longer Measurement Gap Repetition Periodicity (MGRP) or to skip certain gaps for inter-frequency measurements. However, if in RAN4 the UE only predicts measurement results for a single cell, it will not be possible to reduce the MGRP. This is because the predictability of one cell does not imply that all other cells within the same frequency are also predictable or can be skipped. Additionally, the Measurement Gap (MG) is configured to monitor multiple inter-frequency layers, not just one [6] So the output will be cell-level RSRP of multiple cells.

The reduction of MGRP is unlikely to be explicitly reflected in the specifications, as it may depend on network implementation. Decisions regarding the extension of the MGRP or the possibility of skipping certain gap occasions should consider all frequency layers.

Encourage companies to bring proposals on the definition of relative RSRP and absolute RSRP.

Subsequently, companies may also consider following candidate options to introduce RSRP accuracy requirements:

- Option 1: RAN4 to consider relative RSRP accuracy requirements only.

- Option 2: RAN4 to consider absolute RSRP accuracy requirements only.

- Option 3: RAN4 to consider both absolute and relative RSRP accuracy requirements.

Proposal 6: To support the increase of MGRP, RAN4 should investigate the feasibility of predicting measurement results for multiple cells per frequency layer. Additionally, it should examine the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps.

Proposal 7: In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells.

By leveraging measurement data from two frequency layers, an AI/ML model can be trained to capture correlations between radio measurements across these layers. This enables two key applications: (1) predicting measurements on the first layer based on data from the second layer, or (2) determining whether a UE connected to a macro layer can establish a viable connection to a micro layer. A clustering-based method could be employed, where measurements from multiple cells serve as input features for prediction. Time-series measurements, alongside auxiliary parameters (e.g., mobility patterns or signal quality metrics), may further enhance model robustness. Importantly, inter-frequency measurement prediction differs fundamentally from temporal domain predictions, as both the input data requirements (e.g., cross-layer dependencies) and performance evaluation metrics (e.g., accuracy, latency, generalizability) are distinct. (see Fig 7)

Figure 7: Multiple Cell/Frequency Input to AI/ML model for multi-frequency multi cell prediction

Proposals 8: Scenario 3 (Frequency Domain Prediction) can be extended to incorporate multi-cell and multi-frequency layer inputs, as well as time-domain samples as well as multiple cell outputs. This integration would enable measurement gap reduction by leveraging cross-layer and temporal correlations in the data.

Proposal 9: RAN2’s assumption of collocated cells for frequency domain prediction may represent an unrealistic scenario. RAN4 to investigate frequency domain prediction methodologies for non-collocated cells, ensuring alignment with real-world deployment conditions. (scenario #3)

Proposal 10: For frequency-domain prediction, RAN4 to evaluate whether an AI/ML model trained at one site can generalize effectively to another site. Additional conditions must be identified to ensure consistency between training and inference phases for generalization purposes. (scenario #3)

Recent proposals highlight the need to evaluate how AI/ML models for L3 cell-level measurement prediction (e.g., RSRP/RSRQ) interact with cell identification—specifically, whether predicted cells are classified as known (previously detected/measured) or unknown. While RAN2 may not yet recognize the implications of this issue, RAN4 is the appropriate forum for such discussions, given its responsibility for cell identification and measurement accuracy in standardization.

For Scenarios 2 and 4 (intra-cell prediction), cell identification is not critical, as the UE has already detected and measured the serving cell. These scenarios can be excluded from further analysis. For Scenario 3 (FR1-to-FR1 inter-frequency prediction), however, cell identification becomes a key concern. Since the UE does not continuously measure target cells, cells predicted by AI/ML models—but not recently measured—may be treated as unknown, risking misalignment between prediction outputs and network-state awareness.

Proposal 11: RAN4 should evaluate how unknown cell predictions impact AI/ML model accuracy (e.g., false positives/negatives in cell identification). Additionally, RAN4 must define criteria for classifying cells as known or unknown in AI/ML training and inference phases, such as time thresholds or signal stability metrics. Finally, RAN4 should assess whether AI/ML models can reliably predict measurements for unknown cells using historical or cross-frequency correlation data

AI/ML Measurement Prediction Without Measurement Reduction (mobility optimization, second goal)

Another aspect of measurement prediction is the enhancement of mobility and handover (HO) performance when the time and spatial density of measurements remain unchanged, meaning there is no reduction in measurements. In this use case, future measurements predicted using AI/ML are leveraged to optimize mobility decisions.

The measurements may be predicted by the UE and either reported to the network or utilized within the UE to derive further predictions, such as forecasting handovers, radio link failures, or measurement events. The specific use cases and decisions regarding how the predicted measurements are employed to optimize mobility performance, along with their signaling impact, are still under further study in RAN2

Ultimately, the interplay between achievable prediction accuracy and mobility performance is crucial. The evaluation of the benefits provided by AI/ML-assisted mobility should consider KPIs related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will shape the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures

Proposal 12: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures. (scenario #4)

Temporal domain prediction in Scenario #4:

In scenario 4, beam/cell level RSRP are predicted in temporal domain for FR2. However, it is not clear if temporal prediction of RSRP also implies the neighbour cell beam change from the observation to the prediction window. That is, does the temporal prediction can predict RSRP changes for the same beam or across different beams?

Proposal 13: RAN4 to evaluate the assumptions for single-link temporal domain prediction accuracy (single-beam operation) versus tracking multiple beams, including potential beam changes (best Tx/Rx pair) within the observation/prediction window.

Figure 7: Multi-link RSRP temporal domain prediction for Scenario #4

For FR2 the problem of single link RSRP prediction can be extended to a multi-link temporal domain prediction, where RSRP to be predicted are across multiple cells, multiple Tx beams and Rx beams as shown in Fig. 7

Proposal 14:  RAN4 to investigate the extension of Scenario 4 to a multi-link temporal domain prediction across different cells and optimum TX/RX pairs

Proposal 15:  For the multi-link RSRP temporal-prediction extension of Scenario RAN4 to investigate the impact of cell dependent measurement errors (different SINRs and channel conditions) to the prediction accuracy of the AI/ML models (scenario #4)

Proposal 16: RAN4 to evaluate whether an AI/ML model trained at one site can be effectively applied to another site under Scenario 4 and its multi-link extension. Additional conditions must be defined to ensure consistency between training and inference phases for generalization purposes

Impact of measurement error on AI/ML Prediction Accuracy for Mobility

- In the context of mobility scenarios, it is inevitable that measurement inaccuracies will occur, affecting the data used as input or ground truth for AI/ML systems. Such discrepancies can have a substantial negative effect on network performance, leading to incorrect predictions that compromise mobility management, hinder effective handover processes. Performance of AI/ML-based measurement prediction is highly dependent on the accuracy of the input data.

- Based on RAN4#122bis an agreement was reached to evaluate the effect of measurement errors (simulation methodology on R4-2417211) to the AI/ML spatial prediction BM case 1. RAN4 to leverage the simulation methodology and evaluation results for the impact of measurement errors in the predicted measurements for mobility.

Issue 2-1-3: Impacts of measurement error on prediction accuracy

- Candidate options:

  - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase

  - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase

  - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4)

  - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event

During RAN4#114 the following agreement was reached:

Issue 2-1-4: Impacts of measurement error on prediction accuracy

Agreement:

For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

- Proposal 17: RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. RAN4 should validate any RAN2 conclusions in the presence of realistic measurements errors.

Proposal 18: RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions.

Proposal 19: Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, hardware compatibility, signal propagation characteristics) must be defined to ensure consistency between training and inference phases for robust generalization.

Ground Truth Definition for RSRP accuracy

- In RAN4#114 the following agreement was reached:

Issue 2-1-2: Ground Truth Definition for RSRP accuracy

- Agreement:

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

  - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as:

    - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction

    - Impact of L3 filtering

    - Number of samples for L1 filtering

    - Timing window, if it exists, where UE can’t measure the ground truth in Alt2

    - channel condition

    - Avoid UE cheat in the test

    - S(I)NR variations in the test at different time instances (for Alt2)

    - Other aspects are not precluded

  - Companies are encouraged to provide analysis on all or some of the aspects in the list

With the definition of ground truth for L3-RSRP (Layer 3 Reference Signal Received Power) in FR1 is critical for training and validating AI/ML models for measurement prediction accuracy. However, concerns exist regarding the practicality of defining ground truth as either:

- Specification-defined expected L3-RSRP (theoretical values), or

- UE-reported L3-RSRP (noise-free),as these approaches may not align with real-world deployment scenarios where AI/ML models must operate under imperfect channel conditions, UE hardware variability, and dynamic environments.

- For Temporal Domain Case A, continuous measurement results within the prediction window are generated using historical measurements from the observation window. As RRM measurements are performed continuously, the ground truth (actual values) for each time instance can be directly derived from the UE’s reported measurements.

- In Temporal Domain Case B, if measurements in the prediction window are intentionally skipped, the RSRP value corresponding to specific time instances cannot be obtained. Defining the ground truth as the UE’s reported RSRP measurements would necessitate continuous UE measurements, effectively aligning its behavior with Case A. This overlap raises concerns about the validity of using reported RSRP as ground truth for Case B. Therefore, further study is required to evaluate whether temporal domain Case B warrants distinct ground truth definitions or modified measurement protocols.

- Observation 2: In Temporal Domain Case B, if measurements in the prediction window are intentionally skipped, the corresponding L3-RSRP values for those time instances may not be available. This absence of reported measurements undermines their viability as ground truth for validation, necessitating alternative approaches or re-evaluation of measurement protocols for such scenarios.

For the option to define the ground truth of L3 RSRP as the reported RSRP measurement under certain SNR. As discussed critical limitation arises in Temporal Domain Case B, where measurements in the prediction window may be intentionally skipped (e.g., to optimize UE power efficiency or reduce network overhead). In such cases, the corresponding L3-RSRP values for skipped time instances may not be available, rendering them unusable as ground truth. This creates a significant validation gap, as the absence of reported measurements undermines the reliability of the model’s predictions.

An alternative approach would be to define the ground truth as the measured RSRP from time t0 to t1, contingent on the channel conditions during the prediction window (t1 to t2) being identical to those in the measurement window (t0 to t1). This method requires the test environment to replicate channel characteristics (e.g., fading profiles, interference levels) at the predicted time instance to ensure reproducibility. By enforcing consistency in channel conditions across both windows, this approach avoids dependency on real-time UE measurements and remains applicable to both Temporal Domain Cases A and B. Unlike previous option, it circumvents the issue of skipped measurements in Case B by decoupling ground truth validation from continuous UE reporting.

Proposal 20: Define the ground truth of L3-RSRP as the measured RSRP from time t₀ to t₁ (observation window), under the condition that the channel characteristics during the prediction window (t₁ to t₂) remain identical to those in the observation window. The goal is to establish a reproducible and scenario-agnostic validation framework for AI/ML-based L3-RSRP prediction accuracy, particularly addressing limitations in scenarios where real-time UE measurements are unavailable or unreliable.

- Proposed Methodology for Proposal 20: Ground truth definition

- Channel-Consistency
The channel conditions (e.g., large-scale fading, Doppler spread, interference levels, and multipath profiles) must remain unchanged between the observation window (t₀–t₁) and the prediction window (t₁–t₂). This ensures that the RSRP measured in the observation window (t₀–t₁) accurately reflects the conditions under which predictions are validated in t₁–t₂.

- Measurement Protocol:

  - During the observation window (t₀–t₁), RSRP is measured continuously using UE or test equipment (TE) under controlled channel emulation.

  - In the prediction window (t₁–t₂), the AI/ML model’s predicted RSRP is compared against the pre-measured ground truth from t₀–t₁, assuming identical channel conditions.

- Emulation and Reproducibility:

  - A channel emulator must replicate time-invariant channel parameters (e.g., path loss, shadowing, delay spread) across both windows.

  - For FR1, consistency in large-scale fading (e.g., path loss models) is prioritized. For FR2, beam-specific parameters (e.g., beam alignment, polarization) must also remain static.

  - Validation requires a correlation coefficient ≥ 0.95 between channel realizations in t₀–t₁ and t₁–t₂ to confirm stationarity.

- Temporal Granularity:

  - The duration of t₀–t₁ and t₁–t₂ is defined based on UE mobility:

    - Low mobility (e.g., pedestrian): 20–50 ms windows.

    - High mobility (e.g., vehicular): 5–10 ms windows to minimize Doppler-induced variations.

- Current discussions in RAN2 on AI/ML-based frequency-domain prediction (e.g., inter-frequency L3-RSRP prediction) focus on scenarios where single or multiple cells serve as inputs, and predictions generate outputs for a single target cell. However, real-world deployments often require predicting RSRP for multiple cells (e.g., neighbor cells in mobility/handover contexts), where relative RSRP accuracy (differences in predicted RSRP between cells) becomes critical for network decisions like cell reselection or load balancing. While some proposals advocate for defining relative RSRP accuracy now, RAN2’s work remains limited to single-cell output scenarios. This necessitates a phased standardization approach.

- Observation 3:  Relative RSRP accuracy is essential for use cases where predictions rank or compare multiple cells (e.g., identifying the strongest neighbor cell for handover). However, RAN2’s current simulations and requirements focus on absolute RSRP accuracy (deviation between predicted and ground truth RSRP for a single cell).

- Observation 4: Existing AI/ML models and test frameworks in RAN2 do not yet address scenarios where predictions output RSRP for multiple cells simultaneously

- Proposal 21:  RAN4 to consider following steps for defining RSRP relative accuracy requirements:

- Step 1: Prioritize Absolute RSRP Accuracy

- Define absolute RSRP accuracy as the primary KPI for initial AI/ML model validation.

- Metrics include:

  - Mean Absolute Error (MAE) between predicted and ground truth RSRP.

  - Tolerance thresholds (e.g., ±3 dB for FR1, ±4 dB for FR2).

- Scope: Apply to current RAN2-supported scenarios (single-cell output).

- Step 2: Monitor RAN2 Progress on Multi-Cell Output Scenarios

- RAN4 to collaborate with RAN2 to investigate AI/ML models that predict RSRP for multiple cells(e.g., neighbor cell lists).

- Identify use cases where relative RSRP accuracy becomes critical (e.g., handover optimization, carrier aggregation).

- Step 3: Define Relative RSRP Accuracy Requirements (Conditional)

- If RAN4/RAN2 standardizes multi-cell output prediction scenarios, RAN4 will:

  - Develop relative RSRP accuracy metrics, such as:

    - Rank order accuracy (correctly ranking cells by predicted RSRP).

    - Differential error thresholds (e.g., ±1 dB tolerance for relative differences between top-K cells).

  - Align test methodologies with multi-cell prediction dynamics (e.g., beam/cell correlation, interference coupling).

Life Cycle Management (LCM) Related Requirements

With the integration of AI/ML models into essential network functions—such as managing mobility, facilitating handovers, and optimizing radio resource allocation—it is important to ensure these models operate efficiently in real-time, adapting to the diverse and dynamic conditions of wireless channels. This consideration is crucial for both UE and network-side implementations. To ensure the sustained effectiveness of these AI/ML models, it is important to establish and monitor KPIs like prediction accuracy and latency, which are indicative of their real-time operational performance. [3]

When an AI/ML model's performance drops below specified thresholds, it's critical for either the network or the UE to implement corrective actions. This could involve updating the model, tweaking its configurations, or modifying the input data. If performance significantly deteriorates, it may be necessary to disable the AI/ML model entirely and revert to traditional methods, such as legacy measurement and reporting systems, to ensure consistent and reliable network operation. This proactive approach helps to maintain network integrity, ensuring that the use of AI/ML does not undermine performance during instances of model failure or reduced efficacy

- Proposal 22:  RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

Conclusion

This paper provides an overview of the Rel-19 AI/ML-based mobility scenarios and RRM requirements under study within the RAN4 working group. The following Observations and Proposals were made:

Observation 1: Scenario #2 for AI/ML in Release-19 Mobility is similar to Release-18 AI/ML beam prediction (Case-2), except that BM Case-2 allows set B to differ from set A. They are identical when set B = set A

- Observation 2: In Temporal Domain Case B, if measurements in the prediction window are intentionally skipped, the corresponding L3-RSRP values for those time instances may not be available. This absence of reported measurements undermines their viability as ground truth for validation, necessitating alternative approaches or re-evaluation of measurement protocols for such scenarios.

- Observation 3:  Relative RSRP accuracy is essential for use cases where predictions rank or compare multiple cells (e.g., identifying the strongest neighbor cell for handover). However, RAN2’s current simulations and requirements focus on absolute RSRP accuracy (deviation between predicted and ground truth RSRP for a single cell).

- Observation 4: Existing AI/ML models and test frameworks in RAN2 do not yet address scenarios where predictions output RSRP for multiple cells simultaneously

- Proposal 1: RAN4 to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E)

Proposal 2: RAN4 to investigate the impact of measurement errors (RF and baseband) in AI/ML model input data on the accuracy of predicted measurements. Additionally, RAN4 should investigate whether RAN2 simulation results and conclusions regarding achievable prediction accuracy remain valid under such measurement errors for scenario 2.

Proposal 3: For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C).

Proposal 4: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR.

Proposal 5: The proposed cell-level RSRP prediction accuracy is defined as follows:

- Absolute RSRP Accuracy for Cell-Level = predicted L3-RSRP of cell i - ideal received L3-RSRP of cell i

  - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy.

  - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test)

Proposal 6: To support the increase of MGRP, RAN4 should investigate the feasibility of predicting measurement results for multiple cells per frequency layer. Additionally, it should examine the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps.

Proposal 7: In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells.

Proposals 8: Scenario 3 (Frequency Domain Prediction) can be extended to incorporate multi-cell and multi-frequency layer inputs, as well as time-domain samples as well as multiple cell outputs. This integration would enable measurement gap reduction by leveraging cross-layer and temporal correlations in the data.

Proposal 9: RAN2’s assumption of collocated cells for frequency domain prediction may represent an unrealistic scenario. RAN4 to investigate frequency domain prediction methodologies for non-collocated cells, ensuring alignment with real-world deployment conditions. (scenario #3)

Proposal 10: For frequency-domain prediction, RAN4 to evaluate whether an AI/ML model trained at one site can generalize effectively to another site. Additional conditions must be identified to ensure consistency between training and inference phases for generalization purposes. (scenario #3)

Proposal 11: RAN4 should evaluate how unknown cell predictions impact AI/ML model accuracy (e.g., false positives/negatives in cell identification). Additionally, RAN4 must define criteria for classifying cells as known or unknown in AI/ML training and inference phases, such as time thresholds or signal stability metrics. Finally, RAN4 should assess whether AI/ML models can reliably predict measurements for unknown cells using historical or cross-frequency correlation data

Proposal 12: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures. (scenario #4)

Proposal 13: RAN4 to evaluate the assumptions for single-link temporal domain prediction accuracy (single-beam operation) versus tracking multiple beams, including potential beam changes (best Tx/Rx pair) within the observation/prediction window.

Proposal 14:  RAN4 to investigate the extension of Scenario 4 to a multi-link temporal domain prediction across different cells and optimum TX/RX pairs

Proposal 15:  For the multi-link RSRP temporal-prediction extension of Scenario RAN4 to investigate the impact of cell dependent measurement errors (different SINRs and channel conditions) to the prediction accuracy of the AI/ML models (scenario #4)

Proposal 16: RAN4 to evaluate whether an AI/ML model trained at one site can be effectively applied to another site under Scenario 4 and its multi-link extension. Additional conditions must be defined to ensure consistency between training and inference phases for generalization purposes

- Proposal 17: RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. RAN4 should validate any RAN2 conclusions in the presence of realistic measurements errors.

Proposal 18 : RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions.

Proposal 19: Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, hardware compatibility, signal propagation characteristics) must be defined to ensure consistency between training and inference phases for robust generalization.

Proposal 20: Define the ground truth of L3-RSRP as the measured RSRP from time t₀ to t₁ (observation window), under the condition that the channel characteristics during the prediction window (t₁ to t₂) remain identical to those in the observation window. The goal is to establish a reproducible and scenario-agnostic validation framework for AI/ML-based L3-RSRP prediction accuracy, particularly addressing limitations in scenarios where real-time UE measurements are unavailable or unreliable.

- Proposed Methodology for Proposal 20: Ground truth definition

- Channel-Consistency
The channel conditions (e.g., large-scale fading, Doppler spread, interference levels, and multipath profiles) must remain unchanged between the observation window (t₀–t₁) and the prediction window (t₁–t₂). This ensures that the RSRP measured in the observation window (t₀–t₁) accurately reflects the conditions under which predictions are validated in t₁–t₂.

- Measurement Protocol:

  - During the observation window (t₀–t₁), RSRP is measured continuously using UE or test equipment (TE) under controlled channel emulation.

  - In the prediction window (t₁–t₂), the AI/ML model’s predicted RSRP is compared against the pre-measured ground truth from t₀–t₁, assuming identical channel conditions.

- Emulation and Reproducibility:

  - A channel emulator must replicate time-invariant channel parameters (e.g., path loss, shadowing, delay spread) across both windows.

  - For FR1, consistency in large-scale fading (e.g., path loss models) is prioritized. For FR2, beam-specific parameters (e.g., beam alignment, polarization) must also remain static.

  - Validation requires a correlation coefficient ≥ 0.95 between channel realizations in t₀–t₁ and t₁–t₂ to confirm stationarity.

- Temporal Granularity:

  - The duration of t₀–t₁ and t₁–t₂ is defined based on UE mobility:

    - Low mobility (e.g., pedestrian): 20–50 ms windows.

    - High mobility (e.g., vehicular): 5–10 ms windows to minimize Doppler-induced variations.

- Proposal 21:  RAN4 to consider following steps for defining RSRP relative accuracy requirements:

- Step 1: Prioritize Absolute RSRP Accuracy

- Define absolute RSRP accuracy as the primary KPI for initial AI/ML model validation.

- Metrics include:

  - Mean Absolute Error (MAE) between predicted and ground truth RSRP.

  - Tolerance thresholds (e.g., ±3 dB for FR1, ±4 dB for FR2).

- Scope: Apply to current RAN2-supported scenarios (single-cell output).

- Step 2: Monitor RAN2 Progress on Multi-Cell Output Scenarios

- RAN4 to collaborate with RAN2 to investigate AI/ML models that predict RSRP for multiple cells(e.g., neighbor cell lists).

- Identify use cases where relative RSRP accuracy becomes critical (e.g., handover optimization, carrier aggregation).

- Step 3: Define Relative RSRP Accuracy Requirements (Conditional)

- If RAN4/RAN2 standardizes multi-cell output prediction scenarios, RAN4 will:

  - Develop relative RSRP accuracy metrics, such as:

    - Rank order accuracy (correctly ranking cells by predicted RSRP).

    - Differential error thresholds (e.g., ±1 dB tolerance for relative differences between top-K cells).

  - Align test methodologies with multi-cell prediction dynamics (e.g., beam/cell correlation, interference coupling).

- Proposal 22:  RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

- References

- RP-242393, Revised SID on AIML for mobility in NR , OPPO, 3GPP TSG RAN Plenary #105, Melbourne, Australia, September 9-12, 2024.

- R2-2406309, Text proposal of 38.744, OPPO, RAN2#127, Maastricht.

- R4-2416449, On AIML Mobility Scenarios Evaluation and RRM Requirements Impacts, NOKIA Hefei, China, Oct 14 – Oct 18, 2024

- R4-2415577, General discussion on RRM requirement impact analyses on AI mobility, Samsung, Hefei, China, Oct 14 – Oct 18, 2024

- R4-2415033, Study of impacts on RAN4 requirements for AI mobility, OPPO , Hefei, China, Oct 14 – Oct 18, 2024

- R4-2414931, Discussion on RRM requirements of AI mobility, MediaTek , Hefei, China, Oct 14 – Oct 18, 2024