# 3GPP TSG-RAN WG4 Meeting #114bis                               	R4-2503527

# Wuhan, China, April 7 – April 11, 2025

Agenda item:	7.26.4.1

Source:	Apple

Title:	Discussion of Simulation Assumptions for LP-WUR RRM Requirements

Document for:	Discussion

# Introduction

Relevant objectives of the work item to RRM are the following [1]:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In last RAN4#112bis meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [2], and open issues have been summarized in [3] and [4]. On the other hand, during RAN4#113 there was no discussion on this sub-agenda. Hence, in this contribution we continue the discussion on simulation assumptions, provide preliminary results, and propose, again, a framework for RAN4 RRM to manage the consolidation of companies’ simulation results moving forward.

# Methodology

RAN4 RRM has been actively engaging in discussions regarding simulation assumptions and parameters, with the aim of establishing the necessary RRM requirements for LP-WUS/WUR once the RAN1 design is completed. At the time of these discussions, consensus on the LP-SS sequence design, duration, and modulation had not yet been reached. RAN1 has made progress in downselecting binary sequences, but during April’s RAN1 meeting it is still expected that further downselection will take place.

However, RAN4 has decided to proceed with the RRM requirements discussion, allowing companies to align on the aspects that can be addressed in advance. More detailed and specific discussions, including the precise numerical requirements, are still deferred until the RAN1/RAN2 design is fully finalized.

This approach implies that various simulation scenarios should be tested ahead of time to ensure the selection of the right scenario or scenarios. We do not propose RAN4 to be fully dependent on RAN1's progress, but rather that RAN4 should adopt a systematic approach to move forward, minimizing excessive reliance on RAN1's timeline.

Observation 1: RAN1 continues the design of LP-SS binary sequences via further sequence downselection.

Observation 2: RAN1 simulation results collection prioritizes LP-SS binary sequence selection based on sync accuracy subject to a minimum RRM measurement accuracy of 3.5dB. RAN4 focus rather on RSRP/RSRQ measurement accuracy and the minimum number of required samples, subject to the LP-SS binary sequence by RAN1.

Observation 3: The results of the RAN1 simulation campaign are still quite misaligned, suggesting the further alignment will be beneficial for RAN4 requirements.

We believe that a critical factor in maintaining progress is achieving alignment on all LP-SS simulation parameters to ensure that results from different companies can be directly compared, and to continue refining results alignment moving forward. We believe this step is crucial for identifying outliers, potential misunderstandings in simulation assumptions or signal processing, and determining whether discrepancies in the reported results are attributable to legitimate differences in LR/UE implementation or stem from other factors.

Proposal 1: RAN4 to continue simulation results campaign and alignment until at least RAN4#116. Consider email offline discussions in between meetings in addition to online meetings to secure good progress.

Proposal 2: Determine accuracy and delay requirements no earlier than RAN4#116 to allow simulation results to converge and being fully understood.

# Simulation Assumptions

As previously discussed during RAN4#112bis, we believe that the simulations conducted by RAN4 should closely align with the simulation assumptions used in RAN1’s evaluation. In general, RAN4 may introduce modifications and improvements as needed, tailored to our specific requirement design work. Our proposal is aimed at ensuring that RAN4’s assumptions for requirement design are closely aligned with the performance expectations of the RAN1 WG’s work, as well as with the ongoing RAN4 RF agenda, which continues to progress in parallel.

Observation 4: For LP-WUR based RRM requirements design, RAN1 link level simulation assumptions and RAN4 RF session’s simulation assumptions should be continuously monitored as the baseline for RAN4 RRM simulation work, specially the discussion about noise figure (NF) and implementation margin.

Furthermore, RAN1 has made progress in refining the set of M and L parameters for LP-SS binary sequences design, including the specific sequences.

Top of Form

Bottom of Form

Observation 5: Candidate sequences are based on 1) M=1, L= {4, 6, 8}, 2) M=2, L= {8, 12, 16}, and 3) M=4, L= {16, 32}. The number of occupied OFDM symbols by LP-SS has been narrowed down to 4, 6 and 8 symbols.

Observation 6: RAN4 has defined initial sets of sequences for simulation alignment purposes, waiting RAN1 final design.

Moving forward, another aspect that needs to be clarified is the source of interference in the current requirements scenario. In one case, we are defining an OOK interferer that collides with the LP-SS of the serving cells. In another scenario, we define an OFDM interferer. At least in concept, it is evident that each interference source has a distinct impact. Refer to the figure below.

Moving forward, we aim to fully understand the effect on delay and accuracy of the different sources of interference being considered. As discussed before, when estimating RSRP as the difference between ON-duration energy and OFF-duration energy, no significant issues are observed, since OFDM interference does not follow a specific time-domain pattern and can be modeled as random QPSK noise. However, in the case of a colliding OOK interferer, the interference during the ON duration may differ from that during the OFF duration.

Proposal 3: RAN4 RRM to analyze the impact of the interference source on delay, accuracy and bias.

Proposal 4: RAN4 to eventually define whether to consider 1) OFDM interference scenario vs 2) colliding OOK interference scenario, or 3) consider both scenarios for delay/accuracy requirements. Decision should be strictly based on submitted simulation results by interested companies.

Top of Form

Bottom of Form

Finally, another aspect that needs to be discussed is the use of OFDM overlaid sequences. From past RAN1 agreements, it has been decided that overlaid sequences will be transmitted.

RAN1 Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

RAN1 agreement

In case of overlaid OFDM sequence not carrying information:

- Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol.

- Note 1: multiple overlaid OFDM sequences are specified.

- Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

Agreement

For overlaid OFDM sequence at least for LP-WUS,

- For OOK-4 with M>1, support ZC sequence for overlaid sequence.

  - FFS details

Observation 7: It is still unclear how LP-SS will be deployed across multiple cells in terms of time and frequency resources. Therefore, RAN4 should consider both the case of OFDM interference into LP-SS and the case of colliding LP-SS interference into the serving cell LP-SS.

Proposal 5: For LP-SS overlaid sequence configuration, use L=127 ZC sequences with different root indexes: q=1 for the serving cell, and q=63 for the interfering cell.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Observation 1: RAN1 continues the design of LP-SS binary sequences via further sequence downselection.

Observation 2: RAN1 simulation results collection prioritizes LP-SS binary sequence selection based on sync accuracy subject to a minimum RRM measurement accuracy of 3.5dB. RAN4 focus rather on RSRP/RSRQ measurement accuracy and the minimum number of required samples, subject to the LP-SS binary sequence by RAN1.

Observation 3: The results of the RAN1 simulation campaign are still quite misaligned, suggesting the further alignment will be beneficial for RAN4 requirements.

Proposal 1: RAN4 to continue simulation results campaign and alignment until at least RAN4#116. Consider email offline discussions in between meetings in addition to online meetings to secure good progress.

Proposal 2: Determine accuracy and delay requirements no earlier than RAN4#116 to allow simulation results to converge and being fully understood.

Proposal 3: RAN4 RRM to analyze the impact of the interference source on delay, accuracy and bias.

Proposal 4: RAN4 to eventually define whether to consider 1) OFDM interference scenario vs 2) colliding OOK interference scenario, or 3) consider both scenarios for delay/accuracy requirements. Decision should be strictly based on submitted simulation results by interested companies.

Observation 7: It is still unclear how LP-SS will be deployed across multiple cells in terms of time and frequency resources. Therefore, RAN4 should consider both the case of OFDM interference into LP-SS and the case of colliding LP-SS interference into the serving cell LP-SS.

Proposal 5: For LP-SS overlaid sequence configuration, use L=127 ZC sequences with different root indexes: q=1 for the serving cell, and q=63 for the interfering cell.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024