# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503531

# Wuhan, China, 7th – 11th April, 2025

Agenda item:	7.17.2.2

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#114, and the open issues have been summarized in the topic summary for [114][217] NR_RRM_Ph5_Part1 [2].

In this contribution, we continue the discussion for FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

# Discussion

Issue 1-3-1: Tentative solution 1: measure one serving CC per band

Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap.

Based on the agreement for solution 1 in last meeting, we prepared one draft CR of CSSF enhancement with solution 1 in this meeting [4].

In addition, RAN4 agreed in #113 meeting that, after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable. In our view, the solution 1 is controlled by network, e.g., enable such CSSF enhancement and indicate which SCC to be measure in certain band, and therefore it would be also beneficial to expand it to FR1 case as well. If the channel conditions are quite different from network perspective and individual measurement shall be performed on all CCs in the same band, network can also disable such feature to UE and UE will follow the Rel-15 spec to perform serving and neighbor cell measurements.

Proposal 1: expand the solution 1 to FR1 band.

Regarding the UE capability of solution 1, we assume UE needs a separated CSSF enhancement capability from the 3 searcher solution, and it can be designed as an optional per-UE capability.

Proposal 2: to have a new optional per-UE capability for CSSF enhancement solution 1 in Rel-19 and it shall be a separated UE capability from 3 searcher solution (i.e., solution 3).

Issue 1-3-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF

Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

Since RAN4 agreed to have separated optional UE capability for FR1 CA/DC only and for FR1+FR2 CA/DC and some companies had concern on the NR-DC, we think the UE capability can be defined separately for {FR1 only CA, FR1 NR-DC, FR1+FR2 CA, FR1+FR2 NR-DC} and then it shall be also fine to the UE which cannot support 3 searchers for NR-DC.

Proposal 3: NR-DC can be included in solution 3 if the UE capability is defined separately for {FR1 only CA}, {FR1 NR-DC}, {FR1+FR2 CA}, and {FR1+FR2 NR-DC}.

For how to use 3 searcher, we think the current option 3 is the easiest way for speeding up the RRM measurement. Even though it cannot enhance a specific SCC measurement, the baseline gain is reduce the measurement delay for each SCC by half. For instance, in FR1 NR-DC case that UE has PCC+PSCC+3SCCs, the CSSF for SCC measurement is 2*3, and option 3 can improve the CSSF to 3. In [3], we also had option 1 and 2 on the table,

Discussion: - RAN4 to discuss the detailed CSSF design: - Option 1: - The 3-searcher solution is PCC occupied one searcher and all SCC share two searchers. - Option 2: - For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - Use a dedicated searcher (the 3rd searcher) for speeding up the prioritized carriers(s), which is provided by NW. - Option 3: - For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing.

If option 2 is used, it will make the allocation mechanism very complicated. When it allocates a dedicated searcher for a specific SCC, then the resource of other SCCs needs to be reallocated, e.g., in the above example, the target SCC CSSF will be 1, PSCC will be 2, and other 2SCCs will be 2*2; and when considering other scenarios, it will be different from scenario to scenario. From our perspective, we prefer a general solution for all scenario to limit the UE complexity.

Proposal 4: use Option 3 for CSSF enhancement solution 3.

- Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: expand the solution 1 to FR1 band.

Proposal 2: to have a new optional per-UE capability for CSSF enhancement solution 1 in Rel-19 and it shall be a separated UE capability from 3 searcher solution (i.e., solution 3).

Proposal 3: NR-DC can be included in solution 3 if the UE capability is defined separately for {FR1 only CA}, {FR1 NR-DC}, {FR1+FR2 CA}, and {FR1+FR2 NR-DC}.

Proposal 4: use Option 3 for CSSF enhancement solution 3.

- Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing.

# References

[1] R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#114

[2] R4-2500531, Topic summary for [114][217] NR_RRM_Ph5_Part1, Moderator (Apple), RAN4#114

[3] R4-2502576, Ad-hoc minutes for NR_RRM_Ph5 WI, Apple, RAN4#114

[4] R4-2503529, Draft CR for Rel-19 CSSF enhancement solution 1, Apple, CATT, Huawei, HiSilicon, Qualcomm, Ericsson, ZTE, MediaTek, Samsung, RAN4#114bis