3GPP TSG-RAN4 Meeting #114Bis	R4-2503559

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Ambient IoT device backscattering and SFO error

Source:	Qualcomm Incorporated

Agenda item:	7.24.2.2

Release:	Rel-19

WI: 	Ambient_IoT_Solutions-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4#114 approved a WF [7] for device requirements. In that WF an agreement on the Transmit output power. In this paper we discuss how a realistic design for the backscattering device 1 could be accommodated via RAN4 requirements.

# 2. 	Discussion on backscattering loss

WF agreement in question is as follows:

Issue 3-2-1: Transmit output power

Agreement:

- Measurement configuration including input CW power and potential other conditions need to be defined.

In the agreement, no discussion of the realistic design aspects was captured. RAN1 has been assuming 6 dB loss in their link level analysis but half of is due to the modulation loss and 3 dB was reserved for implementation losses. RAN4 should discuss what is a realistic assumption for the backscattering loss and for that, some design assumptions should be discussed

## 2.1	Modulation loss

If we assume CW is modulated with equal 1’s and 0’s in ASK, then half of the time the CW is not reflected back. The principle is illustrated in Figure 1

Figure 1. Modulated OOK signal in BB – yellow; ideal reflected RF waveform with modulation – blue; reflected waveform with non-ideal backscattering circuitry – red. The RF frequency is greatly lower frequency than real frequency for illustrative purposes

The reflected waveform is shon in Figure 1 with red. The peak power is attenuated by 15.5 dB from the ideal waveform and when the device is absorbing the signal, reflected power is 27.8 dB below the ideal power. The circuitry non-idealities are discussed in section 2.3.

The modulation work in such way that compared to the continuous CW signal, expressing 1’s and 0’s means that the CW is not reflected for 50 % of the time. In RF domain, this means 3 dB loss of power.

Observation 1: Ambient IoT device loses 3 dB of reflected power only because of modulation process

## 2.2	Antenna efficiency

TR  assumes 0 dBi antenna, which maybe a good assumption if conducted requirements were to be defined but a small sized tag in a sub 1GHz frequency will have losses.

Figure 1. Snippet from TR 38.769 for device antenna assumption’s part

For normative requirements an assumption of greater than 0 dB losses should be used.

Proposal 1:  Use -7 dB as antenna efficiency for Ambient IoT device 1 in normative work

## 2.3	Backscattering circuitry loss

Also called matching inaccuracies, in an ideal case, the reflection circuitry either reflects all the power back or then absorbs it. Antenna from the air interface in ideal conditions would look like a perfect reflector or a perfectly matched load. In addition to the antenna efficiency discussed in section 2.2,  the circuitry is not perfect open or perfectly matched.

Proposal 2: Ambient IoT device 1 matched load reflection coefficient is -8 dB

Proposal 3: Ambient IoT device 1 unmatched load reflection coefficient is -1 dB

## 2.4	Conclusion on backscattering loss

Adding the above numbers together yields the power backscattering loss is 18.5 dB. If the residual power is added due to non-ideal load then that adds 0.25 dB of power and resulting loss is 18.25 dB but we propose to omit that aspect for now.

Proposal 4: Ambient IoT device 1 backscattering loss used to define normative requirements is 18.5 dB.

## 2.5	Side conditions for transmit output power

The backscattering loss can not be specified without specifying side conditions such as the input power where the output power is measured. Since the device is assumed to operate in various distances from the reader and node providing CW, at least two values should be defined and they will then form an operating range of the device.

Proposal 5: Define a range of CW input power for Ambient IoT transmit output power requirement.

At this point of time, the range is TBD waiting for conclusion on reference sensitivity. The lowest level of input CW power should be aligned with the lowest detectable level of R2D signal.

# 3 Discussion on SFO

Sample Frequency Offset is the offset from the defined sample frequency of 1.92 MHz. The offset can be observed in two ways, 1) in time domain as longer than expected symbol time and e.g. time interval between repeated transmissions or 2) frequency offset The device creates a small frequency offset and RAN1 is discussing how many different offset the device will support. In Figure 3 we show spectrum of the device output with four different offsets.

RAN1 is currently assuming 10 % initial device SFO. This would mean that the 16 x frequency shifted spectrum in Figure 3 is shifted more than the one full sample frequency, 1.6 times more. It maybe challenging to detect the signal this way but it also makes the test easy: set the device to maximum possible small frequency shift and then measure center of the transmitted spectrum.

Alternative way, by using the time domain measurements is also easy if device is set to repeat the message 2 multiple times. Observing the preamble from the last repeated message multiplies the error.

Observation 2: By using maximum frequency shift or time domain repetition it is easy to observe the SFO.

Before defining the test or even the requirement, Ran4 needs to wait for RAN1 conclusion on the max SFO, max repetitions and maximum frequency shifts.

Figure 3. Ambient IoT device spectrum with four different small frequency shifts.

RAN4 should discuss and possibly even liase RAN1 on feasible methods for SFO detection.

Proposal 6: RAN4 to discuss feasible method candidates how to specify device SFO requirement

# Conclusion

We discussed various device requirements and made the following 7 observations

Observation 1: Ambient IoT device loses 3 dB of reflected power only because of modulation process

Observation 2: By using maximum frequency shift or time domain repetition it is easy to observe the SFO.

Proposal 1:  Use -7 dB as antenna efficiency for Ambient IoT device 1 in normative work

Proposal 2: Ambient IoT device 1 matched load reflection coefficient is -8 dB

Proposal 3: Ambient IoT device 1 unmatched load reflection coefficient is -1 dB

Proposal 4: Ambient IoT device 1 backscattering loss used to define normative requirements is 18.5 dB.

Proposal 5: Define a range of CW input power for Ambient IoT transmit output power requirement.

Proposal 6: RAN4 to discuss feasible method candidates how to specify device SFO requirement

# Reference

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2420470 “WF on UE RF requirements for A-IoT”, Huawei, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, United States,18 – 22 Nov., 2024

[3]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP

[4]	R4-2500402, “Discussion on Ambient IoT deployment scenario in Rel-19”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2500401, “Discussion on Ambient IoT Testability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[6]	R4-2500490, “Ambient IoT device requirements”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[7]	R4-2502858, “WF on requirements for system parameters and A-IOT device”,  CMCC, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17– 21 February, 2025