3GPP TSG-RAN4 Meeting #114-bis	R4-2503602

# Wuhan, China, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.908 | CR | 0002 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [C1-1 focus] Spec An MU to be updated for n104 band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Spec An(Spectrum Analyzer) instrument would need higher MU allowance for the newer n104 band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increase MU allowance from 0.26 dB to 0.50 dB for C1-1 line item |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | We are concerned that locking in the MU value of 0.26 dB used for lower carrier frequencies will bring down measurement quality of the new test case for U6GHz 6.9.5 of 38.141-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 37.941 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.141-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | In RAN4#114(Athens), we were able to include PWS as an acceptable method for U6GHz EEIRP Test case (6.9.5 in 38.141-2).The MU values were considered TBD back then.Hoping to finalize the Spectrum Analyzer MU to a more practical value of 0.5 dB considering the higher frequency of usage here. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

## 6.6	Measurement uncertainty

The EEIRP measurement is very similar to the in-band TRP measurements, as such the in-band emission MU budgets from TR 37.941 can be used as a starting point for the EEIRP budgets.

Additional sources of uncertainty are:

Dynamic range uncertainty

One major difference with EEIRP, compared to TRP, is the necessity to measure complete EIRP pattern with focus on the sidelobe region (i.e. for a real-life site this is the region pointing towards the sky). This means measurement receiver must be considered in such way that the dynamic range can handle the power created by the peak main beam as the maximum level and detecting the low levels in the side lobe region with accurate power. The power level accuracy is typically different for high power with large SNR compared to coming closer to the noise floor of the measurement receiver.In addition, to avoid the impacts (Reflections and refractions of the main beam within the chamber) from main beam to sidelobe beam, the chamber spatial isolation is necessary to remove the effect of the main beam.

The lowest EEIRP requirement is 15dBm/MHz

Currently the dynamic range contribution is 0.51dB, in order to maintain that level of accuracy in the above scenario an isolation of approximately 58dB under the assumption of peak EIRP 63.7dBm for main beam would be required:

Pint=10*log101063.7-5810+101510=15.5dBm/MHz i.e. 0.5dB error

EEIRP sampling Error

The EEIRP sampling error is caused by the fact that coverage steering range is sampled with a limited set of test beams.

The measure EEIRP profile using different granularity of test beams distributed within the setting range will proceed error within EEIRP bin.

According to the agreed test beam pattern, errors between the EEIRP simulation results of test beam and EEIRP simulation results of reference testing beams (e..g 2.4K or 8K or 10K) are provided in the following table.

Table 6.6.-1.Errors per bin between the EEIRP simulation results of test beam and EEIRP simulation results of reference testing beams

| ZTE |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ant 4, 12degree | Error for Rect Ref | -0.98 | -0.17 | -0.70 | -0.28 | -0.30 | 0.04 | 0.36 |
|  |  | Error for Ellipse Ref | 0.25 | -0.22 | -0.03 | 0.07 | 0.21 | -0.11 | -0.40 |
|  | Ant 4, 16degree | Error for Rect Ref | -1.51 | -1.38 | -0.78 | 0.64 | -0.42 | -0.20 | 0.54 |
|  |  | Error for Ellipse Ref | -0.09 | -0.97 | 0.38 | -0.03 | 0.20 | -0.05 | -0.54 |
|  | Ant 1, 12degree | Error for Rect Ref | -0.98 | -0.17 | -0.70 | -0.28 | -0.30 | 0.04 | 0.36 |
|  |  | Error for Ellipse Ref | 0.25 | -0.22 | -0.03 | 0.07 | 0.21 | -0.11 | -0.40 |
|  | Ant 1, 16degree | Error for Rect Ref | -1.13 | -0.09 | 0.03 | 0.09 | 0.14 | 0.33 | 0.72 |
|  |  | Error for Ellipse Ref | 0.14 | 0.10 | 0.03 | 0.02 | -0.01 | -0.06 | -0.12 |
| Huawei |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant 4, 12degree | Error for Rect Ref | -1.25 | -0.32 | -1.15 | -0.81 | -0.46 | -0.26 | 0.36 |
|  |  | Error for Ellipse Ref | 0.28 | 0.07 | 0.01 | 0.07 | 0.17 | 0.10 | -0.40 |
|  | Ant 4, 16degree | Error for Rect Ref | -0.44 | -1.12 | -0.82 | 0.38 | -0.77 | -0.34 | 0.37 |
|  |  | Error for Ellipse Ref | 0.43 | 0.27 | -0.06 | -0.01 | 0.10 | 0.03 | -0.48 |
|  | Ant 1, 12degree | Error for Rect Ref | -1.08 | -0.18 | -0.05 | 0.01 | 0.05 | 0.28 | 0.66 |
|  |  | Error for Ellipse Ref | 0.14 | 0.01 | 0 | 0 | 0.02 | -0.03 | -0.15 |
|  | Ant 1, 16degree | Error for Rect Ref | -1.15 | -0.34 | -0.17 | -0.09 | -0.01 | 0.25 | 0.60 |
|  |  | Error for Ellipse Ref | -0.01 | 0.01 | 0.03 | 0.04 | 0.03 | 0 | -0.18 |
| Ericsson |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant 4, 16degree | Error for Rect Ref | -1,35 | -0,43 | 1,03 | 0,7 | 0,82 | 0,49 | 1,96 |
|  |  | Error for Ellipse Ref | 0,48 | -0,19 | 0,2 | 0,57 | 0,11 | 0,15 | 1,07 |
|  | Ant 1, 16degree | Error for Rect Ref | -0,66 | -0,311 | 0,04 | 0,05 | 0,09 | 0,23 | 0,63 |
|  |  | Error for Ellipse Ref | 0,41 | -0,3 | 0,1 | 0,17 | 0,06 | 0,02 | 0,06 |
| Nokia |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant 1, 12degree | Error for Rect Ref | -1.47 | -0.12 | -0.33 | -0.15 | 0.06 | 0.28 | 0.90 |
|  |  | Error for Ellipse Ref | 1.16 | -0.10 | -0.26 | -0.12 | 0.04 | 0.13 | 0.62 |

In the following section, the probability distribution curve for error across different antenna configuration and steering range is provided to check the distribution function. Meanwhile the probability distribution curve for error among all 7 bins across different antenna configuration and steering range is provided to check the distribution function.

These two errors have been added to the MU budget as follows:

| Table 6.6-1: IAC MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-1 | Positioning misalignment between the BS and the reference antenna |  |  | 0.03 | Rectangular | 1.73 | 1 |  |  |  |  | 0.02 |  |
| A1-2 | Pointing misalignment between the BS and the receiving antenna |  |  | 0.3 | Rectangular | 1.73 | 1 |  |  |  |  | 0.17 |  |
| A1-3 | Quality of quiet zone |  |  | 0.1 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4a | Polarization mismatch between the BS and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5a | Mutual coupling between the BS and the receiving antenna |  |  | 0 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.50 | Gaussian | 1.00 | 1 |  |  |  |  | 0.50 |  |
| A1-7 | Impedance mismatch in the receiving chain |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A1-8 | Random uncertainty |  |  | 0.1 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| A2-21 | Measurement system dynamic range uncertainty |  |  | [0.5] | Rectangular | 1 | 1 |  |  |  |  | [0.3] |  |
| *new | [EEIRP sampling error] |  |  | [TBD] | [TBD] | 1 | 1 |  |  |  |  | [TBD] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-9 | Impedance mismatch between the receiving antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-11 | Impedance mismatch between the reference antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-3 | Quality of quiet zone (normal test conditions) |  |  | 0.10 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4b | Polarization mismatch between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5b | Mutual coupling between the reference antenna and the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-3 | Uncertainty of the network analyzer |  |  | [0.20] | Gaussian | 1.00 | 1 |  |  |  |  | [0.20] |  |
| A1-12 | Influence of the reference antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A1-13 | Reference antenna feed cable loss measurement uncertainty |  |  | 0.06 | Gaussian | 1.00 | 1 |  |  |  |  | 0.06 |  |
| A1-14 | Influence of the receiving antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A1-15 | Uncertainty of the absolute gain of the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [0.95] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [1.87] |
|  | # <End of change 1> # <Start of change 2> |  |  |  |  |  |  |  |  |  |  |  |  |
| Table 6.6-2: CATR MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) |  |  | 0.30 | Rectangular | 1.73 | 1 |  |  |  |  | 0.173 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.50 | Gaussian | 1.00 | 1 |  |  |  |  | 0.50 |  |
| A2-2a | Standing wave between BS and test range antenna |  |  | 0.21 | U-shaped | 1.41 | 1 |  |  |  |  | 0.148 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.001 |  |
| A2-4a | QZ ripple experienced by BS |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.093 |  |
| A2-12 | Frequency flatness of test system |  |  | 0.25 | Gaussian | 1.00 | 1 |  |  |  |  | 0.250 |  |
| A2-17 | Measurement system dynamic range uncertainty |  |  | [0.5] | Rectangular | 1 | 1 |  |  |  |  | [0.3] |  |
| *new | [EEIRP sampling error] |  |  | [TBD] | [TBD] | 1 | 1 |  |  |  |  | [TBD] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A2-6 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A2-7 | Influence of the calibration antenna feed cable |  |  | 0.02 | U-shaped | 1.41 | 1 |  |  |  |  | 0.02 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A2-8 | Misalignment positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) |  |  | 0.50 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.25 |  |
| A2-9 | Rotary joints |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.03 |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  |  | 0.09 | U-shaped | 1.41 | 1 |  |  |  |  | 0.06 |  |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) |  |  | 0.01 | Gaussian | 1.00 | 1 |  |  |  |  | 0.01 |  |
| A2-11 | Switching uncertainty |  |  | 0.26 | Rectangular | 1.73 | 1 |  |  |  |  | 0.15 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [1.03] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [2.03] |

| Table 6.6-3: PWS MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A7-1a | Misalignment and pointing error of BS |  |  | 0.10 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.50 | Gaussian | 1.00 | 1 |  |  |  |  | 0.50 |  |
| A7-2a | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for BS antenna |  |  | 0.20 | Rectangular | 1.73 | 1 |  |  |  |  | 0.12 |  |
| A7-3 | RF leakage (calibration antenna connector terminated) |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.09 |  |
| A7-4a | QZ ripple experienced by BS |  |  | 0.77 | Rectangular | 1.73 | 1 |  |  |  |  | 0.44 |  |
| A7-5 | Miscellaneous uncertainty |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A7-14 | System non-linearity |  |  | 0.22 | Rectangular | 1.73 | 1 |  |  |  |  | 0.13 |  |
| A7-13 | Frequency flatness of test system |  |  | 0.13 | Rectangular | 1.73 | 1 |  |  |  |  | 0.08 |  |
| A2-21 | Measurement system dynamic range uncertainty |  |  | [0.50] | Rectangular | 1.73 | 1 |  |  |  |  | [0.30] |  |
| *new | [EEIRP sampling error] |  |  | TBD | [TBD] | 1 | 1 |  |  |  |  | TBD |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A7-6 | Mismatch (i.e. reference antenna, network analyser and reference cable) |  |  | 0.45 | U-shaped | 1.41 | 1 |  |  |  |  | 0.32 |  |
| A7-7 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A7-3 | RF leakage (calibration antenna connector terminated) |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.09 |  |
| A7-8 | Influence of the calibration antenna feed cable |  |  | 0.10 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A7-9 | Misalignment of positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A7-1b | Misalignment and pointing error of calibration antenna |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A7-10 | Rotary joints |  |  | 0.00 | U-shaped | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A7-2b | Longitudinal position uncertainty (i.e. standing wave and imperfect field synthesis) for calibration antenna |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A7-4b | QZ ripple experienced by calibration antenna |  |  | 0.20 | Rectangular | 1.73 | 1 |  |  |  |  | 0.12 |  |
| A7-11 | Switching uncertainty |  |  | 0.02 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A7-12 | Field repeatability |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | TBD |

<End of change 2>