3GPP TSG-RAN WG4 Meeting # 114bis	                                      R4-2503604

Wuhan, CN, April 7 – 11, 2025

Agenda Item: 7.27.2.2

Source: MediaTek Inc.

Title: Discussion on CSI-RS L1 measurement for LTM

Document for: Discussion

- Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of CSI-RS L1 measurements.

- Discussion

2.1 known cell definition in CSI-RS based L1 measurement

In RAN4#112bis, RAN4 agreed that RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells. In the past two meetings, RAN4 discussed known cell definition as shown below. In the next, we would like to discuss the remaining parts.

RAN4#112bis Issue 2-1: CSI-RS based L1 measurement for known/unknown cell - Agreement: - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells Issue 2-2: known cell definition in CSI-RS based L1 measurement For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition RAN4#113 Issue 3-1-1: Whether to consider SSB-based L3 measurement only? < Agreement > - Consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition.

Regarding known cell definition, the first bullet is to guarantee that UE has detected the cell and acquired the rough timing & frequency of the cell.

The second bullet is to make sure the associated SSB of the CSI-RS is detectable. As proposed in Proposal 3 in 2.2, CSI-RS for L1 measurement shall be Type-D QCL to SSB for L1 measurement in FR2-1.

Proposal 1: Keep L1 measurement for FR2-1 in the second condition of known cell definition in CSI-RS based L1-RSRP measurement.

The third bullet is to guarantee the channel quality of CSI-RS is not too bad. Regarding the third bullet, we prefer to use a side condition. CSI-RS is not used for detection. It is not accurate to use the wording “Detectable”.

Proposal 2: The third condition of known cell definition in CSI-RS based L1-RSRP measurement is: SNR of the CSI-RS from the target cell configured for L1 measurement is above -3dB.

2.2 Measurement procedure of CSI-RS based L1 measurement

In the past meetings, RAN4 had extensive discussions on the measurement procedure of CSI-RS based L1 measurement. In RAN4#113, the following agreement was reached.

Issue 3-2: Measurement procedure <Agreement> in RAN4#113 - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

Last meeting, after discussion, we had the following options:

Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 - Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

Although most of the companies are open to Option A. But as the candidate cell may also be a serving cell, e.g., PSCell or SCell, then there will be two totally different measurement procedures for the same serving cell. From UE side, it is not acceptable to have two totally different CSI-RS measurement procedures for the same cell. UE will get confused on how to perform the measurement. So we still prefer Option B.

Proposal 3: For CSI-RS based L1-RSRP measurement on neighbor cell in FR2-1, only consider the following procedure:

- Step 1: SSB based L3 measurements

- Step 2: SSB-based L1 measurements and reports on the same candidate cell

- Step 3: CSI-RS based L1 measurements on the same candidate cell

  - Note: The CSI-RS in Step 3 should be Type-D QCL with the SSB in Step 2 and configured with repetition OFF.

2.3 Definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement

In the past meetings, RAN4 discussed whether and how to define intra-frequency for CSI-RS L1 measurement and reached the following progress.

Issue 4-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement Candidate options: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

As the definition is only for the convenience of capturing the requirements, we can be a bit open. We understand the intention of option 1 is to align with legacy intra-frequency definition of L3 CSI-RS measurement and SSB based measurement. But if we look back the legacy intra-f and inter-f measurement in LTE, intra-f is always measured without gap and inter-f is measured with gap. We understand due to configuration flexibility that there are intra-f without gap, intra-f with gap, inter-f with gap and inter-f without gap in NR. But these concepts usually confuse others who are not familiar with RRM spec. Comparing the two options, we still prefer Option 2.

Proposal 4: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

2.4 The case to define requirements for

Issue 4-4: Measurement type Issue 4-7: RTD assumption Candidate options: - Option 1: CMCC, ZTE, CATT, China Telecom - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - Define requirements for RTD > CP, as optional UE feature. FFS details - Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - No Ran4 requirements for RTD > CP. - Option 3: - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - Define requirements for support RTD > CP, as optional UE feature. FFS details - Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - No Ran4 requirements for RTD > CP. Agreement: - Conclude this issue in the next meeting.

Last meeting, there were lots of discussions on whether to define requirements for RTD>CP case and/or CSI-RS measurement with gap.

Single FFT is assumed for multiple cell measurements for CSI-RS based L3 measurement. For SSB based L1 measurement, RTD>CP case is supported with UE capability. In previous meeting, RAN4 agreed that “For CSI-RS based L1-RSRP measurement on neighbor cell, define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs”. If to support RTD>CP case for CSI-RS based L1 measurement, another FFT is needed to cover as wide as 48PRBs, while SSB only occupies 20PRBs. The cost will be much higher to support RTD>CP for CSI-RS compared to SSB. In addition, compared to SSB based L1 measurement, we don’t see much gain to support CSI-RS based L1 measurement. It is sufficient to use SSB based L1 measurement. We propose not to support RTD>CP case for CSI-RS based L1-RSRP measurement.

For CSI-RS L1 measurement with gap, as the gap occasion should be shared among SSB based L3 measurement, SSB based L1 measurement and CSI-RS based L3 measurement. The measurement delay would be very long. We doubt whether this feature is useful. In addition, last meeting, we agreed that “In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration.” If there is no restriction on measurement timing configuration, we don’t think it is possible to cover all the CSI-RS resources with a single MG. During the discussion, there were arguments that this is in WID. In our understanding, CSI-RS L1 measurement is in the WID. RAN4 still can make the decision on which type to define requirements for.

Proposal 5: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case and CSI-RS based L1-RSRP measurement with gap in R19.

2.5 RRM requirements for CSI-RS based L1-RSRP measurement without gap when RTD<CP

For CSI-RS based L1-RSRP measurement without gap when RTD<CP, UE can do the measurement on different cells simultaneously in FR1. Measurement delay of serving cell can be reused, i.e., max(TReport, ceil(M*P)*TCSI-RS). Measurement restriction and scheduling restriction requirements can also be reused.

Proposal 6: For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR1, legacy measurement period, measurement restriction and scheduling restriction requirements are all applicable.

In FR2, due to Rx beam for CSI-RS on different cells may be different, UE may not be able to perform CSI-RS L1-RSRP measurement on different cells simultaneously. In legacy requirements, when two CSI-RS resources overlap in time domain and require different Rx beam, measurement restriction apply. This is also applicable when two CSI-RS resources from difference cells overlap in time domain. Then legacy measurement period and scheduling restriction are also applicable.

Proposal 7: For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, define measurement restriction when two CSI-RS resources from difference cells overlap in time domain.

Proposal 8: For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, legacy measurement period and scheduling restriction requirements are both applicable.

2.6 Others

For CSI acquisition, RAN1 had reached the following agreement.

- Baseline: - CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE - The report is sent directly to target cell - Optional with a UE capability: - CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

For the baseline (CSI acquisition after LTM CSC MAC-CE reception), UE will acquire CSI after UE is ready to receive data on target cell. We do not see any RRM requirement to be defined for this case.

For the optional feature (CSI acquisition before LTM CSC MAC-CE reception), it is far more complex than CSI-RS L1-RSRP measurement. Similar issues with CSI-RS based L1-RSRP measurement from the candidate cell should be discussed, such as the pre-requisite condition, maximum RTD value, gap-less vs. gap-based, etc. But not all the agreements for CSI-RS L1-RSRP measurement are applicable here. For example, UE can perform L1-RSRP measurement on truncated CSI-RS resources (48 RBs), but this may be not workable for CSI acquisition.

xx

- Summary

In this paper, we provide some views on the R19 mobility. We have the following observation and proposals:

Proposal 1: Keep L1 measurement for FR2-1 in the second condition of known cell definition in CSI-RS based L1-RSRP measurement.

Proposal 2: The third condition of known cell definition in CSI-RS based L1-RSRP measurement is: SNR of the CSI-RS from the target cell configured for L1 measurement is above -3dB.

Proposal 3: For CSI-RS based L1-RSRP measurement on neighbor cell in FR2-1, only consider the following procedure:

- Step 1: SSB based L3 measurements

- Step 2: SSB-based L1 measurements and reports on the same candidate cell

- Step 3: CSI-RS based L1 measurements on the same candidate cell

  - Note: The CSI-RS in Step 3 should be Type-D QCL with the SSB in Step 2 and configured with repetition OFF.

Proposal 4: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Proposal 5: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case and CSI-RS based L1-RSRP measurement with gap in R19.

Proposal 6: For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR1, legacy measurement period, measurement restriction and scheduling restriction requirements are all applicable.

Proposal 7: For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, define measurement restriction when two CSI-RS resources from difference cells overlap in time domain.

Proposal 8: For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, legacy measurement period and scheduling restriction requirements are both applicable.

4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#113.