3GPP TSG-RAN WG4 Meeting # 114bis	                                      R4-2503610

Wuhan, CN, April 7 – 11, 2025

Agenda Item: 8.2

Source: MediaTek Inc.

Title: Discussion on the question from RAN2 on R18 LTM

Document for: Discussion

- Introduction

In RAN2 LS R4-2503115/R2-2501392, they asked a question on granularity definition of pdcch-RACH-AffectedBandsList-r18. We will discuss our view in this contribution.

- Discussion

For the convenience of discussion, we copy the content of R2 LS here:

The UE capability pdcch-RACH-AffectedBandsList-r18 (FG39-4) is defined in RAN4 with the granularity as “Per band pair per band combination (between the target band for RACH transmission and band under UE’s current band combo)” For each reported capability, there are two possible values: interruption or noInterruption. Regarding the band pair (one source band +one target band) of the BC, RAN2 understands that the target band refers to the target band towards which the UE performs PDCCH-ordered early RACH. However, the definition of the source band of the band pair remains unclear in RAN2. - Understanding 1: The source band of the band pair represents the source band (or PCell) on which the PDCCH order is sent to the UE to perform PDCCH-ordered early RACH to the target cell/band. Regarding the interruption band(s), there are two further understandings, denoted by 1a and 1b: - Understanding 1a: The value “interruption/noInterruption” means that interruption is needed or not for all serving bands of the current band combination. - Understanding 1b: The value “interruption/noInterruption” means that interruption is needed or not for the source band (the same band in which the PDCCH order is sent) of the current band combination. For other serving bands, noInterruption can be always assumed. - Understanding 2: The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band. Please note that in Understanding 1, the UE cannot indicate which specific source band of the BC has an interruption or not (either all bands in current BC in 1a or the same source band that receives the PDCCH order in 1b). In Understanding 2, the information about which source band receives the PDCCH order is not provided.

The capability FG39-4 is to indicate whether UE will cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission. For UEs with different UE capabilities, it is possible there is DL interruption on one serving band but not on another band as shown in the following Table. That’s why RAN4 agreed the granularity is “Per band pair per band combination (between the target band for RACH transmission and band under UE’s current band combo)”. Understanding 2 is aligned with RAN4’s intention.

| Serving cell | Target cell | UE capability | Interruption on DL or not |
| --- | --- | --- | --- |
| Inter-band CA f1 on band A f2 on band B | f3 on band C | UE supports f1+f3 CA, and does not support f2+f3 CA. | Maybe no interruption on DL in f1 Interruption on DL in f2 |
|  |  | UE supports f2+f3 CA, and does not support f1+f3 CA | Interruption on DL in f1 Maybe no interruption on DL in f2 |
|  |  | UE supports f1+f3 CA and f2+f3 CA, but does not support f1+f2+f3 CA | At least have interruption on DL in either f1 or f2. Depend on reported UE capability. |
|  |  | UE supports f1+f2+f3 CA | Maybe no interruption on DL in both f1 and f2 |
|  |  | UE does not support either f1+f3 CA or f2+f3 CA | Interruption on DL in both f1 and FR2 |

In RAN2 LS, it seems they have some concern on understanding 2 due to the information about which source band receives the PDCCH order is not provided. In our understanding, the capability reflects whether UE can transmit RACH on the candidate cell and receive DL on a particular serving band simultaneously. The band that UE receives the PDCCH order has no impact on the capability. So it is not necessary to indicate which band UE receives the PDCCH order.

Proposal 1: For the UE capability pdcch-RACH-AffectedBandsList-r18 (FG39-4),

- RAN4’s understanding is understanding 2, i.e.,

- The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band.

- Summary

In this paper, we provide our views on the open issues of R18 L1/L2 triggered mobility. We have the following proposals:

Proposal 1: For the UE capability pdcch-RACH-AffectedBandsList-r18 (FG39-4),

- RAN4’s understanding is understanding 2, i.e.,

- The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band.

Annex

3GPP TSG-RAN WG4 Meeting #114	bis					                  R4-25xxxxx

Wuhan, CN, 7th – 11th April, 2025

Title:	Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18

Response to:	R2-2501392/ R4-2503115

Release:	Release 18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:	Ada Wang

E-mail Address:	< ada.wang@mediatek.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN4 thanks RAN2 for the LS on granularity definition of pdcch-RACH-AffectedBandsList-r18. RAN4 discussed the question from RAN2 and reached the following consensus:

- RAN4’s understanding is understanding 2.

2. Actions:

To RAN2.

ACTION: 	RAN4 kindly asks RAN2 to take above information into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #115		              19 - 23 May 2025 			     Malta

TSG RAN WG4 Meeting #116		              25 - 29 August 2025			  Bengaluru, India