3GPP TSG-RAN WG4 Meeting #114bis										          R4-2503611

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.1.4

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [114bis][201] NR_ENDC_RF_Ph4

Document for:	Information

Introduction

This email thread discusses the RRM requirements of WI on R19 NR_ENDC_RF_Ph4.

Online handling

(Online) Issue 2-1: Antenna connection configuration for 6 Rx capable UE

(Online) Issue 2-2: Whether a new clause of antenna connection for 6 Rx capable UEs is to be introduced

(Online) Issue 2-3: SNR level for RLM and BFD/CBD testing for 6Rx capable UE

Topic #1:	RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503891 | Huawei, HiSilicon | Proposal 1: RLM and BFD testing for 6Rx capable UEs reuse SNR levels specified for 4Rx capable UE. Proposal 2: A new clause of antenna connection for 6 Rx capable UEs is to be introduced, the requirements of “A.3.6.1.2 Antenna connection for 8 Rx capable UEs” shall be applied, with “8RX” being replaced with “6RX”. |
| R4-2504374 | Ericsson | Observation 1: Existing interruption requirements at SRS antenna port switching were reused for 6Rx UEs. Proposal 1: New antenna configurations are to be defined for 6 Rx capable UEs targeting single carrier operation using the existing 4Rx configurations as reference. Proposal 2: RAN4 to discuss the modified SNR levels for RLM and BFD tests for 6 Rx capable UEs. |

Open issues summary

Sub-topic 1: RRM core requirements for 6Rx capable UE

Issue 1-1: Interruption requirements at SRS antenna port switching

- Proposals

  - Option 1(Ericsson): Existing interruption requirements at SRS antenna port switching were reused for 6Rx UEs.

- Recommended WF

In RAN4#112bis meeting, the follow agreement was made [R4-2416848].

<Agreement> Issue 1-1: Interruption requirements at SRS antenna switching for 6Rx capable UE Agreement: - The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed. - This can be revisited if the existing requirement cannot be applied to any SRS antenna switching pattern agreed in RAN1.

Option 1 further confirmed the agreement. No further discussion is needed.

Sub-topic 2: RRM performance requirements for 6Rx capable UE

(Online) Issue 2-1: Antenna connection configuration for 6 Rx capable UE

- Proposals

  - Option 1(Huawei): New antenna configurations are to be defined for 6 Rx capable UEs, where antenna connection for 8Rx capable UEs in A.3.6.1.2 shall be applied with “8RX” being replaced with “6RX”.

  - Option 2(Ericsson): New antenna configurations are to be defined for 6 Rx capable UEs targeting single carrier operation using the existing 4Rx configurations as reference.

- Recommended WF

New antenna connection configurations are to be defined for 6 Rx capable UE, taking the antenna connection configuration of existing 8RX capable UE as reference.

(Online) Issue 2-2: Whether a new clause of antenna connection for 6 Rx capable UEs is to be introduced

- Proposals

  - Option 1(Huawei): A new clause of antenna connection for 6 Rx capable UEs is to be introduced

- Recommended WF

Further discussion.

(Online) Issue 2-3: SNR level for RLM and BFD/CBD testing for 6Rx capable UE

- Proposals

  - Option 1(Huawei): RLM and BFD testing for 6Rx capable UEs reuse SNR levels specified for 4Rx capable UE.

  - Option 2 (Ericsson): RAN4 to discuss the modified SNR levels for RLM and BFD tests for 6 Rx capable UEs.

- Recommended WF

Further discussion.