3GPP TSG-RAN WG4 Meeting #114bis       				                                      	R4-2503629

Wuhan, CN, 7th-11st April 2025

Agenda item:			7.27.3

Source:	China Telecom

Title:	Topic summary for [114bis][219] NR_Mob_Ph4_Part2

Document for:	Information

Introduction

This summary includes the proposals from companies on RRM requirements for conditional LTM. The relevant objectives in the latest WID and RAN2’s conclusions are duplicated here for information:

Objectives on conditional Intra-CU LTM in WID - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

RAN2’s conclusions in last meeting 1. From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. 2. The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. 3. For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell). 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM. 7. Do not support DG-based first UL transmission for RACH-less conditional LTM. 8. For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed. 9. For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions. 10. Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-2-3: The components of conditional LTM delay

Issue 1-2-4: Tinterrupt

Issue 1-2-8: TCLTM_execution

Issue 1-2-9: TLTM-RRC-processing

Issue 1-2-11: Tfirst-RS / TRS

Issue 1-2-12: TRS-proc

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement

Issue 1-2-6: TEvent_DU

Issue 1-2-7: Tmeasure

Issue 1-2-15: Other ad hoc

Issue 1-2-14: Twait_RS ad hoc

Moderator’s recommendation is also provided under issue.

Topic #1: RRM discussion

Companies’ contributions summary

| TDoc | Source | Proposals |
| --- | --- | --- |
| R4-2503989 | China Telecom | Proposal 1: Define RRM requirements for UE based TA measurement. Proposal 2: DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. Where: - TRRC is the RRC procedure delay. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. - Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts. - TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. - Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell. - TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during LTM cell switch. Proposal 3：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions. Proposal 4：CR skeleton Expected impact; Notes; New or impacted section; Companies. Definitions and abbreviations; 3.1 3.3. CLTM delay requirements; Depending on RAN2 conclusion; (new) 6.x; China Telecom. UL transmit timing requirements; TBD: UE based TA measurement; 7.1 7.3. PDCCH-order RACH on neighbor cell; Reuse the related R18 RAN4 requirements; 6.2.2C. TCI state activation; Reuse the related R18 RAN4 requirements; 8.25. L1-RSRP measurement; Reuse the related R18 RAN4 requirements; 9.14 9.15. L3 measurement based CLTM. |
| R4-2503303 | Xiaomi | Proposal 1: CHO requirements framework can be used as baseline for Conditional LTM. Proposal 2: RAN4 to confirm that the starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. Proposal 3: RAN4 to define the conditional LTM delay as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution, where - TRRC is the RRC procedure delay, - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch, - Tmeasure is the measurement time. - For conditional LTM based on L3 measurements, Tmeasure can be the same as one measurement period of SSB based L3-RSRP measurements for neighbor cell. - For conditional LTM based on L1 measurements, Tmeasure can be the same as one measurement period of SSB based L1-RSRP measurements for neighbor cell. - Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell. Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU, where the TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU can refer to clause 6.3.1.3, - TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, including both RACH based and RACH less cases, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. |
| R4-2504052 | vivo | Proposal 1: RAN4 to define RRM requirements for UE-based TA measurement in CLTM Proposal 2: RAN4 to define the timeline of CLTM cell delay requirement as follows. The definition and corresponding value of each delay component can be further determined. DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms Proposal 3: For CLTM, the definition and corresponding value of exsisting TRRC in CHO can be reused Proposal 4: For CLTM, TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. Specifically, it can cover both cases: - UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation - Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met. Proposal 5: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. - When the condition is L1-based: - For SSB based L1-RSRP measurement, RAN4 to use the Rel-18 L1 measurement period for neighbor cell to define Tmeasure. - If CSI-RS based L1-RSRP measurement requirements is defined in Rel-19 LTM, it can be reused in Tmeasure - When the condition is L3-based: Tmeasure to follow the definition of Tmeasure for CHO. Proposal 6: For subsequent conditional LTM, Apart from the starting point of the delay, the delay requirement on conditional LTM can be reused to define the remaining components of subsequent CLTM. |
| R4-2503435 | CATT | Proposal 1: The requirements for deactivated SCell and the cells with same frequency can be defined only in R19 CLTM if time is allowed. Proposal 2: The legacy delay requirments for L3 CHO and R18 LTM can be a baseline, that is, DCLTM = TRRC + TEvent_DU + Tmeasure+ TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU+ TCLTM_execution. Proposal 3: For interruption time, the optimization like option 1 can be introduced in the subsequent release, and companies need to further clarify the necessity of optimization for reducing interruption delay in CLTM. Proposal 4: At least TRRC will not be considered in subsequent conditional LTM delay. Proposal 5: TRRC is the RRC procedure delay [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM]. Proposal 6: The following general description in proposal 1 is ok for us: - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. Proposal 7: Tmeasure can refer to proposal 1a. Proposal 8: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. Proposal 9: RAN4 to focus on how to define the feasible requirements for UE based TA measurement for conditional intra-CU LTM. |
| R4-2503759 | CMCC | Proposal 1: For the case which CLTM is triggered by L1 measurement, it is proposed to define measurement requirements for L1 measurement on deactivated SCC and the cells with same frequency as deactivated SCell. Proposal 2: since RAN2 is agreed that UE based TA measurement mechanism is supported for conditional intra-CU LTM, it is proposed to define related RAN4 requirements for UE based TA adjustment. Proposal 3: for conditional LTM, cell switch delay is proposed as: DCLTM = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt , TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU , - TRRC_delay is the RRC procedure delay. - TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM. - Tmeasure is measurement time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18. - TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during CLTM cell switch Proposal 4: for subsequent conditional LTM, cell switch delay is proposed as: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. , TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU , - TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. - Tmeasure is measurement time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18. |
| R4-2504528 | Qualcomm Incorporated | UE based TA Acquisition Proposal 1: RAN4 to define RRM requirements for UE based TA measurement as follows: - Tx timing for the target cell = current Tx timing + 2 x (downlink frame boundary of reference cell – downlink frame boundary of the target cell), where the definition of ‘timing reference cell’ is the reference cell defined in the clause of ‘7.1 UE transmit timing’ in TS38.133. - FFS on the side conditions in terms of SNR, timeline for the downlink frame boundary estimation, etc. Sequence and Interruption during Conditional LTM Execution Proposal 2: RAN4 to not rearrange the sequence of components in the C-LTM cell switch latency requirements compared to the legacy CHO requirements. Specifically, the last SSB sample (conceptually used for fine time/frequency tracking) should occur before the start of the interruption. |
| R4-2503573 | Apple | Observation 1: RAN2 already agreed new design on how to handle TA for C-LTM. Therefore, C-LTM can work well without UE based TA measurement. Observation 2: UE based TA measurement has been widely discussed in R18. Eventually RAN4 realized that it is very challenging to define corresponding RAN4 requirements. Observation 3: UE based TA measurement is a R18 optional feature. It is not clearly in scope of RAN4 R19 requirements. Proposal 2: Do not define RRM requirements for UE based TA measurement in R19. Proposal 3: minimum requirements for CLTM delay: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms Proposal 4: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM. Proposal 5: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. |
| R4-2503605 | MediaTek Inc. | Proposal 1: For C-LTM, conditional cell switch delay and interruption can be defined as: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Twait_RS and TCLTM_interrupt = TRS + TRS-proc + TLTM-processing + TLTM-IU, where o TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. o TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional cell switch command until a condition exists at the measurement reference point which will trigger the conditional cell switch, o Tmeasure is the time for the UE to measure and realize the condition for Conditional cell switch is fulfilled. - For L3 event triggered conditional intra-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. - For L3 event triggered conditional inter-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9. - For L1 event triggered conditional intra-frequency cell switch, the measurement time delay shall be less than Tidentify intra with index defined in clause 9.2.5.1 plus TL1-RSRP_Measurement_Period_SSB_intra defined in clause 9.14.5.1. - For L1 event triggered conditional inter-frequency cell switch, the measurement time delay shall be less than Tidentify_inter_with_index defined in clause 9.3.4 plus TL1-RSRP_Measurement_Period_SSB_inter defined in clause 9.15. o TLTM-RRC-processing can be up to 10ms. o TLTM-processing is time for UE processing. Tprocessing can be up to 20 ms. o TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. o Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell. o TRS is time for fine time tracking and acquiring full timing information of the target cell. TRS can be up to 2ms. o Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms. Observation 1: Only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB. Proposal 2: Not to define the requirements for UE based TA measurement in RAN4 in R19. |
| R4-2503885 | Huawei, HiSilicon | Proposal 1: If companies would like to define L1 measurement requirements on deactivated SCell and the cells with same frequency as deactivated SCell, the following aspects need to be considered: - Whether interruptions exist on activated serving cells - Whether to extended L1 measurement period for deactivated SCC. Proposal 2: Conditional LTM delay can take legacy CHO delay as a starting point. Proposal 3: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. Proposal 4: TEvent_DU in CHO can be reused in conditional LTM delay, as it can cover both cases: 1) UE completed early TCI state activation and/or early UL synchronization before the execution condition is met 2) Neither early TCI state activation nor early UL synchronization is successfully performed before the execution condition is met. Proposal 5: In C-LTM delay, Tmeas is the L1 measurement period - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM. - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy. - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19. - FFS on CSI-RS based L3-RSRP measurement. Proposal 6: TCHO_execution (i.e., TLTM-RRC-processing) can be reused in conditional LTM. Proposal 7: TCHO_execution (i.e., TLTM-RRC-processing) is not be part of TCLTM_interrupt. Proposal 8: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused for R19 C-LTM. Proposal 9: TCLTM_interrupt includes TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU Proposal 10: C-LTM delay can be depicted as DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TCLTM_interrupt where TCLTM_interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU Proposal 11: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM. |
| R4-2504202 | Nokia | Proposal 1: RAN4 not to define CLTM measurement requirements for a deactivated SCell, when the condition to trigger cell switch is L1-based. Consequently, conditional cell switch to a deactivated SCell based on L1 measurements is not to be supported. Proposal 2: If RAN4 agrees to define requirements for UE based TA estimation in Rel-19 (Option 2), at least the issues listed below need to be taken into account.  Requirements to cover not only Rel-19 CLTM but ideally also Rel-18 LTM.  Which reference signals are expected to be used when performing UE-based TA measurements?  How long does the acquisition of the UE-based TA take after candidate cell configuration?  UE-based TA measurements shall not cause any impact to the network scheduling, for example, interruptions.  Existing transmit timing accuracy requirements shall apply.  How to ensure the accuracy of the UE-estimated TA? Proposal 3: If the issues in Proposal 2 cannot be solved within Rel-19, then Option 3 is preferred. Observation 1: Early TCI state activation may in most cases benefit interruption reduction for CLTM cell switch only if condition becomes met and cell switch is triggered within 160 ms/480 ms after TCI state activation is completed. Proposal 4: RAN4 to discuss how to ensure usefulness of early TCI state activation for CLTM cell switch interruption reduction in the definition of Tfirst-RS and TRS-proc. Proposal 5: Consider either removing the limitation of 160/480 ms for DL sync after TCI state activation or defining a UE capability to indicate UE is able to keep track of fine DL synchronization of an activated TCI state without limitations (until cell switch or TCI state deactivation), or at least for longer than 160 ms with L1 measurements, or for longer than 480 ms/160 ms with L3 measurements. Proposal 6: If UE received PDCCH order and consequently a TA MAC-CE, Tfirst-RS and TRS-proc are zero based on successful preamble transmission and while the TAT is running. Proposal 7: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. Proposal 8: The CLTM cell switch delay DCLTM is the delay from the end of the last TTI containing the RRC command for CLTM until the time the UE transmits the first UL message on the target cell: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt, Where: TCLTM_interrupt = TCLTM-processing + TCLTM-IU Proposal 9: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331. Proposal 10: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a CLTM command/configuration until the execution condition becomes met over the air. Proposal 11: When the execution condition is L3-based, Tmeasure in CLTM to follow the definition of Tmeasure in CHO. Proposal 12: When the execution condition is L1-based, Tmeasure in CLTM to be defined using the existing L1 measurement period for serving cell and candidate cells, with Treport = 0. Proposal 13: TCLTM_RRC-processing covers the delay related to the processing of the CLTM candidate cell configuration. There is no need to add an additional UE execution preparation time to the CLTM cell switch delay. Proposal 14: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous cell switch. Proposal 15: The subsequent CLTM cell switch delay DCLTM-subsequent is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell: DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt, Where: TCLTM_interrupt = TCLTM-processing + TCLTM-IU Proposal 16: Define initial and subsequent CLTM cell switch delay requirements in the same section under section 6.3. Observation 2: In Rel-18 L1 measurement requirements, after TCI state activation the UE is allowed to prioritize measurements on candidate cells with active TCI states and hence there are no requirements for L1 measurements on candidate cells without active TCI states. Therefore, it is unclear whether the UE continues L1-based condition evaluation (if configured) on the cells without active TCI states. Proposal 17: RAN4 to clarify whether the UE will continue L1-based condition evaluation for candidate cells without active TCI states after TCI state activation for at least one candidate cell. |
| R4-2503752 | Ericsson | The following have been proposed in the current contribution:  Proposal 1: Define requirements for L1 measurement on deactivated Scell.  Proposal 3: RAN4 do not define RRM requirements for the UE based TA measurement/estimation.  Proposal 4: RAN4 specify the 1st time CLTM and subsequent CLTM delay separately .  Proposal 5: The 1st time CLTM delay can be defined as DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interuption + + Tfirst-RS + TRS-proc where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU  Proposal 6: The subsequent CLTM delay can be defined as: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption + Tfirst-RS + TRS-proc, where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU |
| R4-2503856 | ZTE Corporation, Sanechips | Proposal 1：For the case which CLTM is triggered by L1 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included. Proposal 2: RAN4 should define RRM requirements for UE based TA measurement. Proposal 3: For conditional LTM delay, CHO requirements framework can be used as baseline. Proposal 4: Use DCLTM = [TRRC] + TEvent_DU + Tmeasure + TLTM-RRC-processing +TLTM-processing + Tfirst-RS + TRS-proc+ TLTM-IU as baseline and discuss whether TRRC is needed. Proposal 5: use TCLTM_interrupt = TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms as baseline and further discuss whether T/F fine tracking (TΔ) if needed can be executed at first and then TLTM-processing during conditional LTM. Proposal 6: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Similar as SCPAC, TRRC will not be considered in subsequent conditional LTM delay and TEvent_DU needs to be updated. |

Open issues summary

General aspects and early synchronization

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement

RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

- Candidate solutions:

  - Option 1: Define requirements for L1 measurement on deactivated Scell. (CMCC, E///, ZTE)

  - Proposal 1a: Only in R19 CLTM. (CATT)

  - Proposal 1b: (Huawei)

- Whether interruptions exist on activated serving cells

- Whether to extend L1 measurement period for deactivated SCC.

- Option 2: Define requirements for UE based TA measurement. (CTC, vivo, CMCC, ZTE)

  - Proposal 2a: Focus on how to define the feasible requirements. (CATT)

  - Proposal 2b: (QC)

  - Tx timing for the target cell = current Tx timing + 2 x (downlink frame boundary of reference cell – downlink frame boundary of the target cell), where the definition of ‘timing reference cell’ is the reference cell defined in the clause of ‘7.1 UE transmit timing’ in TS38.133. (QC)

  - FFS on the side conditions in terms of SNR, timeline for the downlink frame boundary estimation, etc.

  - Proposal 2c: The issues listed below need to be taken into account. (Nokia)

•	Requirements to cover not only Rel-19 CLTM but ideally also Rel-18 LTM.

•	Which reference signals are expected to be used when performing UE-based TA measurements?

•	How long does the acquisition of the UE-based TA take after candidate cell configuration?

•	UE-based TA measurements shall not cause any impact to the network scheduling, for example, interruptions.

•	Existing transmit timing accuracy requirements shall apply.

•	How to ensure the accuracy of the UE-estimated TA?

- Option 3: Not define requirement for any of them. (Apple, MTK, Nokia)

- Recommended WF

  - Discussion is needed.

  - Considering we only have three meetings left, if we could not achieve any agreement on option 1 and option 2. Maybe we could consider option 3.

Conditional LTM delay

Issue 1-2-1: General aspects of Conditional LTM delay

- Candidate solutions:

- Proposal 1: CHO requirements framework can be used as baseline for Conditional LTM. (Xiaomi, Huawei, ZTE)

- Recommended WF

  - CHO requirements framework can be used as baseline for Conditional LTM.

Issue 1-2-2: The starting point of conditional LTM delay

RAN4#114 < Agreement > ad hoc - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM, [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM]. - Revisit if RAN2 has more progress.

- Candidate solutions:

- Proposal 1: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. (Xiaomi, Huawei, Nokia)

- Recommended WF

  - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

Issue 1-2-3: The components of conditional LTM delay

- Candidate solutions:

- Proposal 1: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt. (vivo [TRRC], Apple)

- Proposal 2: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt . (CTC, Xiaomi, CMCC, Huawei)

- Proposal 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interuption +   + Tfirst-RS + TRS-proc. (E///)

- Proposal 4: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt. (Nokia)

- Proposal 5: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Twait_RS. (MTK)

- Proposal 6: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TLTM-RRC-processing +TLTM-processing + Tfirst-RS + TRS-proc+ TLTM-IU. (ZTE)

- Proposal 7: DCLTM = TRRC + TEvent_DU + Tmeasure+ TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU + TCLTM_execution. (CATT)

- Proposal 8: RAN4 to not rearrange the sequence of components in the C-LTM cell switch latency requirements compared to the legacy CHO requirements; specifically, ‘the last SSB sample (conceptually used for fine time/frequency tracking) should occur before the start of the interruption’

- Recommended WF

  - For those issues about conditional LTM delay, the controversial points are summarized as follows, we could discuss one by one.

    - Whether keep both TCLTM_execution  and TCLTM-RRC-processing, if not, keep which one.

    - Whether TCLTM_execution and/or TCLTM-RRC-processing should be included in Tinterrupt.

    - Whether Tfirst-RS and TRS-proc should be included in Tinterrupt.

    - Whether TEvent_DU  covers early TCI state activation and/or PDCCH order PRACH. If not, which component capture early TCI state activation and/or PDCCH order PRACH.

    - The details of Tmeasure, i.e., how to define the requirements on L1 based CLTM and L3 based CLTM.

    - The order of each components.

Issue 1-2-4: Tinterrupt

- Candidate solutions:

- Proposal 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. (CTC, Xiaomi, Huawei, ZTE)

- Proposal 2: TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. (vivo, CMCC, Apple)

- Proposal 3: TCLTM_interrupt = TRS + TRS-proc + TLTM-processing + TLTM-IU. (MTK, ZTE)

- Proposal 4: TCLTM_interrupt = TCLTM-processing + TCLTM-IU. (Nokia)

- Proposal 5: TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU. (E///)

- Recommended WF

  - Discussion is needed.

Issue 1-2-5: TRRC

- Candidate solutions:

- Proposal 1: TRRC is the RRC procedure delay. (CTC, Xiaomi, vivo, CATT, CMCC, MTK, Nokia)

- Recommended WF

  - TRRC is the RRC procedure delay.

Issue 1-2-6: TEvent_DU

- Candidate solutions:

- Proposal 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. (CTC, Xiaomi, vivo, Huawei, CMCC, Apple, MTK,)

    - Proposal 1a: It can cover both cases: (vivo, Huawei)

- UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation

- Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met.

- Proposal 2: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a CLTM command/configuration until the execution condition becomes met over the air. (Nokia)

- Recommended WF

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

    - It can cover both cases:

- UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation

- Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met.

Issue 1-2-7: Tmeasure

- Candidate solutions:

- Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. (CTC, vivo, CATT, CMCC, Apple)

    - Proposal 1a:

  - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM. (Huawei, Xiaomi, vivo, CATT)

  - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy. (Huawei, Xiaomi, vivo, CATT)

  - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19. (Huawei, vivo, CATT)

  - FFS on CSI-RS based L3-RSRP measurement. (Huawei, CATT)

    - Proposal 1b: (Nokia)

      - When the execution condition is L1-based, Tmeasure in CLTM to be defined using the existing L1 measurement period for serving cell and candidate cells, with Treport = 0.

      - When the execution condition is L3-based, Tmeasure in CLTM to follow the definition of Tmeasure in CHO.

- Proposal 2: Tmeasure is the time for the UE to measure and realize the condition for Conditional cell switch is fulfilled. (MTK)

    - For L3 event triggered conditional intra-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

    - For L3 event triggered conditional inter-frequency cell switch, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9.

    - For L1 event triggered conditional intra-frequency cell switch, the measurement time delay shall be less than Tidentify intra with index defined in clause 9.2.5.1 plus TL1-RSRP_Measurement_Period_SSB_intra defined in clause 9.14.5.1.

    - For L1 event triggered conditional inter-frequency cell switch, the measurement time delay shall be less than Tidentify_inter_with_index defined in clause 9.3.4 plus TL1-RSRP_Measurement_Period_SSB_inter defined in clause 9.15.

- Recommended WF

- Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

    - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

    - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy.

    - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

Issue 1-2-8: TCLTM_execution

- Candidate solutions:

- Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. (CTC, Xiaomi, CATT, CMCC, Huawei)

    - Proposal 1a: TCLTM_execution can be up to 10ms. (CMCC)

- Proposal 2: TLTM_RRC-processing covers the delay related to processing of the LTM candidate cell configuration. There is no need to add an additional execution/preparation delay to the cell switch delay. (Nokia)

    - Proposal 2a: Keep TCLTM_execution, remove TLTM_RRC-processing. (Huawei)

- Recommended WF

  - Discussion is needed.

Issue 1-2-9: TLTM-RRC-processing

- Candidate solutions:

- Proposal 1: TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. (CMCC)

    - Proposal 1a: TLTM-RRC-processing  can be up to 10ms. (MTK)

- Recommended WF

  - Discussion is needed.

Issue 1-2-10: TLTM-processing

- Candidate solutions:

- Proposal 1: TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. (CTC, Xiaomi, CMCC, Huawei)

    - Proposal 1a: TLTM-processing can be up to 20ms. (MTK)

- Recommended WF

  - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

Issue 1-2-11: Tfirst-RS / TRS

- Candidate solutions:

- Proposal 1: Tfirst-RS / TRS is the time for fine time tracking and acquiring full timing information of the target cell. (CTC, Xiaomi, CMCC, MTK, Huawei)

    - Proposal 1a: TRS can be up to 2ms. (MTK)

- Proposal 2: RAN4 to discuss how to ensure the usefulness of early TCI state activation for CLTM cell switch interruption reduction in the definition of Tfirst-RS and TRS-proc. (Nokia)

- Proposal 3: Consider either removing the limitation of 160/480ms for DL sync after TCI state activation or defining a UE capability to indicate UE is able to keep track of fine DL synchronization of an activated TCI state without limitations (until cell switch or TCI state deactivation), or at least for longer than 160ms with L1 measurements, or for longer than 480ms/160ms with L3 measurements. (Nokia)

- Proposal 4: If UE received PDCCH order and consequently a TA MAC-CE, Tfirst-RS and TRS-proc are zero based on successful preamble transmission and while the TAT is running. (Nokia)

- Recommended WF

  - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

Issue 1-2-12: TRS-proc

- Candidate solutions:

- Proposal 1: TRS-proc is the time for SSB processing. (CTC, Xiaomi, CMCC, MTK, Huawei)

    - Proposal 1a: TRS-proc can be up to 2ms. (MTK)

- Recommended WF

  - TRS-proc is the time for SSB processing.

Issue 1-2-13: TLTM-IU

- Candidate solutions:

- Proposal 1: TLTM-IU is the interruption uncertainty during LTM cell switch. (CTC, Xiaomi, CMCC, MTK, Huawei)

- Recommended WF

  - TLTM-IU is the interruption uncertainty during LTM cell switch.

Issue 1-2-14: Twait_RS

- Candidate solutions:

- Proposal 1: Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell. (MTK)

- Recommended WF

  - Discussion is needed.

Issue 1-2-15: Other

- Proposal 1: RAN4 to clarify whether the UE will continue L1-based condition evaluation for candidate cells without active TCI states after TCI state activation for at least one candidate cell. (Nokia)

- Recommended WF

  - Discussion is needed.

Subsequent Conditional LTM

Issue 1-3-1: General aspects of Subsequent Conditional LTM delay

- Candidate solutions:

- Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. (CTC, vivo, Apple, Huawei, ZTE)

    - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay. (CATT, ZTE)

    - Proposal 1b: TEvent_DU needs to be updated. (ZTE)

    - Proposal 1c: Apart from the starting point of the delay. (vivo, HW)

- Proposal 2: The subsequent CLTM cell switch delay DCLTM-subsequent is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell. (Nokia)

- Recommended WF

  - Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Issue 1-3-2: The starting point of Subsequent conditional LTM delay

- Candidate solutions:

- Proposal 1: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. (Nokia)

- Recommended WF

  - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Issue 1-3-3: The components of subsequent conditional LTM delay

- Candidate solutions:

- Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. (CMCC)

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. (Apple)

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- Proposal 2: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption + Tfirst-RS + TRS-proc, where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU. (E///)

- Proposal 3: DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt, where the TCLTM_interrupt = TCLTM-processing + TCLTM-IU. (Nokia)

- Recommended WF

  - Waiting for the conclusions of CLTM delay.

Issue 1-3-4: Other

- Proposal 1: Define initial and subsequent CLTM cell switch delay requirements in the same section under section 6.3. (Nokia)

- Recommended WF

  - Discussion is needed.

CR skeleton

| Expected impact | Notes | New or impacted section | Companies |
| --- | --- | --- | --- |
| Definitions and abbreviations |  | 3.1 3.3 |  |
| CLTM delay requirements | Depending on RAN2 conclusion | (new) 6.x or 6.3 | China Telecom |
| UL transmits timing requirements | TBD: UE based TA measurement | 7.1 7.3 |  |
| PDCCH-order RACH on neighbor cell | The related R18 RAN4 requirements could be as baseline | 6.2.2C |  |
| TCI state activation | The related R18 RAN4 requirements could be as baseline | 8.25 |  |
| L1-RSRP measurement | The related R18 RAN4 requirements could be as baseline | 9.14 9.15 |  |
| L3 measurement based CLTM | The CHO requirements could be as baseline |  |  |