# 3GPP TSG-RAN WG4 Meeting #114bis                            R4-2503642

# Wuhan, China, 7th-11th Apr, 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on A-MPR of 7MHz for band n26

# Agenda Item:   7.8.2

# Document for:  Approval

- Introduction

In last meeting, RAN4 agreed that NS_12 and NS_15 should be supported for 7MHz CBW, but NS_13 and NS_14 need further study[1]. In this paper, we provide some preliminary simulation results and propose our view on A-MPR of 7MHz CBW.

2-3-4: NS_12, NS_13, NS_14, NS_15 Agreement: - For n26 - NS_12 and NS_15 shall be supported for 7 MHz channel BW. - To be further discussed if NS_13 and NS_14 shall also be supported.

- Discussion

Existing A-MPR requirements of band n26 for NS_13 and NS_14 are shown in Figure 1. The additional UE emission requirements in NS_13 applies for NR carriers of 3MHz and 5MHz CBW with lower channel edge at or above 817MHz, and the additional UE emission requirements in NS_14 applies for NR carriers of 10MHz, 15MHz and 20MHz CBW with lower channel edge at or above 824MHz.

Figure 1  Existing A-MPR requirements of band n26 for NS_13 and NS_14

Whether 7MHz should be added to A-MPR requirements for NS_13 and NS_14 should be discussed[1].

## 2.1 NS_13

To evaluate the A-MPR for NS_13, first we calibrate the PA model to 1dB MPR point by using reference waveform with 20MHz CBW, 15kHz SCS, QPSK, 100RB0, and:

IQ image = 28dB

Carrier leakage = 28dBc

CIM = 60dBc

Then, we provide our preliminary simulation results in Annex. The results shows that NS_13 is needed to be considered for 7MHz CBW in band n26, since there is a large RB configuration region where A-MPR is needed to satisfy the additional emission requirements of NS_13. Based on our simulation results, legacy A-MPR values in 38.101-1 Table 6.2.3.22-2 can be reused, but the A-MPR regions for 7MHz CBW should be defined.

Proposal 1: NS_13 should be considered for 7MHz CBW in band n26.

Proposal 2: Reuse the A-MPR for NS_13 in 38.101-1 Table 6.2.3.22-2. Update A-MPR regions for 7MHz CBW in 38.101-1 Table 6.2.3.22-1 as follows.

Table 6.2.3.22-1  A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
| 7MHz | 820.5 ≤ Fc < 823.5 | ≤2.70 | <1.44 | A1 |
|  |  | ≤2.70 | ≥1.44 | A2 |
|  | Fc ≥ 823.5 | ≤1.08 | <1.44 | A1 |
|  |  |  | ≥3.60 | A3 |

Table 6.2.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

## 2.2 NS_14

Legacy A-MPR requirements for NS_14 is copied as follows.

Table 6.2.3.23-1: A-MPR regions for NS_14

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10MHz | ≤0.18 | <1.08 | A1 |
|  | ≥0 | ≥9 | A2 |
| 15MHz | ≤1.8 | <1.8 | A1 |
|  | ≥0 | ≥9 | A2 |
| 20MHz | ≤3.42 | <1.8 | A3 |
|  | ≥0 | ≥9 | A2 |

Table 6.2.3.23-2: A-MPR for NS_14

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3 |  | ≤ 3 |
| DFT-s-OFDM 256 QAM |  |  | ≤ 8 |
| CP-OFDM QPSK | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 6 |  | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 8 |  | ≤ 8 |

We also checked whether 7MHz CBW is needed for NS_14. The results shows that no A-MPR is need when the lower channel edge at or above 824MHz. Besides, we find that A-MPR regions are becoming smaller from 20MHz CBW to 10MHz CBW, and the A-MPR region of 10MHz CBW for NS_14 is very small, as shown in Figure 2. Therefore, we think there is no need to consider NS_14 for 7MHz CBW in band n26.

Figure 2  A-MPR regions for 10MHz, 15MHz and 20MHz CBW with 15kHz SCS

Observation 1: A-MPR regions of CBW from 20MHz to 10MHz are becoming smaller for NS_14, and the A-MPR region of 10MHz CBW for NS_14 is very small.

Proposal 3: Propose not to consider NS_14 for 7MHz CBW in band n26.

- Conclusion

Proposal 1: NS_13 should be considered for 7MHz CBW in band n26.

Proposal 2: Reuse the A-MPR for NS_13 in 38.101-1 Table 6.2.3.22-2. Update A-MPR regions for 7MHz CBW in 38.101-1 Table 6.2.3.22-1 as follows.

Table 6.2.3.22-1  A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
| 7MHz | 820.5 ≤ Fc < 823.5 | ≤2.70 | <1.44 | A1 |
|  |  | ≤2.70 | ≥1.44 | A2 |
|  | Fc ≥ 823.5 | ≤1.08 | <1.44 | A1 |
|  |  |  | ≥3.60 | A3 |

Table 6.2.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

Observation 1: A-MPR regions of CBW from 20MHz to 10MHz are becoming smaller for NS_14, and the A-MPR region of 10MHz CBW for NS_14 is very small.

Proposal 3: Propose not to consider NS_14 for 7MHz CBW in band n26.

- Reference

- R4-2502870 WF on requirements for 7MHz channel bandwidth, Ericsson, Athens, Greece, Feb. 2025.

- RP-250373 Revised WID on NR FR1 7 MHz Channel Bandwidth, T-Mobile USA, China Telecom, Incheon, KR, Mar. 2025.

Annex

- A-MPR for DFT-s-OFDM under different carrier frequency since 820.5MHz:

- A-MPR for CP-OFDM under different carrier frequency since 820.5MHz