3GPP TSG-RAN WG4 Meeting #114bis	R4-2503650

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	On OTA performance metric for handheld NTN UE

Source:	Samsung

Agenda item:	7.14.2.2

Document for:	Discussion

1.	Introduction

In RAN4#113 meeting, NTN OTA performance metric for handheld UE was agreed to be treated separately according to different scenarios [1]:

Issue 3-1-2: General view on Usage scenarios for handheld IoT-NTN and NR-NTN Agreements: - For IoT-NTN and NR-NTN, typical usage scenarios are: - Scenario 1: both talk mode (head+hand, and hand only) and browsing mode. - Scenario 2: browsing mode (hand only) - For IoT-NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode. - UEs supporting Scenario 1 are tested with head+hand and hand only modes - UEs supporting Scenario 2 only are only tested with hand only mode Issue 3-1-3: Handheld IoT-NTN and NR-NTN performance metric Agreements: - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 1 the performance metrics are full sphere TRP and TRS - For IoT-NTN and NR-NTN handheld UEs supporting Scenario 2 only, further study performance metric

In RAN4#114 meeting, it was further agreed that OEM declaration of scenario applicability can be used to declare the supported usage scenario.

Issue 3-1-1: UE manufacturer declaration of supported usage scenario for different NTN UE Agreements: - OEM declaration of scenario applicability can be used to declare the supported usage scenario. - Details on declaration and spec language can be further discussed. Issue 3-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: - RAN4 further discuss the performance metric for UE declared only support scenario 2. - Details on performance metric data processing should also be discussed.

However, the performance metric for UE declared only support Scenario 2 is not concluded yet.

In this contribution we share our views on performance metric for handheld NTN UE only supporting Scenario 2 by introducing more sub-scenarios in Scenario 2.

2. 	Discussion

NTN is novel technology and its usage scenario is still under evolution. The Scenario 1, which is both talk mode (head+hand, and hand only) and browsing mode, is clear already as it is similar as TN handheld UE, and the same full sphere TRP TRS performance metric is agreed which is same as TN’s handheld UE; the Scenario 2, which is browsing mode (hand only), however, is still not quite clear yet. Some commercially available smart phones supporting satellite communication with message only belong to this scenario. For this kind of UE, a partial sphere metric may be applicable, but we’d like to highlight that it is only a subset of Scenario 2. There could be more diverse device types under scenario 2, e.g., handheld UE supporting web browsing with NTN is also subset of Scenario 2. In web browsing mode, the user may hold the device in any possible position and thus a full sphere metric would be more suitable rather than partial sphere metric.

Based on above analysis we can see that there is big difference between the SMS usage and web browsing usage, though both belong to Scenario 2. The main difference between SMS usage and web browsing usage is that the former is non-real time usage where a special holding gesture is allowed for better link with satellite, while the latter is real-time usage where one is not expected to hold the device in a special holding gesture for a long time.

Observation 1:	Scenario 2 (browsing mode – hand only) has versatile sub-scenarios. Partial sphere metric is only suitable for non-real time usage e.g. SMS, and not suitable for real time usage e.g. web browsing where users can hold the device in any positions.

Therefore, it is not suitable to apply partial sphere metric for all Scenario 2 usage scenarios. For real time usage, full sphere TRP TRS is more appropriate. It would be reasonable to further split Scenario 2 into two sub-scenarios, i.e., real-time usage and non-real time usage. Partial sphere metric applies to non-real time usage only and full sphere TRP TRS metric apply to real time usage. In summary, the performance metric and testing scenario for NTN handheld UE can be as following table.

| Scenarios of NTN usage |  | performance metric | test scenario |
| --- | --- | --- | --- |
| Scenario 1 both talk mode (head+hand, and hand only) and browsing mode. | / | full sphere TRP TRS | head+hand phantom; hand phantom |
| Scenario 2 browsing mode (hand only) (for UE only supporting Scenario 2) | Scenario 2a non-real time usage of NTN, e.g. SMS | partial sphere metric (for UE only supporting scenario 2a) | hand phantom |
|  | Scenario 2b real-time usage of NTN, e.g. web browsing | full sphere TRP TRS | hand phantom |

Table 1. usage scenario and performance metric for handheld NTN UE

Proposal 1:	Further split Scenario 2 into two sub-scenarios, i.e., real-time usage and non-real time usage. Partial sphere metric applies to non-real time usage only (for UEs only supporting non-real time usage) and full sphere TRP TRS metric apply to real time usage, as summarized in Table 1.

A companion draft CR is provided in [3].

3. 	Conclusion

Observation 1:	Scenario 2 (browsing mode – hand only) has versatile sub-scenarios. Partial sphere metric is only suitable for non-real time usage e.g. SMS, and not suitable for real time usage e.g. web browsing where users can hold the device in any positions.

Proposal 1:	Further split Scenario 2 into two sub-scenarios, i.e., real-time usage and non-real time usage. Partial sphere metric applies to non-real time usage only (for UEs only supporting non-real time usage) and full sphere TRP TRS metric apply to real time usage, as summarized in Table 1.

| Scenarios of NTN usage |  | performance metric | test scenario |
| --- | --- | --- | --- |
| Scenario 1 both talk mode (head+hand, and hand only) and browsing mode. | / | full sphere TRP TRS | head+hand phantom; hand phantom |
| Scenario 2 browsing mode (hand only) (for UE only supporting Scenario 2) | Scenario 2a non-real time usage of NTN, e.g. SMS | partial sphere metric (for UE only supporting scenario 2a) | hand phantom |
|  | Scenario 2b real-time usage of NTN, e.g. web browsing | full sphere TRP TRS | hand phantom |

Table 1. usage scenario and performance metric for handheld NTN UE

A companion draft CR is provided in [3].

4. 	References

- R4-2419808    “WF for [113][325] TRP_TRS_Ph3”, vivo

- R4-2502370    “Way Forward for [114][327] TRP_TRS_Ph3”, vivo

- R5-2503651    “Draft CR to TR 38.870 on NTN usage scenario and performance metric”, Samsung