3GPP TSG-RAN WG4 Meeting #114bis	R4-2503656

Wuhan, Hubei, China, 07th – 11nd April, 2025

Agenda item:	7.1.3

Source:	Samsung

Title:	Discussion on UE demodulation performance requirements for 6Rx

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The WID for performance part has been updated as below [2] according latest RAN4#114 meeting agreements [3].

The objectives of performance part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas - Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

This is the first meeting for performance part. In this contribution, we provide our analysis on 6Rx PDSCH demodulation performance and CSI requirements, and 6Rx SDR requirements.

- Discussion

  - General

Since this is the first meeting for 6Rx performance part, the scope should be clarified firstly, such as duplex mode, number of Tx antennas, number of Rx antennas, SCS and Bandwidth, TDD UL-DL configuration, DRMS configuration.

Duplex mode

In the latest WID [2], the expected bands for supporting 6Rx as below

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104

According the defination in 38.101-1 as below, these expected bands are all FR1 TDD bands. Hence we prefer to define 6Rx requirements only for FR1 TDD.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n7712 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n7917 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n10417,18 | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD |

Proposal 1: Define 6Rx requirements only for FR1 TDD according the expected bands in the WID.

Number of Tx antennas and Rx antennas

Considering 4 MIMO layers performance requirments are needed to be defined for both FWA and handheld UE, 6 MIMO layers performance requirements are needed to be defined for FWA, 8Tx antennas are suitable and also typical in the real network.

Proposal 2: Set 8 Tx antennas and 6 Rx antennas for 6Rx requirements.

SCS and Bandwidth

For TDD case, we think use typical configuration SCS as 30kHz and Bandwidth as 40MHz is sufficient.

Proposal 3: Typical configuration SCS as 30kHz and Bandwidth as 40MHz is sufficient.

TDD UL-DL configuration

For the TDD UL-DL configuration, 7DS2U configuration defined in FR1.30-1 is a good selection.

Proposal 4: For the TDD UL-DL configuration, use 7DS2U configuration defined in FR1.30-1.

DMRS configuration

Per our understanding and the description of DMRS configurations described in 38.211 and 38.331, DMRS type 1 with one additional OFDM symbol for DMRS ports and maxLength=1 is the typical DMRS configuration (Rel-15 1+1 DMRS symbols) for 4 MIMO layers as Figure 2-1 (a), it is straightforward to configure maxLength=2 in order to support 6 MIMO layers as Figure 2-1 (b), this is the typical configuration for real network.  For Rel-18 1+1(6L), it only could be supported when the feature of Rel-18 enhanced DMRS is supported.

38.212, DMRS ports configurations

- Rel-15 typical 4 ports configuration

- Rel-15 typical 6 ports configuration

- Rel-18 6 ports configuration

Figure 2-2. DMRS ports configurations for Rel-15 DMRS and Rel-18 DMRS

Proposal 5: For DMRS configurations, use the typical configuration Rel-15 1+1(4L), Rel-15 2+2(6L) as below.

| PDSCH DMRS configuration | DMRS Type | Type 1 |
| --- | --- | --- |
|  | Number of additional DMRS | 1 |
|  | Antenna ports indexes | {1000-1003} for Rank 4 {1000-1003, 1004, 1006} for Rel-15 Rank 6 |
|  | Position of the first DMRS for PDSCH mapping type A | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data | 2 for Rank4 and Rank 6 tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 for rank 4 2 for rank 6 |

  - PDSCH demodulation performance requirements

Based on the configurations on the above General clause, more parameters need to be aligned for PDSCH demodulation performance requirements before simulation results comparison and alignments, for example, rank value, MCS level, correlation matrix, propagation channel.

Propagation channel

For the propagation channel, TDLA30-10 is usually used for Rank 4 and Rank 8 PDSCH demodulation performance requirements. Since no special impact factors for 6Rx need to be considered, use TDLA30-10 for 6Rx is enough.

Proposal 6: For PDSCH demodulation performance requirements of 6Rx, set propagation channel as TDLA30-10.

MCS Table and MCS level

For PDSCH demodulation performance requirements, MCS17 and MCS Table 1 are assumed for Rel-18 8 Rx which is introduced for FWA. Hence, we prefer to use MCS17 and MCS Table 1 as the starting point for 6Rx.

Proposal 7: For PDSCH demodulation performance requirements of 6Rx, select MCS17 and MCS Table 1 as the starting point.

Rank

According the latest WID [2], “6 MIMO layers performance requirements will be only defined for FWA” and “4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices.”, we prefer to define requirements for rank 4 and rank 6.

Proposal 8: For PDSCH demodulation performance requirements of 6Rx, define Rank 4 performance requirements for FWA and handheld UE devices, and define Rank 6 performance requirements only for FWA.

N1, N2 for codebook

For 8 CSI ports, two kinds of N1, N2 configurations are supported by RAN1 specification, we prefer to use the N1=4, N2=1 configuration as it’s more typical in real network.

Proposal 9: For PDSCH demodulation performance requirements of 6Rx, define (CodebookConfig-N1, CodebookConfig-N2) = (4,1).

Correlation matrix

Currently, both the PDSCH demodulation requirements for Rank 4 and Rank 8 are using ULA Low correlation, no matter the number of Rx antennas is 4 or 8. Hence we prefer to define the correlation matrix as ULA Low for PDSCH demodulation requirements for 6Rx.

Proposal 10: For PDSCH demodulation performance requirements of 6Rx, define the correlation matrix as ULA Low.

  - CSI reporting performance requirements

CQI reporting requirements

For the CQI estimation processing, there is no UE handling difference for 6Rx and 4Rx/8Rx, hence we don’t think new CQI reporting requirements are necessary for 6Rx. In current spec, CQI requirements with rank 4 under static channel are defined for 8Rx, from this point of view, it is also acceptable to define CQI requirements with rank 4 under static channel for 6Rx.

Proposal 11: For CQI reporting requirements, prefer not to define new CQI reporting requirements for 6Rx. It is also acceptable to only define CQI requirements with rank 4 under static channel for 6Rx.

PMI reporting requirements

For the PMI reporting requirements, test requirements has been standardized for 2Rx and 4Rx UE antennas, which could verify the PMI selection processing enough. For 8Rx UE antennas, there is no new test requirements introduced also. Considering no new codebook introduced for 6Rx, we think it’s not necessary to introduce PMI reporting requirements for 6Rx UE antennas.

Proposal 12: For PMI reporting requirements, prefer not to define new PMI reporting requirements for 6Rx.

RI reporting requirements

Similar reasons as above for CQI and PMI reporting requirement, we don’t prefer to define new test requirements for RI reporting for 6Rx.

Proposal 13: For RI reporting requirements, prefer not to define new RI reporting requirements for 6Rx.

  - SDR requirements

MIMO layers and Modulation

For MIMO layer as 6, new requirements of SDR for up to maximum 256QAM need to be defined. For 1024QAM, current spec only define SDR requirements up to MIMO layer 2 even for 8Rx, hence new SDR requirements for 1024QAM with MIMO layer 2 should be defined, and no need to do the feasibility of 1024QAM with 4 MIMO layers.

Proposal 14: For the SDR requirements of 6Rx, define new SDR requirements for MIMO layer 6 with up to 256QAM. For 1024QAM, define new SDR requirements for MIMO layer 2, and no need to do the feasibility of 1024QAM with 4 MIMO layers.

Furthermore, in current spec, for MIMO layer 4 and 8, only the requirements with scaling factor 1 for 256QAM and 64QAM changed, hence we need to do maximum MCS feasibility for MIMO layer 6 with 256QAM and 64QAM with scaling factor 1.

Proposal 15: Define new SDR requirements of 6Rx with MIMO layer 6 as below table, and do maximum MCS feasibility for 256QAM and 64QAM with scaling factor 1.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | TBD |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | TBD |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Proposal 16: For 1024QAM, define SDR requirements for MIMO layer 2 as below, and study if MCS24 is achieveable for scaling factor 1.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | TBD |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |

- Conclusion

In this contribution, we provide our analysis and views on the PDSCH demodulation performance, CSI reporting performance requirements and SDR requirements on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

For General

Proposal 1: Define 6Rx requirements only for FR1 TDD according the expected bands in the WID.

Proposal 2: Set 8 Tx antennas and 6 Rx antennas for 6Rx requirements.

Proposal 3: Typical configuration SCS as 30kHz and Bandwidth as 40MHz is sufficient.

Proposal 4: For the TDD UL-DL configuration, use 7DS2U configuration defined in FR1.30-1.

Proposal 5: For DMRS configurations, use the typical configuration Rel-15 1+1(4L), Rel-15 2+2(6L) as below.

| PDSCH DMRS configuration | DMRS Type | Type 1 |
| --- | --- | --- |
|  | Number of additional DMRS | 1 |
|  | Antenna ports indexes | {1000-1003} for Rank 4 {1000-1003, 1004, 1006} for Rel-15 Rank 6 |
|  | Position of the first DMRS for PDSCH mapping type A | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data | 2 for Rank4 and Rank 6 tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 for rank 4 2 for rank 6 |

For PDSCH demodulation performance requirements

Proposal 6: For PDSCH demodulation performance requirements of 6Rx, set propagation channel as TDLA30-10.

Proposal 7: For PDSCH demodulation performance requirements of 6Rx, select MCS17 and MCS Table 1 as the starting point.

Proposal 8: For PDSCH demodulation performance requirements of 6Rx, define Rank 4 performance requirements for FWA and handheld UE devices, and define Rank 6 performance requirements only for FWA.

Proposal 9: For PDSCH demodulation performance requirements of 6Rx, define (CodebookConfig-N1, CodebookConfig-N2) = (4,1).

Proposal 10: For PDSCH demodulation performance requirements of 6Rx, define the correlation matrix as ULA Low.

For CSI reporting requirements

Proposal 11: For CQI reporting requirements, prefer not to define new CQI reporting requirements for 6Rx. It is also acceptable to only define CQI requirements with rank 4 under static channel for 6Rx.

Proposal 12: For PMI reporting requirements, prefer not to define new PMI reporting requirements for 6Rx.

Proposal 13: For RI reporting requirements, prefer not to define new RI reporting requirements for 6Rx.

For SDR requirements

Proposal 14: For the SDR requirements of 6Rx, define new SDR requirements for MIMO layer 6 with up to 256QAM. For 1024QAM, define new SDR requirements for MIMO layer 2, and no need to do the feasibility of 1024QAM with 4 MIMO layers.

Proposal 15: Define new SDR requirements of 6Rx with MIMO layer 6 as below table, and do maximum MCS feasibility for 256QAM and 64QAM with scaling factor 1.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | TBD |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | TBD |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Proposal 16: For 1024QAM, define SDR requirements for MIMO layer 2 as below, and study if MCS24 is achieveable for scaling factor 1.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | TBD |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#107 meeting, Huawei

[3] R4-2503014, WF on 6Rx UE requirements, RAN4#114 meeting, AT&T