3GPP TSG-RAN WG4 Meeting #114bis	R4-2503657

Wuhan, Hubei, China, 07th – 11nd April, 2025

Agenda item:	7.3.4

Source:	Samsung

Title:	Discussion on UE demodulation performance requirements for less than 5MHz FR1 Phase 2

Document for:	Discussion

- Introduction

In the RAN#107 meeting, the latest WID [1] for “NR channel BW less than 5MHz for FR1 Phase 2” has been revised as below. This is the first meeting for demodulation performance part, and the performance part is targeting to be finished at June/2025 [2], only two meetings scheduled for demodulation performance part.

- Objective of SI or Core part WI or Testing part WI - Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz or 5MHz in band n100 and 5MHz or 10MHz in band n101 NOTE: Consideration of 5MHz channel bandwidth in the above “3MHz or 5MHz” is specific to band n100 and does not apply to other bands. - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified. - Objective of Performance part WI - Specify necessary performance requirements (RAN4)

In this contribution, we provide our analysis and views on demodulation performance impacts for less than 5MHz phase 2.

- Discussion

  - General

Since this is the first meeting for less than 5MHz phase 2 performance part, the scope should be clarified firstly, such as duplex mode, physical channels, number of Tx antennas, number of Rx antennas, SCS.

Duplex mode and Physical channels

For less than 5MHz phase 1 in Rel-18, no PDSCH demodulation test requirements defined for less than 5MHz since almost no performence impact on PDSCH demodulation, and only 3MHz PDCCH demodulation test requirements are introduced for FR1 FDD since 3MHz is only supported by some FR1 FDD bands listed as below Table 2-1.

Table 2-1 3MHz supported Bands defined in 38.101-1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n26 | 814 MHz – 849 MHz | 859 MHz – 894 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n31 | 452.5 MHz – 457.5 MHz | 462.5 MHz – 467.5 MHz | FDD |
| n72 | 451 MHz – 456 MHz | 461 MHz – 466 MHz | FDD |
| n85 | 698 MHz – 716 MHz | 728 MHz – 746 MHz | FDD |
| n10021 | 874.4 MHz – 880 MHz | 919.4 MHz – 925 MHz | FDD |
| n106 | 896 MHz – 901 MHz | 935 MHz – 940 MHz | FDD |

For less than 5MHz phase 2 in Rel-19, co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW are introduced, corresponding demodulation requirements should be defined in 38.101-4. As 3MHz could only be supported on FR1 FDD bands, related PDSCH demodulation performance requirements should focus on FR1 FDD.

Proposal 1: For less than 5MHz phase 2, only PDSCH CA demodulation requirements should be introduced, neither PDCCH nor PBCH demodulation requirements are impacted.

Proposal 2: Define PDSCH CA demodulation requirements with 3MHz only for FR1 FDD since 3MHz supported bands defined in RF spec.

Number of Rx antennas

Currently, the number of Rx antennas for PDSCH CA demodulation requirements are 2, 4 and 8, however, 8 Rx antennas are specified for some bands which are not duplicated with 3MHz supported bands [4]. Hence it is reasonable to support 2Rx and 4Rx for 3MHz PDSCH CA demodulation requirements.

Proposal 3: Define PDSCH CA demodulation requirements with 3MHz for 2Rx and 4Rx antennas, not for 8Rx antennas since 8Rx antennas are specified for some bands which are not duplicated with 3MHz supported bands.

Number of Tx antennas

For 2Rx and 4Rx PDSCH CA demodulation requirements, only 2Tx antennas are defined in current spec. Therefore, the number of Tx antennas set as 2 is enough.

Proposal 4: Define PDSCH CA demodulation requirements with 3MHz for 2Tx antennas.

HST-SFN of HST-DPS

The PDSCH CA demodulation test requirements for different bandwidths are also defined for HST-SFN and HST-DPS in current spec, hence we need to clarify if 3MHz PDSCH CA demodulation requirements should be defined for HST-SFN and HST-DPS scenarios.

Proposal 5: Clarify if 3MHz PDSCH CA demodulation requirements should be defined for HST-SFN and HST-DPS scenarios.

  - PDSCH CA demodulation performance requirements

Based on the configurations on the above General clause, more parameters need to be aligned for 3MHz PDSCH CA demodulation performance requirements, for example, rank value, MCS level, correlation matrix, propagation channel. However, considering only two meetings scheduled for demodulation performance part, it seems we don’t have enough time to do simulations 3MHz PDSCH CA demodulation performance requirements. Therefore, we prefer to use the same simulation assumptions as 5MHz for 3MHz PDSCH CA demodulation requirements, and reuse the test requirements as 5MHz for 3MHz PDSCH CA demodulation requirements.

Proposal 6: Prefer to reuse the same simulation assumptions and test requirements as 5MHz for 3MHz PDSCH CA demodulation requirements.

- Conclusion

In this contribution, we provide our analysis and views on the NR channel BW less than 5MHz for FR1 Phase 2 topic.

The observations and proposals could be summarized as:

Proposal 1: For less than 5MHz phase 2, only PDSCH CA demodulation requirements should be introduced, neither PDCCH nor PBCH demodulation requirements are impacted.

Proposal 2: Define PDSCH CA demodulation requirements with 3MHz only for FR1 FDD since 3MHz supported bands defined in RF spec.

Proposal 3: Define PDSCH CA demodulation requirements with 3MHz for 2Rx and 4Rx antennas, not for 8Rx antennas since 8Rx antennas are specified for some bands which are not duplicated with 3MHz supported bands.

Proposal 4: Define PDSCH CA demodulation requirements with 3MHz for 2Tx antennas.

Proposal 5: Clarify if 3MHz PDSCH CA demodulation requirements should be defined for HST-SFN and HST-DPS scenarios.

Proposal 6: Prefer to reuse the same simulation assumptions and test requirements as 5MHz for 3MHz PDSCH CA demodulation requirements.

- Reference

[1] RP-250463, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, RAN#107 meeting, Intel Corporation

[2] RP-250462, Status Report for NR channel BW less than 5MHz for FR1 Phase 2, RAN#107 meeting, Intel Corporation

[3] RP-250626, RAN4 CRs to Rel-19 NR channel BW less than 5MHz for FR1 Phase 2, RAN#107 meeting

[4] RP-221496, Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1), RAN Meeting #96, Huawei, HiSilicon