3GPP TSG-RAN WG4 Meeting #114bis	R4-2503661

Wuhan, Hubei, China, 07th – 11nd April, 2025

Agenda item:	7.18.2

Source:	Samsung

Title:	Discussion on UE demodulation and CSI performance requirements for 8Rx with MMSE-IRC

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WID [1] for “NR demodulation performance: Phase 5” has been approved. According the plan, this WI is targeting to be finished in RAN#109 meeting on Sep 2025.

Generally, the enhancements for UE demodulation performance focus on two scenarios, the first one is the inter-cell interference with MMSE-IRC receiver scenario, the second one is the intra-cell inter-user interference with MMSE-IRC receiver scenario.

RAN4#112 meeting is the first meeting for this topic, several high-level agreements are achieved [2]. Cover CQI reporting requirements under ICI for 8Rx is agreed and most parameters for simulation assumptions are agreed in RAN4#112bis [3] and RAN4#113 [4], more agreements about scenario coverage and test parameters are achieved in [5].

In this contribution, we provide our further analysis and views on the open issues for PDSCH demodulation performance parts and simulation assumptions for CQI part.

- Discussion

  - General

Definition of MMSE-IRC receiver capability for 8Rx UE

In last RAN4#114 meeting [5], the definition of MMES-IRC receiver capability for 8Rx UE has been discussed but conclusion was not achieved. Several options are proposed as below for MMSE-IRC receiver capability definition.

Issue 1-1-1: Definition of MMSE-IRC receiver capability for 8Rx UE Agreement: - If agreed to change the receiver definition, it also needs to be changed in Rel-18 maintenance. - If receiver definition is agreed to be changed, an LS will be sent to RAN2 for updating TS38.306. Proposals: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability

As we know, for Rel-18 8Rx, two separate UE capabilities based on UE declaration are defined in TS38.101-4 and TS38.306

Section 3.1 in TS38.101-4

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Section 5.2 in TS38.306

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

For MMSE-IRC receiver, considering there is no processing difference for UE implementation when handling SU-MIMO and MU-MIMO, and the 8Rx feature is introduced for non-handheld UE in Rel-18, we think update current definition in TS38.101-4 and TS38.306 to remove the “SU-MIMO” wording is a quite good solution. Furthermore, for some other receiver, e.g., the enhanced R-ML receiver, the UE processing is different for SU-MIMO and MU-MIMO, this is reason why we prefer option 2 to restrict the definition for baseline/simplified 8Rx receiver with “MMSE-IRC” description. However, we could also support option 1, and if option 1 is finally agreed, we’d better have solution for the definition of baseline/simplified 8Rx receiver with R-ML receiver.

Observation 1: Current definition of baseline/simplified 8Rx receiver is for MMSE-IRC receiver. There is no processing difference for UE implementation when handling SU-MIMO and MU-MIMO.

Observation 2: For R-ML receiver, the UE processing is different for SU-MIMO and MU-MIMO.

Proposal 1: For the capability defined in TS 38.101-4, update current definition as option 2, but could also accept option 1.

8Rx UE MMSE-IRC receiver coverage

In previous meetings, the criteria for decision if one or two sets PDSCH requirements to be introduced is agreed, but the value of threshold need further discussion.

Issue 1-1-2: 8Rx UE MMSE-IRC receiver coverage Agreement: - Criteria for decision if one or two sets of PDSCH requirements are to be introduced: - Secure the span is <=2.5 dB for each receiver type by removal of outliers - If difference in the average of each receiver type for a specific configuration is > [TBD:limit] then define two sets of requirements for the specific configuration. - Further discuss the actual value for the [limit]. - Proposals: - Option 1: Set [limit] to 0.5dB - Option 2: Set [limit] to 1.0dB - Option 3: Set [limit] to 1.2dB - Other options not precluded.

As current definition of baseline/simplified 8Rx receiver is based on UE declaration, the corresponding requirements for each receiver should have obvious difference, otherwise the so-called declaration on different receiver is meaningless. Therefore, we prefer to set the threshold as at least 1dB.

Proposal 2: For the criteria for decision if one or two sets of PDSCH requirements are to be introduced, we prefer to set the threshold as at least 1dB, i.e., support option 3 as first priority and option 2 as second priority.

  - PDSCH requirements with intra-cell inter-user interference

Rank 4+4

For the rank value of MU-MIMO with 8Rx, if the requirements of rank 4+4 should be defined is still open.

Issue 1-3-2: Rank 4+4 Proposals: - Option 1: Do not define Rank 4+4 requirements. - Option 2: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

Per our understanding, the rank 4+4 paring is mostly for lab testing scenario, while it is rare at the real network. Hence we prefer not to define rank 4+4 requirements.

Proposal 3: For PDSCH requirements with intra-cell inter-user interference, prefer not to define Rank 4+4 requirements.

  - Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm

Issue 1-4-1: CQI calculation algorithm Proposals: - Option 1: RAN4 to define CQI requirements for two receiver types: - Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing. - Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing. - Option 2: If the difference in gamma between baseline and simplified receiver is 0.1 or lower only one set of requirements shall be defined. Further discuss when more simulation results are available.

Due to similar reason as the issue for one or two sets of PDSCH requirements, we think two sets requirements should be based on obvious performance difference between baseline receiver and simplified receiver.

Proposal 4: For the requirements for CQI reporting, separate requirements for two receiver types should be defined once the gamma value has obvious difference, i.e., the gamma value difference is more than [0.1].

  - Applicability rule and release independent

Test applicability to skip 2/4Rx requirements

There is no conclusion on the issue for test applicability to skip 2/4Rx requirements. The way forward as below.

Issue 1-5-1: Test applicability to skip 2/4Rx requirements Proposals: - Option 1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. TBD: Clarification of what “corresponding 8Rx requirements” covers.

In order to balance the test coverage and test workload, for both demodulation performance tests and CQI reporting tests, we prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 5: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Release independency

Since 8Rx is introduced in Rel-18, we think it is reasonable to define the new requirements for 8Rx should be release independent from Rel-18.

Proposal 6: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Conclusion

In this contribution, we provide analysis and views on NR demodulation performance: Phase 5.

The observations and proposals could be summarized as:

Observation 1: Current definition of baseline/simplified 8Rx receiver is for MMSE-IRC receiver. There is no processing difference for UE implementation when handling SU-MIMO and MU-MIMO.

Observation 2: For R-ML receiver, the UE processing is different for SU-MIMO and MU-MIMO.

Proposal 1: For the capability defined in TS 38.101-4, update current definition as option 2, but could also accept option 1.

Proposal 2: For the criteria for decision if one or two sets of PDSCH requirements are to be introduced, we prefer to set the threshold as at least 1dB, i.e., support option 3 as first priority and option 2 as second priority.

Proposal 3: For PDSCH requirements with intra-cell inter-user interference, prefer not to define Rank 4+4 requirements.

Proposal 4: For the requirements for CQI reporting, separate requirements for two receiver types should be defined once the gamma value has obvious difference, i.e., the gamma value difference is more than [0.1].

Proposal 5: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 6: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Reference

[1] RP-241297, Revised WID on NR demodulation performance: Phase 5, RAN#104 meeting, China Telecom, NTT DOCOMO

[2] R4-2413570, WF on NR demodulation performance: Phase 5, RAN4#112 meeting, China Telcom

[3] R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE, RAN4#112bis meeting, Nokia

[4] R4-2419784, Way forward for [113][322] NR_demod_Ph5_Part2_UE, RAN4#113 meeting, Nokia

[5] R4-2502275, Way forward for [114][324] NR_demod_Ph5_Part2_UE, RAN4#114 meeting, Nokia