3GPP TSG-RAN WG4 Meeting #114bis		R4-2503670

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:		7.29.4.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

In RAN4#114 meeting, we have already discussed the RRM requirements of DL coverage enhancements. There are still some open issues for the requirements if SSB periodicity larger than 20ms in latest WF [1].

In this contribution, we continue to discuss the open issues of RRM requirements for DL coverage enhancement.

- Discussion

In last RAN4#114 meeting, it is agreed the default TSMTC,i should be updated as:

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i) Agreement (online): - In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided. Further discussion: - In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined. - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency

The exact value is still FFS and the applicability is FFS.

For Re-establishment, the UE should perform similar behaviour as initial access process in idle mode. In legacy initial access, RAN1 specified it as 20ms.

In last RAN1#118bis meeting, the agreement for SSB periodicity extension has been approved as 160ms and the assumption during initial access should be update to 160ms. In last meeting, it is agreed that the default value is needed to update for the case where the SMTC for inter-frequency carrier is not provided. By our understanding, the exact value should be aligned with 160ms as the change of initial access update.

Whether this update will apply depends on special UE capability or not, we don’t think there is necessary to specify a dedicated UE capability for Re-establishment. It should be aligned with the same principle as initial access.

Observation 1: For initial access UE behavior, RAN1 is still discussing whether the update to 160ms depends on UE capability or not. There is no conclusion until now.

Proposal 1: For Re-establishment delay requirement, update the default value of: TSMTC,I to 160ms. For the applicability of the update, whether to depend on UE capability or not, it should be aligned with the same as initial access. If there is no new UE capability for initial access finally, this update for RRC re-establishment should be applied to all UEs. Otherwise, it uses the same UE capability in initial access update to 160ms but not a dedicated one. It is only for inter-frequency.

## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Further discussion: - In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided: - Option 1: Yes, an update is needed - Note: With the understanding that the by-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective. - Companies are encouraged to provide details on how to update the specification. - Other option is not precluded.

This issue focus on whether to update the yellow part in current spec:

Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); K_satellite * MAX (800 ms, 13 x TSMTC, i). < -8; FR1; N/A; k * 800 ms Note1, 3. NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: K_satellite is defined in clause 9.3C.4. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

In this note, it is defined by TSMTC, i > 20ms which 20ms are the similar as legacy initial access. When the initial access behavior has been updated to 160ms in Rel-19, we think this should be updated to align with 160ms.

Proposal 2: Update the note for time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell above.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for DL coverage enhancement:

Observation 1: For initial access UE behavior, RAN1 is still discussing whether the update to 160ms depends on UE capability or not. There is no conclusion until now.

Proposal 1: For Re-establishment delay requirement, update the default value of: TSMTC,I to 160ms. For the applicability of the update, whether to depend on UE capability or not, it should be aligned with the same as initial access. If there is no new UE capability for initial access finally, this update for RRC re-establishment should be applied to all UEs. Otherwise, it uses the same UE capability in initial access update to 160ms but not a dedicated one. It is only for inter-frequency.

Proposal 2: Update the note for time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell above.

- Reference

[1] R4-2502605, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm