# 3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503693

Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the simultaneous operation between GNSS and NR NTN

Agenda Item:	7.29.2.1

Document for:	Approval

Introduction

In this meeting, we received the replied LS [1] from RAN2.

RAN2 would like to thank RAN4 for the original LS in R4-2419898. RAN2 has considered the simultaneous operation between GNSS and UL transmission in NR NTN and concluded not to work on the enhancements to the existing IDC solutions in Release 18. RAN2 might consider the extension to IDC TDM solution or autonomous denial solution in Release 19, and would like to receive more information on requirements, e.g. on the periodicity and duration of IDC free time to help the UE read the GNSS in NTN.

In this paper, we’d like to share our views on these values.

Discussion

For GNSS, the time to first fix (TTFF) is different for different start condition. For hot start, the TTFF can be 1 or 2 second(s). But for cold start, the TTFF requirements may be longer than half minute. In my understanding, we don’t need to consider the cold start scenario when RAN4 evaluate the periodicity and duration of IDC free time.

Observation 1: cold start scenario is not considered when RAN4 evaluate the periodicity and duration of IDC free time.

The similar issue was discussed under Rel-17 LTE_NBIOT_eMTC_NTN WI. And the time duration required for the IoT terminals to acquire a GNSS position was specified below.

-- ASN1START GNSS-PositionFixDuration-r18 ::= ENUMERATED{ s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31} -- ASN1STOP

RAN4 can discuss whether these values are applicable to NR NTN UE since such large value may have big impacts on the NR NTN UL transmission.

Proposal 1: RAN4 can discuss whether the existing GNSS-PositionFixDuration specified in TS 36.331 can be used for NR NTN UE or not.

Summary

Observation 1: cold start scenario is not considered when RAN4 evaluate the periodicity and duration of IDC free time.

Proposal 1: RAN4 can discuss whether the existing GNSS-PositionFixDuration specified in TS 36.331 can be used for NR NTN UE or not.

- References

[1] R2-2501568, Response LS on simultaneous operation between GNSS and NR NTN, RAN2