# 3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503695

# Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with UL MIMO

Agenda Item:	7.12.1.2

Document for:	Approval

Introduction

In the RAN4#114 meeting, there are two remaining issues for ATG UE supporting UL MIMO feature based on the approved WF [1].

Issue 3-1-1: ULFPTx mode for UL-MIMO WF: Consider whether and how to introduce this requirement. - Option 1: ULFPTx mode 1/2/Full in the following table can be supported depending on UE PA implementation - Table 2: Example of ATG ULFPTx (R-MOP = x dBm) ATG ULFPTx mode; PA1(dBm); PA2(dBm). Mode2; x; (x-3)~(x-1). Mode-Full; x; x. Mode1; (x-3)~(x-1); (x-3)~(x-1). - Option 2: Current ULFPTx mode requirements for legacy UE in TS38.101-1 can be reused for ATG UE, and there is no other specification affect. - Option 3: No need to extend ULFPTx mode to ATG UE. Issue 3-1-2: whether preclude the single antenna port related requirement WF: - Option 1: Yes. - Option 2: No.

There is no progress in the RAN4#113 meeting.

In this meeting, we’d like to share our views on the remaining open issues.

Discussion

Since ATG UE can report the rated maximum output power, it’s totally different from handheld UE (power class report). Currently, the supported range of MOP for ATG UE is about 23~40dBm. Thus, the typical scenarios for handheld ULFPTx mode are 2*23dBm PA supporting 23dBm full power Tx per layer with 23dBm power class 3 report and 2*26dBm PA supporting 26dBm full power Tx per layer with 26dBm power class 2 report. However, for these scenarios, ATG UE can report higher MOP without full power mode as shown below.

| PA implementation | Handheld ULFPTx mode | ATG UE MOP report |
| --- | --- | --- |
| 2*23dBm PA | 23dBm full power Tx per layer with 23dBm power class 3 | 26dBm without ULFPTx mode |
| 2*26dBm PA | 26dBm full power Tx per layer with 26dBm power class 2 report | 29dBm without ULFPTx mode |

Since Handheld ULFPTx mode only discussed the limited kinds of RF architectures, it can’t cover all the implementations of ATG UE.

If we checked the proposed option 1, only two PAs are assumed for ATG UE implementation. However, generally one PA implementation can be assumed for passive antenna implementation. For the phased array antenna, there are a lot of same PA for ATG UE implementation. We don’t think two PA implementation for ATG UE is typical as handheld UE.

Observation 1: the two PAs implementation in the proposed option 1 is not typical for ATG UE.

For potential ATG UE supporting ULFPTx, It’s not expected to introduce additional spec impacts. Thus, as a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not. It’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation.

Proposal 1: As a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not in the way forward. However, it’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation.

For the issue whether to preclude the single antenna port related requirement, we think Rel-18 ATG RF requirements have included the single antenna port related requirement.

Referring to the contribution [2], it seems that companies think single antenna port related feature is equal to the TxD requirements in the spec. However, in another contribution [3], companies point out that single antenna port (W=1) includes the requirements for indicating txDiversity and not indicating txDiversity.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity, - the requirements in clause 6.2J apply for at least one antenna connector for the ATG UE not indicating the capability antennaArrayType-r18. - the requirements in clause 6.2J apply for at least one TAB connector for the ATG UE indicating the capability antennaArrayType-r18. For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity, - the rated maximum output power applies for sum of two antenna connectors for the ATG UE not indicating the capability antennaArrayType-r18. - the rated maximum output power applies for sum of two TAB connectors for the ATG UE indicating the capability antennaArrayType-r18.

Proposal 2: RAN4 can discuss whether the single antenna port related requirements for ATG UE without indicating TxD have been included in the spec. And further discuss whether to explicitly specify the single antenna port related requirements for ATG UE with indicating TxD.

Summary

Observation 1: the two PAs implementation in the proposed option 1 is not typical for ATG UE.

Proposal 1: As a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not in the way forward. However, it’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation.

Proposal 2: RAN4 can discuss whether the single antenna port related requirements for ATG UE without indicating TxD have been included in the spec. And further discuss whether to explicitly specify the single antenna port related requirements for ATG UE with indicating TxD.

- References

[1] R4-2502952	WF on UE RF for ATG enhancement, CMCC

[2] R4-2501872, Discussion on RF requirement for UL-MIMO for ATG UE in Rel-19, ZTE Corporation, Sanechips

[3] R4-2501902, ATG UE RF requirements for UL-MIMO, LG Electronics