3GPP TSG-RAN4 #114-bis	R4-2503710

Wuhan, China, 7th Apr. – 11th Apr. 2025

Title:	Views on TE emulated channel model for satellite mobility

Source:			Anritsu Corporation

Agenda Item:			7.10.4.1

Document for:			Approval

# 1.	Introduction

At the RAN4 #114 meeting in Athens, we continued a discussion on a methodology to derive time varying doppler shift and propagation delay based on the two-body Keplerian propagator model. And some open issues were captured in the way forward [1].

In this contribution we share our views on the following issue 1-1-1 and issue 1-1-2, extracted for reference.

Extract from [1] Issue 1-1-1 Satellite position/velocity estimation based on equation of motion - WF - RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay based on the Kepler’s equation and equation of motion. It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test. Specify both the satellite position/velocity calculation methods in TS 38.101-5/TS 36.102 - Note: the details models for Kepler’s equation and equation of motion can be referred in Appendix - RAN4 will introduce the accuracy requirement applied for two models compared with GMAT - Companies are encouraged to provide the calibration and comparison results with two models before RAN4#114bis meeting, based on calibration and comparison results, further discuss how to define the related accuracy requirement in the next meeting. - FFS how to capture the TE-emulated time varying doppler shift and propagation delay model in the specification - Option 1: capture into the normative part of Appendix, details need to be further discussed

Extract from [1] Issue 1-1-2 Correction of Propagation Delay Calculation - WF - RAN4 discuss whether the Earth's rotation is considered during signal flight time for more accurate estimations of propagation delay - Considering the earth rotation, the propagation delay, DelayERtR can be derived as DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR> Where tR is the signal’s received time, rtRECEFand vtRECEF are the satellite position and velocity vectors at time tR , ρtRis the difference in satellite and UE position vectors at time tR, K is the unit vector in standard z-axis, (a×b) is the cross-product and <a, b> is the dot product between a and b - The proponent is encouraged to provide the detail information, e.g., reference, if possible, about the propagation delay calculation with considering the signal flight time before RAN4#114bis meeting - Participating companies are encouraged to examine this issue from their perspective.

# 2.	Discussion

  - Views on Issue 1-1-1 Satellite position/velocity estimation based on equation of motion

One of the items captured on Issue 1-1-1 in the WF [1] is extracted as follows.

- RAN4 will introduce the accuracy requirement applied for two models compared with GMAT

Suppose we use GMAT as a basis, we noticed that we need to agree on multiple parameters and configuration files to use commonly for conducting a comparison of the satellite orbit between GMAT and two-body Keplerian model. Otherwise, we won’t be able to compare the obtained results among companies. Following Figure 2.1-1 depicts the actual setup window of GMAT. As can be seen in the setup window, there are several parameters such as Minimum step size, Primary body, Gravity model, etc. Also related to this issue, there is a parameter file which associates with the selected gravity model, also shown an example by Figure 2.1-2. Since we previously agreed in the group that we apply the two-body Keplerian model to emulate the satellite motion, we need to prepare the associated configuration file to compare the calculation results of orbits between GMAT and Keplerian.

Observation 1: There is a need to agree on multiple parameters and configuration files to use commonly for conducting a comparison of the satellite orbit between GMAT and two-body Keplerian along with the comparison among companies.

Observation 2: Associated parameter file for the selected gravity model is also necessary.

Figure 2.1-1: Setup window of GMAT

Figure 2.1-2: Example of gravity model configuration file

  - Views on Issue 1-1-2 Correction of Propagation Delay Calculation

2.2.1 Equation to derive total propagation delay

Next, we would like to show our views on the proposed correction factor to the propagation delay calculation which considers a rotation of the Earth during the time of signal transmission. The equation to derive the total propagation delay can be referred in multiple papers, for example as the equation #34 in the technical note provided from NIST [3], shown by equation (1) below. And the propagation delay error caused by the rotation of the Earth is called Sagnac effect (or Sagnac correction).

If ECI position vectors are referenced to the time of signal transmission, then

tD=tR-tT=rtR-RtTc≈rtT-RtTc+rtT-RtT⋅vc2#1

Where

tD is the total propagation delay,

R is the satellite position vector,

r is the receiver position vector,

tT and tR are GPS time at respectively transmit and receive times, and

v is the receiver velocity vector in the ECIF.

The last term in the right side of the equation (1) is the Sagnac effect which takes into account of the rotation of the Earth. And as can be seen in the equation, calculation of the propagation delay is made based on the timing when signals are transmitted and the receiver velocity on ECI frame.

Observation 3: When considering an effect of a rotation of the Earth in ECI frame, an equation to calculate a total propagation delay can be expressed as follows.

tD=tR-tT=rtR-RtTc≈rtT-RtTc+rtT-RtT⋅vc2#1

Based on the equation (1), we will slightly modify the way to show variables of the equation first from the downlink case. To share the image of the DL case in the group, the image of this calculation is shown in the Figure 2.2.1-1, and we added some suffix to clarify the assumptions of each variable.

Figure 2.2.1-1: Propagation delay calculation of DL case including Sagnac effect

Where

SV (Satellite)		: Transmitter

UE 				: Receiver

ΔtDL= tR- tT  : Downlink total propagation delay

tT					: Time when signals are transmitted from satellite

tR					: Time when signals are received by UE

rR 					: UE position vector at transmitted time tT on ECI Frame

vUE 				: UE velocity vector at transmitted time tT on ECI Frame

rT 					: SV position vector at transmitted time tT on ECI Frame

RDL=rR-rT

ωE  = [0, 0, 7.292115E-5], (|ωE| = 7.292115 x 10-5 [red./s])

c = 2.99792458 x 108 [m/s]

The equation (1) can be slightly modified and expressed as follows.

ΔtDL=RDL(tT)c+vUE(tT)∙RDL(tT)c2#2

The equation (2) can be further transformed. When the first term of the right side of the equation is sufficiently larger than the second term, we can apply the math expression that 1+x=1/(1-x). Therefore, the equation (2) can be transformed to the following form.

Observation 4: The equation (1) can be slightly modified and expressed as follows for the downlink case.

ΔtDL=RDLtTc1-1cvUEtT・RDLtTRDLtT#2'

2.2.2 Views on the proposed equation in WF [1]

Here we would like to share our views on the proposed equation in the previous WF [1]. The associated equation is extracted below for reference.

DelayERtR=ρtRc1-1c<vtRECEF+ωEK×rtRECEF,ρtRρtR>#3

Where tR is the signal’s received time, rtRECEFand vtRECEF are the satellite position and velocity vectors at time tR , ρtRis the difference in satellite and UE position vectors at time tR,  K  is the unit vector in standard z-axis, (a×b) is the cross-product and <a, b> is the dot product between a and b

- When we compare the equation (2’) and (3), though they look similar, there are mainly two differences between them.

- As mentioned above, vtRECEF is the velocity vector of the satellite in the equation (3) while vUE is the UE velocity vector in the equation (2’).

- tR is the signal’s received time in the equation (3) while tT is transmitted time in the equation (2’).

From these two points and based on the explanation of equation (1), since v is the receiver velocity vector in (1), it looks the equation (3) is calculating the total propagation delay in a case of the uplink transmission using the received timing instead of the transmitted timing. Because the original proposal in the group was to consider the effect of the Earth’s rotation, and based on the Figure 2.2.1-1 above, we assume that we need to care the velocity of the receiver (i.e. UE) in the DL case and the timing of the transmission of signals. Thus, we would like to encourage companies to cross check the equations.

Observation 5: Based on the assumption of equation (1), v is the velocity vector of the receiver on ECI frame. In that sense, since v in the equation (3) is the satellite velocity vector, it appears that the equation (3) is calculating the total propagation delay of the uplink transmission case.

Proposal 1: Companies are encouraged to cross check the equations to derive the total propagation delay and its error.

We will show our calculation result of the propagation delay error later in this document. Since there is a difference between the obtained result in [2] and ours, we also propose companies to bring calculated values from each company.

Proposal 2: Companies are encouraged to bring calculated propagation delay error results.

- As shown by the equation (1), the associated effect depends on the velocity of the receiver, i.e. velocity of the UE corresponding the rotation of the Earth in the downlink case, and velocity of the satellite in the uplink case. Therefore, we think that the different equations and corrections must be considered in the downlink case and uplink case. If we consider uplink timing test case for RRM, since the UE adjusts the transmission timing by the reference signals from the satellite, UL timing may be influenced by both the DL signals from the satellite to UE and the UL signals from UE to the satellite. Saying that, the necessity of the corrections should be decided based on the obtained delay errors.

Observation 6: Since the equation (1) contains the receiver velocity vector, it is expected that the effect may vary depending on the case of the downlink and uplink. Study should be made for both the DL case and the UL case.

Observation 7: A necessity of the corrections due to the velocity of the receiver should be decided based on the analysis of the propagation delay errors for both DL case and UL case.

- This is not limited to this delay error calculation case, but since we have been implicitly using the assumption of WGS-84 frame as an ellipsoid to define several parameters like wE, WGS-84 needs to be commonly used as the ellipsoid to determine the geodetic coordinate system. Otherwise, we may have an inconsistency with other parameters, and the UE position and UE velocity cannot be correctly obtained which may cause the additional error of the propagation delay error. In addition to that, since GPS is using WGS-84 and we would obtain the position of the UE by GPS, we shall align that assumption with GPS, too.

Observation 8: When considering the propagation delay error (Sagnac effect), to align an assumption with other parameters, WGS-84 shall be used as the ellipsoid for the geodetic coordinate system.

- Related to the Sagnac effect, we also noticed in the reference paper [3] that the effect to a doppler shift may also need to be considered. The associated equation can be referred to equation (D.15) in [3], also shown below by equation (4).

Ff=1+ΦrR-ΦrTc2-v22c2+V22c2+N∙v-Vc-N∙vc∙N∙Vc+N∙vc2#4

Where

F		: Transmitted frequency

f		: Received frequency

V		: Transmitter velocity vector in ECI coordinates

v		: Receiver velocity vector in ECI coordinates

N		: ECI unit vector in the direction of signal propagation

Φ(r)	: Newtonian gravitational potential at position r in ECI coordinates

rR 		: Receiver position vector at transmitted time tT on ECI Frame

rT 		: Transmitter position vector at transmitted time tT on ECI Frame

Due to the limitation of time, we haven’t started the analysis of this influence on the doppler shift. But since the propagation delay and the doppler shift are closely related, we think we need to consider them as a package and need to decide a way forward after reviewing the impact also to the doppler shift.

Observation 9: The Sagnac effect also has an impact on the calculation of the doppler shift.

Proposal 3: Consider the Sagnac effect on the propagation delay and the doppler shift as a package and decide a way forward after reviewing the impact. Whether or not applying the Sagnac effect to the calculation of delay and doppler shift is FFS.

2.3 Calculation of total propagation delay

2.3.1 Calculation of total propagation delay for downlink case by equation (2)

To compare with the calculated propagation delay error (i.e. Sagnac effect) in the previous contribution [2], we also tried deriving it based on our equation (2) for DL case along with our equation (5) for UL case which we will show at the later part in this sub-clause. This time we conducted the calculation with the zero doppler condition, and other factors are listed in the appendix at the end of this paper. Here we also extract the associated part from the contribution [2] to refer to the point at 128 seconds.

Extract from [2] Fig. 2: Propagation delay error due to ignoring Earth's rotation during signal flight time As illustrated in Fig. 2, at 262 seconds (corresponding to a 10-degree elevation), the delay estimation error is 0.1493 µs, while at 128 seconds (corresponding to a 30-degree initial elevation angle), the delay estimation error is 0.075 µs. We note that the maximum delay error could reach 0.5 µs. This propagation delay error could be significant for certain test cases, such as UL timing test cases. Therefore, we believe this phenomenon warrants further study in RAN4.

As mentioned before, since the previous equation is based on the ECI frame, all the values need to be transformed into ones to fit ECI frame. Table 2.3.1-1 shows the summary of transformation law.

Table 2.3.1-1: Summary of transformation law

|  | ECI Frame: Σ (O,xyz) | ECEF Frame: Σ'(O,x'y'z') | Transformation law |
| --- | --- | --- | --- |
| Bases | ex , ey , ez | ex' , ey' , ez' | － |
| Position | r | r' | r=r' |
| Velocity | vΣ | vΣ' | v Σ=vΣ'+ω×r' |
| Acceleration | aΣ | aΣ' | a Σ =aΣ'+2 ω×vΣ' +ω×(ω×r') |

Calculation of DL propagation delay at t=128 [s]

Step 1: Transformation of Satellite position and velocity from ECEF to ECI at tT

By applying the transformation law in Table 2.3.1-1, we calculate the position and velocity of the satellite at tT (@128 s) as follows. However, values for the satellite velocity are not used for the downlink case. These values are for the UL case which appears later.

rSVECI = rSVECEF

VSVECI = VSVECEF + (ωE×rSVECEF)

Satellite position on ECEF

| Time [s] | rx,SVECEF [m] | ry,SVECEF [m] | rz,SVECEF [m] |
| --- | --- | --- | --- |
| 128 | -3119107.6440345100000000 | 4974557.7431512100000000 | 3787355.7799740000000000 |

Satellite velocity in ECEF

| Time [s] | Vx,SVECEF [m/s] | Vy,SVECEF [m/s] | Vz,SVECEF [m/s] |
| --- | --- | --- | --- |
| 128 | 1736.7646846242100000 | -3692.4648329521400000 | 6342.0523881150100000 |

↓

Satellite position on ECI

| Time [s] | rx,SVECI [m] | ry,SVECI [m] | rz,SVECI [m] |
| --- | --- | --- | --- |
| 128 | -3119107.6440345100000000 | 4974557.7431512100000000 | 3787355.7799740000000000 |

Satellite velocity in ECI

| Time [s] | Vx,SVECI [m/s] | Vy,SVECI [m/s] | Vz,SVECI [m/s] |
| --- | --- | --- | --- |
| 128 | 1374.0142132522200000 | -3919.9137493289300000 | 6342.0523881150100000 |

Step 2: Transformation of UE position and velocity from ECEF to ECI at tT

Similar to above for the satellite, by applying the transformation law in Table 2.3.1-1, we calculate the position and velocity of the UE at tT (@128 s) as follows.

rUEECI = rUEECEF

vUEECI = vSVECEF + (ωE×rUEECEF)

UE position on ECEF

| Time [s] | rx,UEECEF [m] | ry,UEECEF [m] | rz,UEECEF [m] |
| --- | --- | --- | --- |
| 128 | -3025296.9357793700000000 | 4925102.7898296000000000 | 2687544.2100484500000000 |

UE velocity in ECEF

| Time [s] | vx,UEECEF [m/s] | vy,UEECEF [m/s] | vz,UEECEF [m/s] |
| --- | --- | --- | --- |
| 128 | 0.0000000000000000 | 0.0000000000000000 | 0.0000000000000000 |

↓

UE position on ECI

| Time [s] | rx,UEECI [m] | ry,UEECI [m] | rz,UEECI [m] |
| --- | --- | --- | --- |
| 128 | -3025296.9357793700000000 | 4925102.7898296000000000 | 2687544.2100484500000000 |

UE velocity in ECI

| Time [s] | vx,UEECI [m/s] | vy,UEECI [m/s] | vz,UEECI [m/s] |
| --- | --- | --- | --- |
| 128 | -359.1441593025830000 | -220.6081316485070000 | 0.0000000000000000 |

Step 3: Calculation of RDLat tT

Based on the previous equation  RDL=rR-rT,  we calculate the RDL at tT (@128 s). Here we assume the position vectors as follows.

rT = rSVECI (Satellite is the transmitter)

rR = rUEECI (UE is the receiver)

Here the last column of the following table is root sum square of Rx, Ry and Rz.

RDL=rR-rT

| Time [s] | Rx,DLECI(tT) [m] | Ry,DLECI(tT) [m] | Rz,DLECI(tT) [m] | ¦ RDLECI(tT) ¦ [m] |
| --- | --- | --- | --- | --- |
| 128 | 93810.7082551443000000 | -49454.9533216012000000 | -1099811.5699255400000000 | 1104912.5443823400000000 |

Step 4: Calculation of Propagation delay and Propagation delay error (Sagnac effect)

Based on the equation (2) and obtained values until Step 3 above, we calculated propagation delay and the propagation delay error (i.e. Sagnac effect) at tT (@128 s).

Delay DL = | RDL | / c

Propagation delay error (Sagnac DL) = ( vUE・RDL ) / c2

| DelayDL (=RDL/c) [s] | SagnacDL [ns] |
| --- | --- |
| 0.0036855915313999 | -0.2534773 |

From the calculation result above, the propagation delay error for DL case at 128s point is approximately -250 ps, which appears to be negligible compared to the propagation delay.

And comparing to the value provided in the contribution [2] (i.e. 0.075 ms), it seems extremely small. Thus, further cross check might be necessary.

Observation 10: The calculated propagation delay error (i.e. Sagnac effect) for DL case at 128s point is approximately -250 ps, which is 1/300 compared to the value provided in [2]. Further cross check might be necessary.

Figure 2.3.1-1 and Figure 2.3.1-2 depicts the obtained propagation delay and propagation delay error (Sagnac effect) for downlink case.

Figure 2.3.1-1: Propagation delay for downlink without Sagnac effect

Figure 2.3.1-2: Propagation delay error (Sagnac effect) for downlink

2.3.2 Calculation of total propagation delay for uplink case

As mentioned at Observation 6 above, we assume that there is a need to study the propagation delay error also for the uplink case since the velocity of the receiver in the equation is replaced by the satellite this case. Figure 2.3.2-1 is depicted to show our idea of the calculation for the uplink.

Figure 2.3.2-1: Propagation delay calculation of UL case including propagation delay error

Where

SV (Satellite)		: Receiver

UE					: Transmitter

ΔtUL= tR- tT 	: Uplink total propagation delay

tT					: Time when signals are transmitted from UE

tR					: Time when signals are received by satellite

rR 					: SV position vector at transmitted time tT on ECI Frame

VSV 				: SV velocity vector at transmitted time tT on ECI Frame

rT 					: UE position vector at transmitted time tT on ECI Frame

RUL=rR-rT

ωE  = [0, 0, 7.292115E-5], (|ωE| = 7.292115 x 10-5 [red./s])

c = 2.99792458 x 108 [m/s]

Reusing the equation (2), the following equation can be obtained to calculate the total propagation delay for the uplink case.

Observation 11: Reusing the equation (2), the equation to calculate the total propagation delay for the uplink case can be expressed as follows.

ΔtUL=RUL(tT)c+VSV(tT)∙RUL(tT)c2#5

2.3.3 Calculation of total propagation delay for uplink case by equation (5)

Calculation of UL propagation delay at t=128 [s]

Step 1 and 2:

Refer to the downlink case in sub-clause 2.3.1 above since step 1 and 2 to transform the satellite / UE position and velocity are completely same.

Step 3: Calculation of RULat tT

Based on the above equation  RUL=rR-rT,  we calculate the RUL at tT (@128 s). Here we assume the position vectors as follows.

rT = rUEECI (UE is the transmitter)

rR = rSVECI (Satellite is the receiver)

Here the last column of the following table is root sum square of Rx, Ry and Rz.

RUL=rR-rT

| Time [s] | Rx,ULECI(tT) [m] | Ry,ULECI(tT) [m] | Rz,ULECI(tT) [m] | ¦ RULECI(tT) ¦ [m] |
| --- | --- | --- | --- | --- |
| 128 | -93810.7082551443000000 | 49454.9533216012000000 | 1099811.5699255400000000 | 1104912.5443823400000000 |

Step 4: Calculation of Propagation delay and Propagation delay error (Sagnac effect)

Based on the equation (5) and obtained values until Step 3 above, we calculate propagation delay and the propagation delay error (Sagnac effect) at tT (@128 s).

Delay UL = | RUL | / c

Propagation delay error (Sagnac UL) = ( VSV・RUL ) / c2

| DelayUL (=RUL/c) [s] | SagnacUL [us] |
| --- | --- |
| 0.0036855915313999 | 0. 0740168886 |

From the calculation results above, the propagation delay error for UL case at 128s point is approximately 0.074 us which is larger compared to the DL case error (i.e. approx. -250 ps).

And this value appears to be matching the value provided in the contribution [2] (i.e. 0.075 ms).

Observation 12: The calculated propagation delay error for UL case at 128s point is approximately 0.074 ms, which is larger compared to the DL case error.

Figure 2.3.3-1 and Figure 2.3.3-2 depict the obtained propagation delay and propagation delay error (Sagnac effect) for uplink case.

Figure 2.3.3-1: Propagation delay for uplink without Sagnac effect

Figure 2.3.3-2: Propagation delay error (Sagnac effect) for uplink

- Conclusion

In this contribution we share our views on the following issue 1-1-1 and issue 1-1-2 in the previous WF [1].

Observation 1: There is a need to agree on multiple parameters and configuration files to use commonly for conducting a comparison of the satellite orbit between GMAT and two-body Keplerian along with the comparison among companies.

Observation 2: Associated parameter file for the selected gravity model is also necessary.

Observation 3: When considering an effect of a rotation of the Earth in ECI frame, an equation to calculate a total propagation delay can be expressed as follows.

tD=tR-tT=rtR-RtTc≈rtT-RtTc+rtT-RtT⋅vc2#1

Observation 4: The equation (1) can be slightly modified and expressed as follows for the downlink case.

ΔtDL=RDLtTc1-1cvUEtT・RDLtTRDLtT#2'

Observation 5: Based on the assumption of equation (1), v is the velocity vector of the receiver on ECI frame. In that sense, since v in the equation (3) is the satellite velocity vector, it appears that the equation (3) is calculating the total propagation delay of the uplink transmission case.

Proposal 1: Companies are encouraged to cross check the equations to derive the total propagation delay and its error.

Proposal 2: Companies are encouraged to bring calculated propagation delay error results.

Observation 6: Since the equation (1) contains the receiver velocity vector, it is expected that the effect may vary depending on the case of the downlink and uplink. Study should be made for both the DL case and the UL case.

Observation 7: A necessity of the corrections due to the velocity of the receiver should be decided based on the analysis of the propagation delay errors for both DL case and UL case.

Observation 8: When considering the propagation delay error (Sagnac effect), to align an assumption with other parameters, WGS-84 shall be used as the ellipsoid for the geodetic coordinate system.

Observation 9: The Sagnac effect also has an impact on the calculation of the doppler shift.

Proposal 3: Consider the Sagnac effect on the propagation delay and the doppler shift as a package and decide a way forward after reviewing the impact. Whether or not applying the Sagnac effect to the calculation of delay and doppler shift is FFS.

Observation 10: The calculated propagation delay error (i.e. Sagnac effect) for DL case at 128s point is approximately -250 ps, which is 1/300 compared to the value provided in [2]. Further cross check might be necessary.

Observation 11: Reusing the equation (2), the equation to calculate the total propagation delay for the uplink case can be expressed as follows.

ΔtUL=RUL(tT)c+VSV(tT)∙RUL(tT)c2#4

Observation 12: The calculated propagation delay error for UL case at 128s point is approximately 0.074 ms, which is larger compared to the DL case error.

# 4.	References

[1] R4-2502314, “Way Forward for [114][325] NTN_testing_NGSO_channel_model l”, Moderator (Samsung), RAN4 #114, Athens, Greece

[2] R4-2500116, “Discussion on NTN testing for NGSO”, Qualcomm Inc., RAN4 #114, Athens, Greece

[3] NIST Technical Note 1385, “Global Positioning System Receivers and Relativity”, March 1999

Global Positioning System Receivers and Relativity | NIST

[4] Relativity in Rotating Frames: Relativistic Physics in Rotating Reference Frames, G.Rizzi and M.L.Ruggiero, Springer, ISBN 1-4020-1805-3.

Appendix A: Parameters for propagation delay calculation

TS 38.508-1

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 | -3315.1053 (km) |  |
| positionY-r17 | 4151097 | 5396.4261 (km) |  |
| positionZ-r17 | 2263034 | 2941.9442 (km) |  |
| velocityVX-r17 | 21970 | 1.31820 (km/s) |  |
| velocityVY-r17 | -48164 | -2.88984 (km/s) |  |
| velocityVZ-r17 | 114109 | 6.84654 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

UE location (TS 38.508-1)

5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

The position and velocity of the satellite described in ECEF Frame need to be converted based on the definitions in TS 38.331 as EphemerisInfo field descriptions.

EphemerisInfo field descriptions

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

The converted satellite coordinate in ECEF Frame (X_SV, Y_SV, Z_SV):

(X_SV, Y_SV, Z_SV) = ( -3315105.300000 [m], 5396426.100000 [m], 2941944.200000 [m])

The converted satellite velocity in ECEF Frame (VX, VY, VZ):

(VX, VY, VZ) = (1318.200000 [m/s], -2889.840000 [m/s], 6846.540000 [m/s])

Also, the position of the UE needs to be converted to ECEF Frame by the following equations since the definitions of the UE location in TS 38.508-1 above are described based on the ellipsoidal coordinate system.

xyz=N+hcosϕcosλN+hcosϕsinλN1-e2+hsinϕ#

Here f is geodetic latitude, l is east longitude, h is altitude from geoid, a is average earth equatorial radius, e=f(2-f)  where f is the earth’s declination, and

N=a1-e2sin2ϕ1 2

The converted UE coordinate in ECEF Frame:

(X_UE, Y_UE, Z_UE) = (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

Initial conditions of satellite position, velocity and UE position in ECEF frame

(X_SV, Y_SV, Z_SV) = ( -3315105.300000 [m], 5396426.100000 [m], 2941944.200000 [m])

(VX, VY, VZ) = (1318.200000 [m/s], -2889.840000 [m/s], 6846.540000 [m/s])

(X_UE, Y_UE, Z_UE) = (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])