3GPP TSG-RAN4 Meeting #114-bis	R4- 2503722

# Wuhan, China, 6th Apr 2025 – 11th Apr 2025

Source:	LG Electronics

Title:	Discussion for UE RF Rx requirements of NTN_NR_HPUE

Agenda item:	7.10.2.2.2

Document for:	Discussion

Introduction

In the RAN4#114 meeting,  the WF on HPUE requirements for NR-NTN was approved. In this discussion, we share our point of view for the HPUE requirements for NR-NTN based on the approved WF[1]

Discussion

Table 1 Way forward for UE RF requirements for NTN_NR_HPUE [1]

# Open issues Issue 3-1-3: RSD requirements for PC2 on n255 - Proposals - Option 1: Duplex isolation as 51, and PA noise as -117 dBm/Hz (MediaTek) Operating Band n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). 1Tx; [0.4]; [0.4]; [0.5]; [0.5]. 2Tx; [0.8]; [0.8]; [0.9]; [0.9]. - Option 2: Discuss if only default Tx-Rx frequency separation is used. (Huawei) - PA noise as -117 dBm/Hz. - Option 3: Assuming IIP2 54dBm (Qualcomm) Operating Band n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). 1Tx; 1.3; 1.3; 1.3; 1.3. 2Tx; 2.4; 2.4; 2.4; 2.4. Way forward: - Continue align the assumptions and further discuss above options. Issue 3-2: RSD requirements for PC1.5 - Proposals - Option 1: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only if defined. (vivo) - Option 2: Regarding NR NTN HPUE operated in PC1.5, discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254. (MediaTek) Table 4 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [1.8]; [1.8]; [1.8]; [1.8]. n255; [2.4]; [2.4]; [2.5]; [2.5]. n254; [1.8]; [1.8]; [1.8]. Table 5 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [2.2]; [2.2]; [2.2]; [2.2]. n255; [3.2]; [3.2]; [3.3]; [3.3]. n254; [2.2]; [2.2]; [2.2]. - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson) Way forward: - Further discuss above options. Issue 3-3: RSD requirements for PC1 - Proposals - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. And FFS PC1 2Tx or 3Tx. (vivo, Samsung) - Option 2: Regarding NR NTN HPUE operated in PC1, discuss whether the RX RSD values as shown in tables below could be used for band n256. Table 6 RSD from PC3 to PC1 for FDD bands for NR NTN UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3]; [3]; [3]; [3]. Table 7 RSD from PC3 to PC1 for FDD bands for NR NTN UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3.7]; [3.7]; [3.7]; [3.7]. - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson) Way forward: - Further discuss above options.

Reference sensitivity degradation for NTN HPUE

For HPUE in the FDD band, an increase in UL output power can lead to a higher noise level in the DL band than PC3. Considering this, the current TN spec defines RSD for the PC2 FDD band. However, despite the definition of the PC1 FDD band, RSD is not defined for it because PC1 considers a non-handheld form factor only, which provides better isolation characteristics than handheld devices. The current NTN HPUE configuration is being discussed as shown in Table 2.

Table 2 Tx configuration cadidates of NTN according to the power class

|  | 1Tx |  | 2Tx |  | 4Tx |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDD | TDD | FDD | TDD | FDD | TDD |
| PC3 | O (NH and HH) |  |  |  |  |  |
| PC2 | O (NH and HH) |  | O (NH and HH) |  |  |  |
| PC1.5 |  |  | O (NH) Other cases are not precluded |  |  |  |
| PC1 | O (NH) |  |  |  | [O] (NH) |  |

HH: handheld, NH: non-handheld

As shown in Table 1, there is no need to define RSD for PC1.5 and PC1, which only consider non-handheld devices. RSD needs to be defined only for PC2, which considers both non-handheld and handheld devices.

Observation 1: No need to define RSD for PC1.5 and PC1, which only consider non-handheld devices. RSD needs to be defined only for PC2, which considers both non-handheld and handheld devices.

Proposal 1: It is necessary to define the RSD requirement only for NTN HPUE PC2.

Table 3 RSD on mid band in

| Band | UL_L | UL_H | DL_L | DL_H | UL –DL separation ( MHz) | RSD @ 5 MHz 1Tx | RSD @ 5 MHz 2Tx |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 1920 | 1980 | 2110 | 2170 | 190 | 0 | 0 |
| n2 | 1850 | 1910 | 1930 | 1990 | 80 | 0.8 | 1.2 |
| n3 | 1710 | 1785 | 1805 | 1880 | 95 | 0.5 | 1.4 |
| n7 | 2500 | 2570 | 2620 | 2690 | 120 | 0.5 | 0.9 |
| n25 | 1850 | 1915 | 1930 | 1995 | 80 | 0.8 | 1.5 |
| n66 | 1710 | 1780 | 2110 | 2200 | 400 | 0 | 0 |
| n70 | 1695 | 1710 | 1995 | 2020 | 300 | 0 | 0 |

Table 3 summarizes the mid band RSD as defined in TS38.101-1. From the table, it can be seen that for bands with a Tx-Rx frequency separation of 190 MHz or more, the RSD is defined as 0. Additionally, for bands n2, n3, n7, and n25, where the UL-DL separation is around 100 MHz, the RSD shows a maximum difference of 0.3 dB for 1Tx and 0.6 dB for 2Tx, indicating that the differences are not significant.

Table 4 UE TX-RX frequency separation (FR1-NTN) in TS 38.101-5

Table 4 shows the Tx-Rx frequency separation for NTN bands. In the case of bands like n256 and n254, since the Tx-Rx frequency separation is 190 MHz or more, there is no need to define the RSD as in TN HPUE. Additionally, for the RSD value of the n255 band, it is necessary to define it in a way that does not significantly differ from the RSD values defined for the existing TN mid bands (n2, n3, n7, n25).

Observation 2: In the case of bands like n256 and n254, since the Tx-Rx frequency separation is 190 MHz or more, there is no need to define the RSD as in TN HPUE. Additionally, for the RSD value of the n255 band, it is necessary to define it in a way that does not significantly differ from the RSD values defined for the existing TN mid bands (n2, n3, n7, n25).

Proposal 2: Do not specify the RSD requirements for n256 and n254.

RSD analysis results

Assumption

- PA emission power: -117 dBm/Hz for 1Tx, -113 dBm/Hz for 2Tx

- duplexer isolation factor : 51 for both 1Tx and 2Tx

Table 5 n255 RSD values

|  | 5MHz | 10MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| RSD@1Tx | 0.4 | 0.4 | 0.4 | 0.4 |
| RSD@2Tx | 1.0 | 1.0 | 1.0 | 1.0 |

In our analysis, the PA emission power assumption for 2Tx is higher than for 1Tx because the Tx signals on 2Tx leak into each other through the antenna. Therefore, the emission power on 2Tx is higher than the PA emission power for 1Tx.

Proposal 3: Proposed RSD values for n255 as shown in table below

|  | 5MHz | 10MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| RSD@1Tx | 0.4 | 0.4 | 0.4 | 0.4 |
| RSD@2Tx | 1.0 | 1.0 | 1.0 | 1.0 |

Conclusion

Observation 1: No need to define RSD for PC1.5 and PC1, which only consider non-handheld devices. RSD needs to be defined only for PC2, which considers both non-handheld and handheld devices.

Observation 2: In the case of bands like n256 and n254, since the Tx-Rx frequency separation is 190 MHz or more, there is no need to define the RSD as in TN HPUE. Additionally, for the RSD value of the n255 band, it is necessary to define it in a way that does not significantly differ from the RSD values defined for the existing TN mid bands (n2, n3, n7, n25).

Proposal 1: It is necessary to define the RSD requirement only for NTN HPUE PC2.

Proposal 2: Do not specify the RSD requirements for n256 and n254.

Proposal 3: Use RSD values for n255 PC2 as shown in table below.

|  | 5MHz | 10MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| RSD@1Tx | 0.4 | 0.4 | 0.4 | 0.4 |
| RSD@2Tx | 1.0 | 1.0 | 1.0 | 1.0 |

- References

# [1] R4-2502954, WF on HPUE requirments for NR-NTN (Samsung)