TSG-RAN Working Group 4 (Radio) meeting #114-bis	R4-2503729

Wuhan, China, 7th to 11th April 2025

Source:	Ericsson

Title:	On the topic of BS RF specification structure improvements

Agenda item:	9.1

Document for:	Approval

# Introduction

At the RAN plenary meeting in Madrid (RAN#106) it was decided not to include the work related to BS RF specification structure improvements into the running BS RF evolution WI. Instead RAN tasked RAN4 to work on a framework aiming for a better specification structure, with respect to transmitter spurious emission requirement for:

- Co-existence with other base stations in the same geographical area.

- Co-location with other base stations at the same site.

The agreement is captured in a moderator summary as an outcome of the discussion related to WID update for Rel-19 BS RF evolution WI in RP-243288 [1].

The agreement can be summarized as:

- Add the new objective of co-existence transmitter spurious emission limits according to the request from ETSI/TFES. Start the work after March 2025 after EERIP work is finished. Prioritize above 5.9 GHz.

- Task RAN4 to work on framework simplification for co-location/co-existence requirements under a dedicated agenda in Q1 and Q2 2025 to come up with the concrete solution and come back in RAN#108 if necessary.

For the work related to improvement of BS RF specifications (including other network node specifications), agreement in bullet 2 is relevant.

In this contribution we present a solution to improve the BS RF specification structure with respect to transmitter spurious emission requirements for co-existence and co-location. In companion contributions we indicate the impact of the implementation in TS 38.104, TS 38.141-1 and TS 38.141-2.

# Discussion

At the last RAN4 meeting (RAN4#114 In Athens) improvements to the specification structure was discussed. The outcome was captured in a topic summary [2]. From the discussion there are two approaches to consider:

- Reduce large tables for co-existence and co-location requirements. This can be done by formulating a general requirement and list exceptions in a short table. With this approach no updates are needed is general requirement is adopted for a new band.

- Reduce duplication of requirements in many specifications. This can be solved by having a new separate specification with co-existence and co-location requirements. With this approach references are used in all BS RF node specifications. If a new band is added, updates are only required for the new specification with co-existence and co-location requirements.

If approach (1) is adopted, the need for many maintenance CR(s) disappear under the condition that the general requirement is adopted for new bands added. Also, the network node specifications stay self-contained with all requirements captured in a common specification per node.

If approach (2) is used a new specification needs to be updated, which would reduce number of maintenance CR(s) but increase the burden to maintain a new specification.

Proposal 1: To improve BS RF specification structure for co-existence and co-location spurious emission requirements adopt approach (1), where the general requirements are captured as common entry, and exceptions is listed as separate entry in the table.

From a BS node specification point of view, it is beneficial to maintain specifications as complete as possible without chains of references. Therefore, it is suggested to adopt approach (1) for all relevant BS RF node specifications.

The very long tables used in current specifications can be restructured in a much shorter table as indicated in Table 2-1 and Table 2-2.

Table 2-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Condition | Basic limits (dBm/MHz) | Note |
| --- | --- | --- | --- |
| Frequency range of downlink operating band | Co-existence with GSM850, CDMA850 or GSM900 | -57 | Note 1 |
|  | Co-existence with DCS1800 or PCS1900 | -47 |  |
|  | Other | -52 |  |
| Frequency range of uplink operating band | Co-existence with GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | Note 1, 2, 3, 4 |
|  | Other | -49 |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in table 6.6.5.2.3-1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 2: For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band. NOTE 3: For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band. NOTE 4: For NR Band n67 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. |  |  |  |

Table 2-2: BS spurious emissions basic limits for BS co-located with another BS

| Frequency range for | Condition | Basic limits |  |  | Measurement | Note |
| --- | --- | --- | --- | --- | --- | --- |
| co-location requirement |  | WA BS (dBm) | MR BS (dBm) | LA BS (dBm) | bandwidth |  |
| Frequency range of co-located uplink operating band | Co-located with GSM850, CDMA850 or GSM900 | -98 | -91 | -70 | 100 kHz | Note 1 |
|  | Co-located with DCS1800 or PCS1900 | -98 | -91 | -80 | 100 kHz |  |
|  | n51, n91, n93 | N/A | N/A | -88 | 100 kHz |  |
|  | n53 | N/A | -91 | -88 | 100 kHz |  |
|  | n100, n101 | -96 | N/A | N/A | 100 kHz |  |
|  | n96, n102 | N/A | -90 | -87 | 100 kHz |  |
|  | n104 | -95 | -90 | -87 | 100 kHz |  |
|  | Other | -96 | -91 | -88 | 100 kHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.6.5.2.4-1 do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band (see table 5.2-1). |  |  |  |  |  |  |

# Conclusion

In this contribution we show a solution for how the requirements for co-existence and co-location can be simplified by adopting a new structure for the table format used to capture the requirement.

Proposal 1: To improve BS RF specification structure for co-existence and co-location spurious emission requirements adopt approach (1), where the general requirements are captured as common entry, and exceptions is listed as separate entry in the table.

In companion contributions we provide draft CR to TS 38.104 [4], draft CR to TS 38.141-1 [5] and draft CR to TS 38.141-2 [6].

# References

[1]	RP-243288, “Moderator's summary of discussion about R19-RAN4-BSRF”, Huawei

[2]	R4-250572, “Topic summary for [114][331] BSRF_Simplify_CoLo_Coex”, Ericsson

[3]	R4-2500880, “On the topic of BS RF specification structure improvements”, Ericsson

[4] 	R4-2503727, “Draft CR to TS 38.104: Introduction of new requirement structure for transmitter spurious emission in subclause 6.6.5.2.3 and subclause 6.6.5.2.4”, Ericsson

[5]	R4-2503730, “Draft CR to TS 38.141-1: Introduction of new requirement structure for transmitter spurious emission in sub-clause 6.6.5.5.1.3 and sub-clause 6.6.5.5.1.4”, Ericsson

[6]	R4-2503728, “Draft CR to TS 38.141-2: Introduction of new requirement structure for transmitter spurious emission in sub-clause 6.7.5.4.5 and sub-clause 6.7.5.5.5”, Ericsson