3GPP TSG-RAN WG4 Meeting #114bis	R4-2503742

Wuhan, China, 7th – 11st April 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN RF requirements for NR NTN Ku band

# Agenda item:	7.11.5

# Document for:	Approval

# 1 Introduction

In RAN#107 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from system parameters perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

# 2 Discussion

  - 2.1 SAN radiated Tx requirements

- Issue 2-1-1: Discussion on the supported SAN type for NR NTN Ku band - Regarding the SAN requirement for the NR NTN Ku band: - Define SAN type 1-O requirements for the FR1-NTN Ku bands - Define SAN type 2-O requirements for the FR2-NTN Ku bands - The SAN type 1-H will not be defined for the FR1-NTN Ku bands - Issue 2-1-2: Alignment of SAN requirement between FR1-NTN and FR2-NTN - Aim to align the following SAN requirements between the FR1-NTN Ku bands and the FR2-NTN Ku bands at least - The tolerance for the radiated transmit power and SAN output power for FR1-NTN and FR2-NTN - The spurious frequency range for FR1-NTN and FR2-NTN - Companies are encouraged to further check other requirements between FR1-NTN and FR2-NTN, including the following aspects - Companies are encouraged to further check whether the RE power control dynamic range, OTA sensitivity and Rx dynamic range requirements shall be specified for NTN Ku-bands - FFS on other requirements - Issue 2-1-4: PFD limits - The PFD limits at the Earth’s surface specified by FCC (47 CFR 25.208) for the Ku-band will not be specified in the 3GPP specification. - WF from the email discussion for the SAN RF part: - Issue 2-1-5: Impact on SAN RF requirements for the NTN Ku band - When introducing radiated SAN requirement for the Ku band, there will be no specification impact on the following sections, (i.e., the existing requirements applies): - Section 9.5 (OTA transmit ON/OFF power), 9.6 (OTA transmitted signal quality), 9.7.2 (OTA occupied bandwidth), 9.7.4 (OTA out-of-band emissions), 9.7.5 (OTA transmitter spurious emissions), 10.2 (OTA sensitivity)

    - 2.1.1 Radiated transmit power

According to the way forward in last meeting, we agreed to introduce SAN type 1-O and 2-O for NR NTN Ku bands. And the proposal of aligning the tolerance for the radiated transmit power for FR1-NTN and FR2-NTN was raised in last meeting. From our understanding, it is reasonable to define the same requirement for radiated transmit power for both FR1-NTN Ku bands and FR2-NTN Ku bands. In terms of the detailed requirements, we propose to reuse the requirement defined for Ka band. Hence, it is proposed to define the tolerance as ± 3.4 dB for the radiated transmit power for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 1: It is proposed to define the tolerance as ± 3.4 dB for the radiated transmit power for both FR1-NTN Ku bands and FR2-NTN Ku bands.

    - 2.1.2 OTA Satellite Access Node output power

For OTA Satellite Access Node output power, to align requirement between the FR1-NTN Ku bands and the FR2-NTN Ku bands, we also propose to reuse the requirement specified for Ka band. Therefore, it is proposed to specify the tolerance as ± 3 dB for OTA SAN output power for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 2: It is proposed to define the tolerance as ± 3 dB for OTA SAN output power for both FR1-NTN Ku bands and FR2-NTN Ku bands.

    - 2.1.3 OTA output power dynamics

From our understanding, Satellite access networks operating in above 10 GHz are most likely to operate simultaneously in several NTN bands (i.e. Ku and Ka-band). In terms of OTA RE power control dynamic range, we think there is no need to specify this requirement for NTN Ku bands like Ka band.

Proposal 3: Not to specify OTA RE power control dynamic range for NTN Ku bands.

For OTA total power dynamic range, it is channel bandwidth dependent requirement. Since there are some new channel bandwidths are introduced for FR1-NTN Ku bands: 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing, we need to specify the corresponding total power dynamic range requirement for these new channel bandwidths. It is proposed to reuse the requirement in TS 38.104 and define the following requirements for these new channel bandwidths for FR1-NTN Ku bands.

Table 2.1.3-1: Total power dynamic range

| BS channel | Total power dynamic range (dB) |  |
| --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS |
| 25 | 21.2 | 18.1 |
| 35 | 22.7 | 19.6 |
| 50 | 24.3 | 21.2 |
| 70 | N/A | 22.7 |
| 100 | N/A | 24.3 |

Proposal 4: It is proposed to define OTA total power dynamic range requirement as Table 2.1.3-1 for these new channel bandwidths for FR1-NTN Ku bands.

    - 2.1.4 OTA transmitter spurious emissions

For spurious frequency range, the upper limit is different between SAN type 1-O and 2-O in current spec. The upper limit of spurious frequency range for measurements is defined following the principle in ITU-R SM.329 as follows. For NTN Ku bands, we also propose to adhere the rule in ITU-R SM.329 to define the upper limit of spurious frequency range as 26 GHz due to Ku bands DL is 10.7-12.75 GHz.

Proposal 5: It is proposed to define the upper limit of spurious frequency range as 26 GHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

  - 2.2 SAN radiated Rx requirements

    - 2.2.1 OTA sensitivity

In terms of OTA sensitivity, it is only specified for SAN type 1-H and 1-O. The reason of no OTA sensitivity requirement for SAN type 2-O is that there are no equivalent conducted requirements and a certain minimum level of beam forming gain is required. There is only one sensitivity requirement for SAN type 2-O, which is REFSENS. For NTN Ku bands, since it only specifies radiated RF requirements, we propose to use the same approach as Ka band not to define OTA sensitivity.

Proposal 6: It is proposed not to specify OTA sensitivity for NTN Ku bands.

    - 2.2.2 OTA reference sensitivity level

For OTA REFSENS, it is channel BW and SCS specific. The requirement for SAN type 1-O is 2.4 dB lower than EISREFSENS in 38.104 with same reference measurement channel. For SAN type 2-O (also for Ka bands), it is a range and declared by the vendor, which is defined by the following equation with the assumption of the smallest supported bandwidth 50 MHz [3, 4].

We propose to use the same approach as Ka bands to define OTA REFSENS with the following assumptions:

PkT = -174 dBm/Hz.

IM = 2 dB.

BW for EISREFSENS_10M for Ka-band SAN is 52 RB, i.e. 52*12*15*1000Hz.

-1dB SNR can be reused for EIS for Ka-band SAN.

NF for GEO and LEO SAN: 3 dB.

G represents SAN Rx antenna gain and we assume the same value as Ka bands in coexistence study, but we only define Set-1. Since OTA REFSENS is range calculated based on antenna gain range, we can reuse SAN antenna gain for Ka bands: G = [50.5-58.5] dBi, [30.5-38.5] dBi, and [30.5-38.5] dBi for GEO, LEO-1200, and LEO-600.

Because the EISREFSENS_10M is the worst EIS in OTA REFSENS RoAoA, so 3dB off peak margin need to be considered. Hence, EISREFSENS_10M can be calculated by the following for GEO class SAN.

So we propose to define OTA REFSENS for both FR1-NTN Ku bands and FR2-NTN Ku bands as follows:

For GEO class SAN, EISREFSENS_10M is an integer value in the range [-178] to [-186] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_10M is an integer value in the range [-158] to [-166] dBm. The specific value is declared by the vendor.

Proposal 7: It is proposed to define OTA REFSENS for both FR1-NTN Ku bands and FR2-NTN Ku bands as follows:

For GEO class SAN, EISREFSENS_10M is an integer value in the range [-178] to [-186] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_10M is an integer value in the range [-158] to [-166] dBm. The specific value is declared by the vendor.

    - 2.2.3 OTA dynamic range

For OTA dynamic range, it is only specified for SAN type 1-O and no requirement for SAN type 2-O. Dynamic range is defined based on the result of system level simulation plus some margin, where the simulation was conducted to investigate IOT (interference over thermal) under in-channel interference. For Ka band, the requirement is no need to define based on the simulation result. Considering similar simulation assumptions, we also think no need to specify OTA dynamic range for Ku band.

Proposal 8: Not to specify OTA dynamic range for NTN Ku bands.

    - 2.2.4 OTA out-of-band blocking

For OTA out-of-band blocking, we need to specify ∆fOOB requirement for Ku band. According to TR 38.922, it is agreed to adopt ΔfOOB = 100 MHz for AAS BS in 14800 - 15350 MHz frequency range. We propose to reuse ΔfOOB = 100 MHz for NTN Ku bands.

Proposal 9: It is proposed to reuse ΔfOOB = 100 MHz for NTN Ku bands.

    - 2.2.5 OTA in-channel selectivity

For ICS, we also propose to define the common requirement for FR1-NTN Ku bands and FR2-NTN Ku bands. The requirement is channel bandwidth specific. Hence, which channel bandwidth as a basis to define the requirement and whether the requirement for SAN type 2-O in current spec can be reused need further discussion.

Proposal 10: For ICS, it is proposed to define the common requirement for FR1-NTN Ku bands and FR2-NTN Ku bands.

# 3 Conclusion

In this contribution, we share some views on SAN RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: It is proposed to define the tolerance as ± 3.4 dB for the radiated transmit power for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 2: It is proposed to define the tolerance as ± 3 dB for OTA SAN output power for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 3: Not to specify OTA RE power control dynamic range for NTN Ku bands.

Proposal 4: It is proposed to define OTA total power dynamic range requirement as Table 2.1.3-1 for these new channel bandwidths for FR1-NTN Ku bands.

Table 2.1.3-1: Total power dynamic range

| BS channel | Total power dynamic range (dB) |  |
| --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS |
| 25 | 21.2 | 18.1 |
| 35 | 22.7 | 19.6 |
| 50 | 24.3 | 21.2 |
| 70 | N/A | 22.7 |
| 100 | N/A | 24.3 |

Proposal 5: It is proposed to define the upper limit of spurious frequency range as 26 GHz for both FR1-NTN Ku bands and FR2-NTN Ku bands.

Proposal 6: It is proposed not to specify OTA sensitivity for NTN Ku bands.

Proposal 7: It is proposed to define OTA REFSENS for both FR1-NTN Ku bands and FR2-NTN Ku bands as follows:

For GEO class SAN, EISREFSENS_10M is an integer value in the range [-178] to [-186] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_10M is an integer value in the range [-158] to [-166] dBm. The specific value is declared by the vendor.

Proposal 8: Not to specify OTA dynamic range for NTN Ku bands.

Proposal 9: It is proposed to reuse ΔfOOB = 100 MHz for NTN Ku bands.

Proposal 10: For ICS, it is proposed to define the common requirement for FR1-NTN Ku bands and FR2-NTN Ku bands.

# 4 Reference

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2502288, Way Forward for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

- R4-2313996, WF on SAN RF, Ericsson, RAN4#108

- R4-2309770, WF on above 10 GHz SAN RF requirements, CATT, RAN4#107