# 3GPP TSG-RAN WG4 Meeting #114 bis                              R4-2503792

# Wuhan, China, 7th – 11st April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on core parts on LP-WUS

Agenda item:	7.26.4.2

Document for:	Approval

Introduction

In RAN #103 meeting the latest WID has been agreed [1] on Low-power wake-up signal and it is the first meeting for RAN4 to discuss the core part in this meeting. The WID can be seen as below:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this paper we will continue to discuss the related issues and provide our views.

Discussion

General aspects

In previous meetings, the basic scenarios were discussed with a few agreements. In this section, we mainly discuss the LP-WUS monitoring and the MR/LR measurements based on RAN1 and RAN2’s agreements.

Whether to keep Case#2

In previous meetings, RAN4 has already agreed the case#1 (MR Fully offload to LR, only LR) without consideration of configured high priority frequency, that is, only LR performs the serving cell measurement and MR is OFF. In previous meeting, we also agreed that MR and LR are in ON state (MR partially offload to LR, MR+LR).

For case#1, the measurement only performed by LR, that is, the LR measurement result will decide the MR is ON or OFF.

For case#3, MR and LR both perform the serving cell measurement, only MR performs neighbour cell measurement.

There are still case#2 and case#4 (measurements with high priority frequency) are not discussed and we will firstly analyze the remaining scenarios and then specify the RRM requirements for all cases.

Firstly, we shall focus on the feasibility of Case#2 and the following figure can be seen:

Figure 1 Different cases

Based on the above figure, we find that the MR and LR are ON and both of them perform the serving cell measurement for Case#2. However, the main difference between Case#2 and Case#3 is that there is no neighbour cell measurement for Case#2. In general, Case#2 and Case#3 depends on the serving cell measurement results. If the service quality of the current cell is good enough, the UE will not do the neighbour cell measurement. However, if measurement result is below the threshold which can enter into Case#1 in figure 1, UE is still in MR+LR state (Case#2). If measurement result is below the legacy neighbour cell measurement triggering threshold, the UE shall perform the neighbour cell measurement (Case#3). We deem that that is not a corner case and Case#2 may be needed. However, we shall have a comprehensive understanding if we decide to study Case#2. To my understanding, whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

We know that if UE stays in MR+LR state, the scaling factor for MR measurement time period is needed. The MR measurement is more infrequent than LR measurement and  MR measurement will cost more power. The periodicity of LR is under discussion in RAN1 which maybe 160ms, 320ms etc, that is, LR will measure more samples than MR, the measurement results may not necessarily be inaccurate. If the accuracy of LR measurement is the key concern, we only need to consider how to keep similar accuracy between LR and MR, such as introduce the offset for LR measurement threshold instead of studying another case. Thus, the Case#2 is not necessary to consider.

Observation 1: Whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

Proposal 1: Case#2 is not necessary to consider.

Threshold for switch between Case#1,Case#3 and Case#5

For entry and exit conditions based on LR and MR measurement, RAN2 has already reached the following conclusion in previous meetings:

127 meeting: For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. (optional 就是网络配就考虑，不配置就不考虑) The exit condition is based on the LR measurement results. 127bis meeting: If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

For scenario 1, there are two cases which from Case#5 to Case#3 or Case#1 (5-->3 or 5-->1) and from Case#1 to Case#3 or Case#5 (1-->3 or 1-->5). For the first case, only MR measurement in Case#5 and if MR measurements result can be satisfied, UE can enter into Case#3. For the second case, only LR performs serving cell measurement, and the LR based measurement can be the only criteria to enter into Case#3 or Case#5 which is the exit condition agreed in RAN2, and the threshold can be the legacy neighbour cell triggering since the neighbour cell measurement would be triggered.

For scenario 2, Case#3 shall enter into Case#5 or Case#1 (3-->5 and 3-->1). MR and LR perform the serving cell measurement together in Case#3. If network configures two different thresholds which are for MR based measurement and LR based measurement, the two measurements can compare to the different thresholds. However, RAN2 agreed that at least MR measurement shall be considered, and LR could also be considered. To my understanding, LR measurement based switching is an optional feature and MR measurement based switching is a mandatory feature. If network configures the threshold for LR based measurement, one issue will be raised. Whether UE shall satisfy the both thresholds like LP-WUS monitoring or not. This issue shall be clarified by RAN2 and it is RAN2’s decision.

- We deem that the threshold between Case#3 and Case#1 is the whether neighbour cell measurement is triggered. Whatever from Case#3 to Case#1 or from Case#1 to Case#3, the LR is always ON and the LR measurement result is also key point to decide whether to switch or not refer to RAN2. However, in current spec, the conditions for neighbour cell measurement is the RSRP of current serving cell which performed by MR measurement. RAN1 has agreed introduced the LP-RSRP and LP-RSRQ in previous meetings, thus, the new threshold for LP-RSRP and LP-RSRQ could be considered. Also, some companies proposed an offset for LR measurement threshold, that is, we could continue to use the existing threshold with an offset for LP-RSRP and LP-RSRQ. For other switching cases, RAN4 shall wait for RAN2’s agreements and follow RAN2’s conclusion.

Observation 2:

- For scenario 1, there are two cases which from Case#5 to Case#3 and from Case#1 to Case#3.

- For scenario 2, Case#3 shall enter into Case#5 or Case#1.

Observation 3: Whatever from Case#3 to Case#1 or from Case#1 to Case#3, the LR is always ON and the LR measurement result is also key point to decide whether to switch or not refer to RAN2. However, in current spec, the conditions for neighbour cell measurement is the RSRP of current serving cell which performed by MR measurement.

Observation 4: LR measurement based switching is an optional feature and MR measurement based switching is a mandatory feature.

Proposal 2: Use existing thresholds for SS-RSRP and SS-RSRQ from Case#3 to Case#1, and define new thresholds for LP-RSRP and LP-RSRQ.

Proposal 3: Add an offset for existing threshold which is compared to MR based measurement for LP-RSRP and LP-RSRQ.

Proposal 4: If network configures the threshold for LR based measurement, one issue will be raised. Whether UE shall satisfy the both thresholds like LP-WUS monitoring or not is based on RAN2’s decision.

LP-WUR status at legacy case

In previous meetings, we do not discuss the UE behaviour at legacy case (Case#5), the following way forward can be seen as below:

- Proposals - P1: LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation. - P1-1: No LP-WUR measurement and evaluation requirement is applicable when LR is “on” at the legacy state; - Note: in detail, no requirement is applicable on LR for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3). - P2: After MR measurement result is higher than MR threshold, UE turn on its LR. The requirement for LR measurement at legacy case should also be defined. - P3: LP-WUR status outside of the LP-WUS monitoring does not have impact to legacy procedures

There is a common understanding that the partially offloading and fully offloading measurement will be performed when LR is ON. LR only performs the serving cell measurement. Regarding this issue, we would like to split two parts:

- Whether the LR is ON in legacy case.

- If LR is ON, whether the RRM requirements will be defined.

- For the first, we have reached the agreement as below:

Agreement: At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

- For the second point, RAN2 has the agreement as below:

- NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

- From our perspective, LR only performs serving cell measurement in Case#3 and Case#1. If LR is ON in legacy case, there is no measurement behaviour, only MR performs L3 measurement. Thus, no requirement is applicable when LR is at legacy case. However, RAN2’s agreement shows that NW configures thresholds for both MR and LR measurement, then the LP-WUS monitoring entry condition is met when all the measured results are above the configured thresholds. It means if LR is ON, then the LR would perform serving cell measurement. We know that the channel condition shall be guaranteed if LR is ON and performs serving cell measurement. However, if LR performs serving cell measurement at legacy case, how to guarantee the channel condition in legacy case and how to ensure the measurement accuracy by LR constitute the main concern, that is, we should not consider this case and do not define any RRM requirements.

- Observation 5: The channel condition shall be guaranteed if LR is ON and performs serving cell measurement.

- Observation 6: If LR performs serving cell measurement at legacy case, how to guarantee the channel condition in legacy case and how to ensure the measurement accuracy by LR constitute the main concern.

Proposal 5: RAN4 shall not define the RRM requirements if LR is ON at legacy case.

LP-WUR operating carrier frequency

In RAN #106 meeting, the following agreements could be seen:

Agreement: - Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band) - Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria) - Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation)

For this issue, we do not recognize the RAN4 RRM impact from the LS from RAN2 [3]. To our understanding, this issue shall be closed.

Proposal 6: The issue for LP-WUR operating carrier frequency shall be closed since there is no RRM impact based on RAN2’s LS.

Higher priority frequency layer

When UE stays in center area of the current cell and the service of the cell is good enough, the UE will not perform neighbour cell measurement without configured high priority frequency. However, the UE shall search for inter-frequency and inter-RAT layers of high priority if high priority frequency information is provided by the serving cell based on TS 38.133:

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

- Observation 5: The different cases can be seen as following table with high priority frequency:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case#1 | OFF | OFF | ON |
| Case#2 (to be confirmed) | ON, with measurement relaxation | OFF | ON |
| Case#3 | ON, with measurement relaxation | ON, with measurement relaxation | ON |
| Case#4 | OFF | ON, with measurement relaxation | ON |
| Case#5 | ON | ON | OFF |

- For Case#4:

UE is in fully LR state and the MR shall be OFF, that is , UE is in the center area of the current cell and the service is good enough. In this case, serving cell has configured the high priority frequency information and UE shall search for high priority frequency layers and perform the neighbour cell measurement. Thus, MR shall be ON from OFF since UE search for the high priority frequency layer at Thigher_priority_search. But only LR performs serving cell measurement. As mentioned before, RAN2 is discussing the neighbour cell measurement relaxation condition and if the condition is the same as legacy, we deem that the legacy relaxed requirement can be as baseline and RAN4 shall only discuss the relaxed scaling factor.

- Observation 7: MR will perform inter-frequency measurement with high priority frequency and inter-RAT measurement with high priority frequency if high priority frequency information is provided by the serving cell even if the service of current cell is good enough.

- Proposal 7: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor.

Detailed LP-WUR requirements at RRC IDLE/INACTIVE state

Accuracy requirements

In previous meetings, we have proposed several times that no dedicated accuracy requirements is defined in performance section for LP-WUR based RRM measurement in Idle/Inactive states since the accuracy is implicitly defined as margin as below:

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that: when rangeToBestCell is not configured: - the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

Bsed on the existing spec, we deem that no dedicated accuracy requirement is defined in the performance section since the existing accuracy requirements are included in the core parts instead of the performance section. Although LR is an introduced new receiver, the legacy methodology could be referred. However, the margin shall be considered because of introduced LR new receiver compared to legacy MR receiver.

Proposal 8: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Periodicity for LP-WUR RRM requirements

In previous meetings, RAN4 has already discussed the periodicity for LP-WUR RRM requirements. There are still some detailed issues shall be discussed and the previous agreement as follows:

RAN4 113 agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity. RAN4 114 way forward: - Proposals - P1: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. - P2: For periodicity for SSB based LP-WUR measurement delay requirements, when LP-SS is configured, the periodicity for SSB based LP-WUR measurement delay requirements can be max { LO periodicity, LP-SS periodicity}. - P3: For periodicity for SSB based LP-WUR measurement delay requirements, postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity. - P4: For periodicity for SSB based LP-WUR measurement delay requirements, no requirement is defined if LO periodicity is smaller than LP-SS periodicity.

We find whether the LP-SS is configured or not, the periodicity for SSB based OFDM LP-WUR measurement delay requirements is based on LO periodicity. However, one remaining issue is if LO periodicity is smaller than LP-SS periodicity, whether the LO periodicity could be used if LP-SS is configured. To our understanding, we deem that only LO periodicity shall be used even if LO periodicity is smaller than LP-SS periodicity. We know that this case mainly consider the power saving gain. LO means LP-WUS occasion, each LO has one or more LP-WUS monitoring occasions. UEs will always wake up in MO to monitor the LP-WUS, thus, the power consumption will not be increased too much if UEs also perform the serving cell measurement by the way in each LO. Consequently, we think we shall not consider the case which LO periodicity is smaller than LP-SS periodicity.

Proposal 9: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

We think the upper bound is need on SSB-based LP-WUR measurement periodicity. Actually, defining the upper bound is beneficial to the service quality for LR measurement. It is a trade-off between power saving and how fast UE can exit LR when the channel condition is worse. To our understanding, in cases where channel condition drastically deteriorates, if there is a rapid exit from the current LP-WUR mode coupled with corresponding adjustments, UEs will be able to respond more effectively to unexpected situations. For instance, during mobility, when a UE transitions from a region with high-quality links to an area with weaker links, it needs to promptly detect this change and implement appropriate measures to maintain communication quality. If the communication quality can not be guaranteed, the power saving gain would not be considered, and we do not think if the upper bound is settled, the power consumption will be largely increased.

Observation 8: If the communication quality can not be guaranteed, the power saving gain would not be considered, and we do not think if the upper bound is settled, the power consumption will be largely increased.

Proposal 10: The upper bound is needed on SSB-based LP-WUR measurement periodicity if LP-SS is not configured.

MR wake-up delay

In legacy, when DRX is configured and there is no traffic transmission, the MR (main receiver) enters a sleep state, only waking up at corresponding moments (DRX ON) to perform paging monitoring. If the specific conditions for entering LR coverage are satisfied, LR allows the UE MR to enter a deep sleep state prior to monitoring the LP-WUS. When the UE receives SIB message, the UE initiates monitoring of LP-WUS, subsequently triggering the awakening of the UE MR to monitor PDCCH, this process aims to further mitigate power consumption:

Figure 3 UE behavior upon detecting LP-WUS with paging indication in IDLE/INACTIVE state

Based on the above procedure, it is crucial that UE shall not miss paging when entry/exit conditions for LP-WUS monitoring are met. RAN4 shall study and define the related limitation on timing from UE receiving SIB message to waking up MR to monitor legacy PO when the entry condition is met. It also can be seen that the time window LP-WUS occasions (LOs) are defined for LP-WUS monitoring, each LO has one or more LP-WUS occasions.

From my perspective, the following shall be considered so that the UE will not miss the paging:

Figure 4 Time delay for paging monitoring

MR wake-up time: UE detects the LP-WUS and decodes it, then LP-WUS wakes up MR to monitor the legacy PO.

When the entry condition for LP-WUS monitoring is met, the time delay shall contain MR wake-up time TMR_wake-up . This time will decide whether LP-WUS can timely wake up MR and ensure that it does not miss the paging monitoring.

There are two scenarios shall be considered for defining the related wake-up delay:

- Scenario 1: Deep sleep.

- Scenario 2: Ultra deep sleep.

For Scenario 1, to our understanding, ramp up time, AGC, time sync shall be contained in wake up delay during this procedure. There are two kinds of LR shall be also considered when defining the wake up delay as follows:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

Ramp-up time (around 20ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (20ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- The measurement is performed by LR before MR awake and LR has the rough timing sync, which can inform MR when LR triggers MR awake. Thus, only 1 SSB could be used to adjust timing and frequency offset for OFDM based receiver, 1~2 SSB could be used to adjust timing and frequency offset for OOK based receiver, since OFDM based receiver has more accurate timing sync performance than OOK based LR. Compared to ultra deep sleep, very coarse synchronization information are involved in deep sleep state, where “very coarse” synchronization information refers to a clock possibly running at low power, so we think maybe 1~2 SSB for timing sync for OOK based LR in deep sleep state.

- For Scenario 2, the wake up delay can be seen as below:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- Based on the above analysis, we deem that 3SSBs are enough for defining wake up delay, the main difference in Scenario 1 and Scenario 2 is only the ramp up time. Consequently, we think only 3SSBs could be used for AGC, timing sync and others.

- Observation 9:

- For deep sleep state:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (around 20ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (20ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- For ultra deep sleep state:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- Observation 10: Compared to ultra deep sleep, very coarse synchronization information are involved in deep sleep state, where “very coarse” synchronization information refers to a clock possibly running at low power, maybe 1~2 SSB for timing sync for OOK based LR in deep sleep state.

- Proposal 11: 3SSBs could be used for AGC, timing sync and others, and 5SSBs are not needed.

LP-WUR at connected mode

We mainly focus on the measurement requirements in the above. For the connected mode, in the latest WID, we can see that no RAN4’s involvement is required:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

It is obviously the CONNECTED state is not in the scope of the WID.

Proposal 12: RAN4 shall not define the measurement requirements at CONNECTED state.

Conclusion

In this paper we provided our views on LPWUS.

Observation 1: Whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

Proposal 1: Case#2 is not necessary to consider.

Observation 2:

- For scenario 1, there are two cases which from Case#5 to Case#3 and from Case#1 to Case#3.

- For scenario 2, Case#3 shall enter into Case#5 or Case#1.

Observation 3: Whatever from Case#3 to Case#1 or from Case#1 to Case#3, the LR is always ON and the LR measurement result is also key point to decide whether to switch or not refer to RAN2. However, in current spec, the conditions for neighbour cell measurement is the RSRP of current serving cell which performed by MR measurement.

Observation 4: LR measurement based switching is an optional feature and MR measurement based switching is a mandatory feature.

Proposal 2: Use existing thresholds for SS-RSRP and SS-RSRQ from Case#3 to Case#1, and define new thresholds for LP-RSRP and LP-RSRQ.

Proposal 3: Add an offset for existing threshold which is compared to MR based measurement for LP-RSRP and LP-RSRQ.

Proposal 4: If network configures the threshold for LR based measurement, one issue will be raised. Whether UE shall satisfy the both thresholds like LP-WUS monitoring or not is based on RAN2’s decision.

- Observation 5: The channel condition shall be guaranteed if LR is ON and performs serving cell measurement.

- Observation 6: If LR performs serving cell measurement at legacy case, how to guarantee the channel condition in legacy case and how to ensure the measurement accuracy by LR constitute the main concern.

Proposal 5: RAN4 shall not define the RRM requirements if LR is ON at legacy case.

Proposal 6: Postpone this issue until RAN2 has the final decision.

- Observation 7: MR will perform inter-frequency measurement with high priority frequency and inter-RAT measurement with high priority frequency if high priority frequency information is provided by the serving cell even if the service of current cell is good enough.

- Proposal 7: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor.

Proposal 8: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 9: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Observation 8: If the communication quality can not be guaranteed, the power saving gain would not be considered, and we do not think if the upper bound is settled, the power consumption will be largely increased.

Proposal 10: The upper bound is needed on SSB-based LP-WUR measurement periodicity if LP-SS is not configured.

- Observation 9:

- For deep sleep state:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (around 20ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (20ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- For ultra deep sleep state:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- Observation 10: Compared to ultra deep sleep, very coarse synchronization information are involved in deep sleep state, where “very coarse” synchronization information refers to a clock possibly running at low power, maybe 1~2 SSB for timing sync for OOK based LR in deep sleep state.

- Proposal 11: 3SSBs could be used for AGC, timing sync and others, and 5SSBs are not needed.

Proposal 12: RAN4 shall not define the measurement requirements at CONNECTED state.

References

[1] RP-221018, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] TR 38.869

[3] R4-2503116