3GPP TSG-RAN WG4 Meeting #114bis 	R4-2503816

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Discussion on the RF requirement for CW

Agenda Item:	7.24.2.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on the boundary of emission requirement and transmit OFF power for the CW node.

- Discussion

- Phase noise

In R19, the CW is outside topology which means the CW signal is non-correlated with the LO used in D2R down-conversion in AIoT BS, and it will harder to cancel its impact and the SNR degradation need to be considered. In TR 38.769, the coverage design target for device in D1T1 is 15m, and we can use this value as the baseline to take a look on the maximum power difference between CW and D2R.

The worst case for the CW interference should be that the Tag is placed at 15m distance away from AIoT BS and the CW node is co-located with AIoT BS. The link budget is listed in Table I

Table I link budget for the power difference between CW and D2R

| Center frequency (GHz) | 0.9 |
| --- | --- |
| CW Tx power(dBm) | 33 |
| Pathloss (15m *2) (dB) | 103.6625 |
| backscatter loss | 6 |
| D2R power @AIoT BS (dBm) | -76.6625 |
| CW cancellation (analog domain) (dB) | 55 |
| CW interference @AIoT BS (dBm) | -27 |
| Power difference (dB) | 54.66246 |

Since the phase noise is overlapped with D2R in frequency domain, so only the analog domain cancellation is considered, and we assume 55 dB can be achieved in RFIC. However, even though the cancellation capability is counted, the CW power still could be 55 dB higher than the D2R.

Observation 1: Considering the worst case, the CW power could be 55 dB higher than D2R.

Another aspect that related to the phase noise is the small frequency shift. Since the phase noise fades away when the frequency is far from the center frequency, so for the same D2R bandwidth, when the D2R is close to CW, the wanted signal suffers from higher interference. Take 15kHz D2R transmission as an example, the Figure I show the impact of phase noise when the D2R in different BLF (backscatter link frequency). The phase noise model in TR 38.803 Example 2 with the scaled center frequency is used.

Figure I Impact of phase noise for 15kHz D2R (SFO is disabled)

Under 25dB higher CW interference, the significant performance degradation is already observed for the small BLF, so the phase noise is not only needed to be considered in D2R SNR evaluation but also a dedicated requirement is needed which can provide guidance on the minimum small frequency shift.

Observation 2: The phase noise has significant impact on the D2R reception performance, and the phase noise should be restricted in CW requirement and also reflect in D2R SNR evaluation.

It is will be hard to define a requirement for phase noise curve. For simplicity, the integrated phase noise for a certain frequency range can be considered as the requirement. As for the boundary of phase noise requirement, based on the simulation above, the D2R with minimum small frequency shift suffer from the highest interference and the requirement can be applied to here, as shown in Figure II.

Figure II Illustration of the phase noise requirement boundary

Proposal 1: Define the integrated phase noise requirement between CW and minimum small frequency shift ± D2R transmission bandwidth/2, and the principle is to make sure the D2R SNR degradation aligns with the dense target in REFSENS formula.

- Unwanted emission

Since there is no co-existence issue for CW, the ACLR is not needed and only the SEM needs to be defined. Recall the discussion in LTE/NR, the SEM definition is related to co-existence, regulation and also need consider the spectrum of modulated signal itself, and all these consideration intents to minimize the impact to other radio system.

Observation 3: Traditionally, the SEM is defined based on co-existence, regulation, and the spectrum of signal, and the intention is to minimize the impact to other radio system.

The only impact from CW we need to consider is the D2R reception performance, and one possible way to define a meaningful SEM is to differentiate the D2R degradation due the CW interference.  The range that SEM applied can be from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth.

Figure III Illustration of the CW SEM boundary

Proposal 3: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth, and principle could be different allowed D2R performance degradation for each dash line of the mask.

- Spurious emission

In the last meeting, we agreed that NR spurious emission is used as starting point. The NR spurious emission comes from ITU-R and it can be applied out of D2R Channel bandwidth.

Figure IV Illustration of the phase noise requirement boundary

Proposal 4: The spurious emission of CW starts from the boundary of D2R channel bandwidth.

- Transmit OFF power

In the last meeting, the transmit OFF power is still pending. Since the D2R and CW are always in same spectrum, the transmit OFF will influence the noise floor in the channel especially when many CW nodes are deployed in the same room. In addition, considering the CW need to be muted when R2D is transmitting, so the transient period is also needed. As for the requirement, the AIoT BS can be the baseline.

Proposal 5: Same OFF power and transient period of AIoT BS can be applied to CW node.

- Conclusion

In this contribution, we provide our preliminary views on CW RF requirement.

Observation 1: Considering the worst case, the CW power could be 55 dB higher than D2R.

Observation 2: The phase noise has significant impact on the D2R reception performance, and the phase noise should be restricted in CW requirement and also reflect in D2R SNR evaluation.

Proposal 1: Define the integrated phase noise requirement between CW and minimum small frequency shift ± D2R transmission bandwidth/2, and the principle is to make sure the D2R SNR degradation aligns with the dense target in REFSENS formula.

Observation 3: Traditionally, the SEM is defined based on co-existence, regulation, and the spectrum of signal, and the intention is to minimize the impact to other radio system.

Proposal 3: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth, and principle could be different allowed D2R performance degradation for each dash line of the mask.

Proposal 4: The spurious emission of CW starts from the boundary of D2R channel bandwidth.

Proposal 5: Same OFF power and transient period of AIoT BS can be applied to CW node.

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei)

- 3GPP TR 38.769

- ETSI EN 302 208