3GPP TSG-RAN4 Meeting #114bis	R4-2503837

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.1.1.2.1

Source:	vivo

Title:	Discussion on UE RF power enhancement for NR single carrier

Document for:	Discussion

- Introduction

In RAN#114 meeting, the WF[1] has been agreed, and the remaining issues are as follows:

### Sub-topic 1-1: Applicable requirements #### Issue 1-1-1: Boundary to apply SE - Option 1: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW - Option 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz - WF: option 1, to meet the ITU recommendation #### Issue 1-1-2: Integral region basis for OOBE - Option 1: The OOBE requirements are based on UE CBW - Option 2: The OOBE requirements are based on extended UE CBW - WF: option 1 #### Issue 1-1-3: A-MPR handling - WF: FFS, considering that additional emission in most cases are applied on one side of the UE CBW #### Issue 1-1-4: How to derive the extended IBE - WF - FFS Extended IBE can be used in the original GB of UE CBW - FFS the extended IBE in the GB of extended CBW #### Issue 1-1-5: Modulation order for the extended IBE - WF: FFS whether the extended IBE with high order EVM could have impact on MPR for new inner RB allocation ### Sub-topic 1-2: Approaches to enable MPR reduction for both scenario 1 and scenario2 Scenarios discussed in previous RAN4 meetings: - Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator) - Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators) - Scenario 2: Narrower UE channel BW within wider BS bandwidth #### Issue 1-2-1: Width/RB numbers of extended CBW at each side of UE CBW - WF: FFS in next meeting - The max size of the extension at each side of UE CBW - Whether extended CBW is fixed or could be flexible ### Sub-topic 1-3: Signaling #### Issue 1-3-1: NW signaling - WF - FFS whether NW signalling need to consider different requirements or just a function enabler without considering specific requirements #### Issue 1-3-2: UE capability - WF - MPR reduction with extended CBW is an optional feature with UE capability - FFS details of UE capability

- Discussion

## 2.1 Extension scheme

### 2.1.1 Extension format & ratio

In total, the evaluation of OOBE requirements should be based on the extended UE CBW, which also matches the WF[1] in the 114 meeting. However, it has been agreed that IBE should be used in the extension part in the WF[2] in 113 meeting, the extended RB numbers should also be clear to the UE to identify the location of IBE assessment as IBE is evaluated as per RB granularity.

Observation 1: Both the extension CBW and the extension RB numbers should be clear to the UE to identify the applying range of OOBE and IBE requirements.

In short, simply extending by 2*the original UE CBW (i.e., extending at 0.5 times the UE CBW on each side) is sufficient to convert all the outer allocations to inner without wasting more extended bandwidth. And the extension could be asymmetrical, the extension condition of any side is that the gap between the original UE CBW and the RX RF CBW is at least 2 times the UE CBW, so the symmetrical extension should be based on the premise that the gap is wide enough on both sides.

Proposal 1: The extension ratio should be fixed to 0.5 times on one side.

- Symmetrical extension is 0.5 * UE CBW & 0.5*NRB on each side.

- Asymmetric extension is 0.5 * UE CBW & 0.5* NRB on one side.

### 2.2.2 GB position & starting point of the extended RB & IBE applying scheme

In total, there are several different ways to perform the extension. The extended RBs' starting point, the GBs' position, and the application scheme of the IBE in the extension part are mutually influenced, and they are divided into two issues as follows:

#### Issue 1: How to arrange the position of the original UE’s GB and extended RBs.

Option1. The extended RBs are allocated from the GB of the original UE, so IBE should be applied in the GB.

Fig1. Option 1: allocating the extended RB from the GB

Option2. The extended RBs are allocated from the edge of the UE CBW, so the GB of the original UE would be reserved without any emission evaluation.

Fig2. Option 2: allocating the extended RB from the UE CBW’s edge

Observation 2: There are two options for the arrangement of the position of the original UE’s GB and extended RBs:

- Option 1: Allocate the extended RBs from the GB of the original UE, the IBE should be applied in the GB.

- Option 2: Allocate the extended RBs from the edge of the UE CBW, reserve the GB of the original UE without any emission evaluation.

#### Issue 2: How to evaluate IBE in the extension RBs.

There are also two mainstream schemes now:

Option 1. Individual evaluation of the extended RBs: Horizontal line with the value equal to the outermost non-allocated RB of UE CBW.

Option 2. Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs.

Apparently, option 1 has no effect on the IBE inside the original UE CBW. However, for option 2, as NRB’ changes with the extended RB, it may also change the IBE inside UE, as the equation is as follows:

IBE_general = max(PRB - 30 dB, )

It is well known that, generally the first several non-allocated RBs would follow the IBE requirement of the equation（1）, and the stepped line is visible in these non-allocated RBs:

20*log10EVM -3-5*∆RB-1LCRB        (1)

This means that for the non-allocated RBs closer to the allocated ones, they are not generally affected by equation (2) below. However, for more distant non-allocated RBs, the value of (1) gradually decreases with ΔRB increasing, causing the step-like curve to decline. As a result, the IBE may eventually need to follow the value given by (2).

-25-10lg(NRB/LCRB)                (2)

One of the prerequisites for this to happen is that (1) < (2), i.e.,

-25-10lgNRBLCRB> 20*log10EVM -3-5*∆RB-1LCRB     (3)

To model the case where (1) takes extreme small values (The stepped line declines to a relative low level and gives (2) the chance to dominant), we could place all the allocated RBs on one side of the UE, at which point the value of ∆RB at the outermost non-allocated RBs takes the value of ∆RB≈NRB-LCRB, and thus inequality (3) changes to:

-25-10lgNRBLCRB> 20*log10EVM -3-5*NRB-LCRB-1LCRB   (4)

By derivation, if 1/LCRB is ignored, then for QPSK (EVM = 0.175), the inequality is satisfied when the following conditions are met: NRB/LCRB⪆2.5, which may exist for outer allocations where the RB allocation is close to one side of the UE. Therefore, the change in NRB may affect the IBE within the UE CBW.

Observation 3: There are two options on how to evaluate IBE in the extension RBs:

- Option 1: Individual evaluation of the extended RBs: Horizontal line with the value equal to the outermost non-allocated RB of UE CBW.

- Option 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB' = NRB + extended RBs.

Observation 4: If option2 is selected, i.e. NRB' = NRB + extended RBs for combined evaluation of extended RBs and non-allocated RBs within the UE CBW, the IBE within the UE CBW may be tightened.

To summarize, there are four distinct cases derived from the interplay of Issue 1 and Issue 2. Further simulation studies are performed on a subset of them.

Table 1. Combination cases of Issue 1&2

| Issue1 Issue 2 | Option1. Allocate the extended RBs from the GB of the original UE | Option2. Allocate the extended RBs from the edge of the UE CBW |
| --- | --- | --- |
| Option1. Individual evaluation of the extended RBs & Horizontal line | Case1 | Case2 |
| Option2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line | Case3 | Case4 |

Firstly, it could be predicted that case 2 is the most relaxed scheme and lots of analysis has been done in #114 meeting [3]. Based on this scheme, it could be ensured that the MPR for the outer allocation could be reduced to inner level.

Observation 5: If allocating the extended RBs from the edge of the extended CBW and evaluating the extension RBs by the IBE of the outmost non-allocated RB in the UE CBW(case 2), then the MPR for the outer region could be reduced to the inner level.

For case 1, it is more stringent than case 2 because the GB of the UE CBW should be evaluated by IBE of the outmost non-allocated RB, but for case 2, no evaluation would be done in the GB instead.

For case3, it is the most stringent one in the four cases as not only the GB needs evaluation but also NRB’ would replace NRB and the stepped line would keep decreasing at least in the first several extended RBs.

For case4, it is more relaxed than case 3 because the GB is reserved. But it is more stringent than case 2 for the decreased stepped line in the extended RBs.

The simulation results for MPR of cases 1 and 3 are as follows. It could be seen from the results that for case 1, even the IBE in the extended RBs are all relaxed to the value of the outmost non-allocated RBs, when the UE CBW is rather wide then the MPR of the outer config with full RB allocation could not reduce to inner’s level which is 0 under the corresponding conditions. The limiting factor is IBE in the GB range.

A similar situation occurs in case 3, and in order to solve the problem of excess emission in the GB, there are two different ways:

Way 1: Skip the IBE evaluation for the extended RBs allocated in the GB.

- It is necessary to identify the number of RBs in the GB and perform skipping, so it is relatively more troublesome because the number of RBs in the GB is usually not an integer. Rounding up to ensure that the entire frequency band of the GB is skipped is a safer approach.

Way2: Relax EVM in the IBE formula to that for BPSK for extended RBs.

- It is relatively simpler but would relax the IBE for the extended RBs not only in the GB but also in the other parts.

Table 2. MPR for case 1 and case 3

| DFT-S-OFDM, PC3, 15K SCS, QPSK, full allocation | Case1 | Case3 | Case3 with Way1, Skip the IBE evaluation for the extended RBs in the GB | Case3 with Way2, For the extended RBs, relax the EVM in IBE to that for BPSK |
| --- | --- | --- | --- | --- |
| UE CBW=15M | -0.3 | 0 | 0 | / |
| UE CBW=20M | 0 | 0 | 0 | / |
| UE CBW=40M | 0 | 0 | 0 | -0.3 |
| UE CBW=50M | 0.3 | 0.3 | -0.1 | -0.3 |

Both methods could relax IBE in the GB somewhat to help bring the MPR down to the inner level, which could be seen in the simulation results in Table 2.

Similarly, for Case 1, if following the same two relaxing ways as Case 3, the IBE-limit problem would not exist anymore, the corresponding simulation results are omitted here.

Observation 6: If the extended RBs are allocated from the GB of the original UE(option1 in Issue1), then regardless of the IBE in the extended RBs takes a stepped line or a horizontal line(case 1 or case3), the MPR of the outer config with full RB allocation may cannot reduce to inner’s level, which is limited by the IBE in the GB of the original UE.

Observation 7: For the cases that the extended RBs occupy the GB of the original UE(case 1 and case 3), if skipping the IBE evaluation or relaxing EVM in the IBE formula to that for BPSK, the IBE inside the GB would be relaxed effectively.

Additionally, for Case 4, it essentially shifts the stepped IBE metrics within the GB to the range outside of the GB. As a result, it is more relaxed compared to Case 3 with Way 1, thus still ensuring the effectiveness of MPR reduction. However, it is a less intuitive scheme because it needs to identify the ∆RB value of the last non-allocated RB in the original UE CBW and +1 on top of that to assign it to the first RB in the extended CBW, and then compute the IBE accordingly, which is relatively cumbersome.

Proposal 2: If the extended RBs are allocated from the GB of the original UE (case1/3), then the IBE in the GB should be relaxed to ensure the MPR of outer allocation reduces to the inner level:

- Way 1: Skip the IBE evaluation for the extended RBs allocated in the GB.

- Way2: Relax EVM in the IBE formula to that for BPSK for extended RBs.

Proposal 3: For the GB position & starting point of the extended RB & IBE applying scheme, it is recommended to choose case 2, or case 1 with IBE relaxation on the GB of the original UE.

## 2.2. New inner region

As analyzed above, determining the extended scheme requires comprehensive consideration of factors such as the GB's position, the starting point of the extended RB configuration, and the IBE evaluation method within the extended RBs, making it difficult to converge. However, the final selected scheme should adhere to a fundamental principle: After RB and bandwidth extension, if the original UE's outer configuration becomes an inner configuration, the MPR must be able to fall back to the inner level.

The simplest verification method is to check under a full-bandwidth configuration, when both the total bandwidth and total RB number expand twice, whether the MPR can be reduced to the inner level. If not, for example, as seen in Case 1 and Case 3 of Table 2, where the MPR remains at 0.3 dB, then such an extension scheme is unreasonable. Moreover, this kind of scheme therefore cannot serve as a basis for redefining the inner and outer regions, as it violates the underlying principles of inner region division in the spec.

Observation 8: The reasonableness of the extension scheme and the accuracy of the new inner region definition must be based on one prerequisite: When the extended configuration becomes part of the inner region, the MPR should reduce to the inner level.

Based on the fixed extension ratio of 0.5 times and a reasonable extension scheme, when both sides are extended, then all the outer allocations including full RB allocation, could convert to inner and the MPR reduction could take effect no matter what the specific RB allocation is. However, for the asymmetric extension, the RB region definition of the original UE CBW should change, and the UE should judge its allocation according to the new definition. And based on the fixed extended ratio, the new RB region could be calculated directly but the UE’s RB allocation stays the same, so there is no need for the NW to indicate extension RB, or RB0shift & BS_NRB to the UE anymore.

Observation 9: When both sides are extended by 0.5* UE CBW, all the outer allocations including full RB allocation could convert to inner, and the MPR could always refer to the inner value regardless of the specific RB allocation.

Observation 10: Based on the fixed extended ratio, the new inner RB region of asymmetric extension could be calculated directly, but the UE’s RB allocation stays the same, so there is no need for the NW to indicate extension RB, or RB0shift & BS_NRB to the UE anymore.

When only one side is extended, the inner region should be redefined.

The following figure shows the range of RB allocation with maximum LCRB in the inner region. The original bandwidth of the UE is A and at maximum LCRB (0.5*NRB), the RB allocation range in the UE CBW should be 1/4A~3/4A. When the UE is extended by 0.5*UE CBW on the higher side, the bandwidth of the extended UE is 1.5A. At maximum LCRB (0.5*NRB_extended UE), the RB allocation range in the extended UE CBW should be 3/8A~9/8A. However, the RBs outside the original UE should not be scheduled so the furthest allocated RB should not exceed A.

According to this principle, assumes RBstart = X at maximum LCRB = A-X for the original UE with extension. For the lower side the parameter RBstart,Low should still follow the principle ≥ max(1, floor(LCRB /2)), and for RBstart,High there is no restriction other than ≤ NRB - LCRB. So A-X=2*X, deducing X= A/3. Therefore, LCRB for the UE with extension should be less than or equal to 2/3 * NRB.

Fig 3. The RB allocation range at maximum LCRB in the inner region, extension on the higher side

Similarly, when the extension is on the lower side, for the original UE with extension, the allowed RB allocation range in the new inner region is 0.5*A~X, X-0.5*A = 2* (1.5*A-X), deducing X= 7/6 *A. The LCRB for the UE with extension should also be less than or equal to 2/3 NRB, and for the higher side the parameter RBstart,High should still follow the principle ≤ NRB – RBStart,Low – LCRB, and for RBstart,Low there is no restriction.

Observation 11: For UEs that extend 0.5 times the original bandwidth on one side, LCRB should be less than or equal to 2/3 * NRB for the inner RB region.

Fig 4. The RB allocation range at maximum LCRB in the inner region, extension on the lower side

As the spec shows, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

RBStart,Low  = max(1, floor(LCRB /2))

RBStart,High = NRB – RBStart,Low – LCRB

If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be defined as:

RBStart,Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *NRB)

Similarly, if only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be defined as:

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *NRB)

Proposal 4:

- If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region.

- If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be redefined as:

RBStart_Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *NRB)

Fig 5. inner region of extended CBW, higher side extends by 0.5*UE CBW

- If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be redefined as:

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *NRB)

Fig 6. inner region of extended CBW, lower side extends by 0.5*UE CBW

## 2.3 Signaling mechanism

The new mechanism of the UE to reduce MPR should be discussed, and based on the fixed extension ratio, i.e., 0.5 * UE CBW on each side, the following could be considered.

First of all, it is up to the NW to judge the deployment environment, when the gap on one side of the UE CBW is at least greater than 0.5* UE CBW, the NW should send the signaling (e.g., ExtendLowerSideRel19, or ExtendHigherSideRel19) to the UE to indicate extension on the corresponding side. Then the UE would judge its allocation type based on the new inner region defined in Proposal 4, and the corresponding MPR would be used.

If the gap on both sides of the UE is at least greater than 0.5* UE CBW, the NW should send two signaling (e.g., ExtendLowerSideRel19 + ExtendHigherSideRel19) or one combined signaling (e.g., ExtendBothSidesRel19) to the UE to indicate that the extension is on both sides and the MPR of inner region could be used no matter what the RB allocation type is.

Proposal 5: Based on the fixed extension ratio, i.e., 0.5* UE CBW on each side, the following signaling mechanism could be considered:

- It is up to the NW to judge the gap width on both sides of the UE.

- When the gap on one side is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends the signaling (e.g., BWExtension -LowerSide-Rel19, or BWExtension-HigherSide-Rel19) to indicate the extension on the corresponding side.

When receiving the above extension signaling, the UE judges its allocation type based on the new RB region defined in Proposal 4.

- When the gap on both sides is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends two signaling (e.g., BWExtension -LowerSide-Rel19 + BWExtension-HigherSide-Rel19) or one combined signaling (e.g., BWExtension -BothSides-Rel19) to indicate the extension on both sides.

When receiving the above extension signaling, the MPR of inner region could be used regardless of the RB allocation type.

The above extension signaling is rather stable as it depends on the original UE CBW and the gap width, and will not change with the RB allocation state, so the signaling overhead is very low. At the same time, as the extension ratio is fixed it should be stipulated in the spec, and there should be no other UE capability other than support for the BW extension.

Observation 12: The extension signaling overhead is very low, as it only depends on the original UE CBW and the gap width and will not change with the RB allocation state.

Proposal 6: The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec, and there should be no other UE capability other than support for the BW extension.

- Conclusion

Observation 1: Both the extension CBW and the extension RB numbers should be clear to the UE to identify the applying range of OOBE and IBE requirements.

Proposal 1: The extension ratio should be fixed to 0.5 times on one side.

- Symmetrical extension is 0.5 * UE CBW & 0.5*NRB on each side.

- Asymmetric extension is 0.5 * UE CBW & 0.5* NRB on one side.

Observation 2: There are two options for the arrangement of the position of the original UE’s GB and extended RBs:

- Option 1: Allocate the extended RBs from the GB of the original UE, the IBE should be applied in the GB.

- Option 2: Allocate the extended RBs from the edge of the UE CBW, reserve the GB of the original UE without any emission evaluation.

Observation 3: There are two options on how to evaluate IBE in the extension RBs:

- Option 1: Individual evaluation of the extended RBs: Horizontal line with the value equal to the outermost non-allocated RB of UE CBW.

- Option 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB' = NRB + extended RBs.

Observation 4: If option 2 is selected, i.e., NRB' = NRB + extended RBs for combined evaluation of extended RBs and non-allocated RBs within the UE CBW, the IBE within the UE CBW may be tightened.

Observation 5: If allocating the extended RBs from the edge of the extended CBW and evaluating the extension RBs by the IBE of the outmost non-allocated RB in the UE CBW(case 2), then the MPR for the outer region could be reduced to the inner level.

Observation 6: If the extended RBs are allocated from the GB of the original UE(option1 in Issue1), then regardless of the IBE in the extended RBs takes a stepped line or a horizontal line(case 1 or case3), the MPR of the outer config with full RB allocation may cannot reduce to inner’s level, which is limited by the IBE in the GB of the original UE.

Observation 7: For the cases that the extended RBs occupy the GB of the original UE(case 1 and case 3), if skipping the IBE evaluation or relaxing EVM in the IBE formula to that for BPSK, the IBE inside the GB would be relaxed effectively.

Proposal 2: If the extended RBs are allocated from the GB of the original UE (case1/3), then the IBE in the GB should be relaxed to ensure the MPR of outer allocation reduces to the inner level:

- Way 1: Skip the IBE evaluation for the extended RBs allocated in the GB.

- Way2: Relax EVM in the IBE formula to that for BPSK for extended RBs.

Proposal 3: For the GB position & starting point of the extended RB & IBE applying scheme, it is recommended to choose case 2, or case 1 with IBE relaxation on the GB of the original UE.

Observation 8: The reasonableness of the extension scheme and the accuracy of the new inner region definition must be based on one prerequisite: When the extended configuration becomes part of the inner region, the MPR should reduce to the inner level.

Observation 9: When both sides are extended by 0.5* UE CBW, all the outer allocations including full RB allocation could convert to inner, and the MPR could always refer to the inner value regardless of the specific RB allocation.

Observation 10: Based on the fixed extended ratio, the new inner RB region of asymmetric extension could be calculated directly, but the UE’s RB allocation stays the same, so there is no need for the NW to indicate extension RB, or RB0shift & BS_NRB to the UE anymore.

Observation 11: For UEs that extend 0.5 times the original bandwidth on one side, LCRB should be less than or equal to 2/3 * NRB for the inner RB region.

Proposal 4:

- If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region.

- If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be redefined as:

RBStart_Low  ≤  RBStart  ≤  NRB – LCRB, and LCRB  ≤  ceil(2/3 *NRB)

Fig 5. inner region of extended CBW, higher side extends by 0.5*UE CBW

- If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be redefined as:

RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(2/3 *NRB)

Fig 6. inner region of extended CBW, lower side extends by 0.5*UE CBW

Proposal 5: Based on the fixed extension ratio, i.e., 0.5* UE CBW on each side, the following signaling mechanism could be considered:

- It is up to the NW to judge the gap width on both sides of the UE.

- When the gap on one side is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends the signaling (e.g., BWExtension -LowerSide-Rel19, or BWExtension-HigherSide-Rel19) to indicate the extension on the corresponding side.

When receiving the above extension signaling, the UE judges its allocation type based on the new RB region defined in Proposal 4.

- When the gap on both sides is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends two signaling (e.g., BWExtension -LowerSide-Rel19 + BWExtension-HigherSide-Rel19) or one combined signaling (e.g., BWExtension -BothSides-Rel19) to indicate the extension on both sides.

When receiving the above extension signaling, the MPR of inner region could be used regardless of the RB allocation type.

Observation 12: The extension signaling overhead is very low, as it only depends on the original UE CBW and the gap width and will not change with the RB allocation state.

Proposal 6: The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec, and there should be no other UE capability other than support for the BW extension.

- References

- R4-2503013, WF on power domain enhancement, Huawei, HiSilicon, RAN4 #114.

- R4-2420527, WF on power domain enhancement, Huawei, HiSilicon, RAN4 #113.

- R4-2500728, Discussion on UE RF power enhancement for NR single carrier, vivo, RAN4 #114.