3GPP TSG-RAN WG4 Meeting # 114bis                                 R4-2503856

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.27.2.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Conditional Intra-CU LTM

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to L3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

Discussion

Compared to R18 LTM, conditional Intra-CU LTM should be supported in R19.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

LTM as introduced in Rel-18 offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures. In Rel-19, enhancements should be specified so that the system can benefit from both the high robustness and short interruption. It’s beneficial to combine LTM and conditional L3 mobility procedures and RAN4 needs to define requirements based on RAN2’s design.

2.1 General

In R18 mobility, RAN4 has agreed that the cell switch delay requirement is applicable for the case without L1 measurement and report under certain conditions for FR1. Meanwhile, for FR2, the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured. Using the same processing method, last time we agreed that CLTM can be triggered through L3 measurement in FR1, while FR2 does not support L3 measurement triggering. In this case, we need to discuss whether to support deactivated SCell on a case by case basis.

Issue 1-1-1: The applicable scenarios for conditional LTM < Agreement > For the case which CLTM is triggered by L3 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included. For the case which CLTM is triggered by L1 measurement, FFS deactivated SCell and the cells with same frequency as deactivated Scell are included.

In the last meeting, we discussed the applicable scenarios for conditional LTM and there us an issue need further discusse that whether to support deactivated SCell and the cells with same frequency as deacticated Scell for CLTM triggered by L1 measurement. As far as we know, this is related to the mobility requirements of R18. In R18, there is no requirement for L1-RSRP measurement on deactivated SCell. Therefore, if we want to support deactivated SCell for L1 triggered CLTM, L1-RSRP measurement on deactivated SCell should be condidered at first.

RAN4#111 Issue 2-1-1: whether to consider L1-RSRP measurement on deactivated SCell Online agreement < Agreement>: - In Rel-18, RAN4 will not define the requirement for L1-RSRP measurement on deactivated SCell. - It is up to UE on how to perform the measurement if the L1-RSRP measurement on deactivated SCell is supported based on RAN1/2 design.

Deactivated SCell is already supported in R18, but we did not define the requirements during the maintenance phase due to potential impact on other work groups and time constraints. In our understanding, if time permits, we tend to consider the case.

Proposal 1：For the case which CLTM is triggered by L1 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included.

Issue 1-2-1: UE based TA measurement < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

Regarding the issue of UE based TA measurement, RAN4 has also discussed the same issue in R18 mobility, but due to time constraints, we did not define dedicated requirements. In the discussion of R19 conditional LTM, RAN2 agreed to consider two TA acquisition methods at the RAN2#127bis meeting, including UE based TA measurement and PDCCH ordered early TA acquisition. At the same time, signaling details were discussed in RAN2#128 meeting. So, from the perspective of RAN4, we should define the relevant requirements. Meanwhile, the dedicated requirements for UE based TA measurement can also be used in R18 LTM.

RAN2#127bis - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. RAN2#128 - The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. - Candidate cell TA is maintained by a new timer.

Proposal 2: RAN4 should define RRM requirements for UE based TA measurement.

2.2 Conditional LTM delay

From the objective, UE evaluated conditions will be specified by other groups to trigger LTM. RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need be considered. Meanwhile, the requirements are applicable to conditional cell switch to change the NR PCell to another NR cell. The relevant requirements can refer to the conditional handover of L3 in section 6.1.4.

6.1.4.2 NR FR1 – NR FR1 conditional handover The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR1 cell. ##### 6.1.4.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover Tmeasure is the measurements time stated in clause 6.1.4.2.2. TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3. Tinterrupt is the interruption time stated in clause 6.1.4.2.4.

Proposal 3: For conditional LTM delay, CHO requirements framework can be used as baseline.

In last meeting, RAN4 has reached agreement that The ending point of conditional LTM is the time the UE transmits the first UL transmission on the target cell, which can be used for both rach-based and rach-less procedure. As to the starting point of conditional LTM delay, the difference is whether to consider TRRC as a component of conditional LTM delay.

Issue 1-3-3: The components of conditional LTM delay < Agreement > FFS the following options ad hoc - Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-processing + TLTM-IU ms - Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms - Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms.

In the last meeting, we have discussed on the components of conditional LTM delay. We can observe from above that there is no other concern on conditional LTM delay but whether TRRC is needed. Therefore, we can use DCLTM = [TRRC] + TEvent_DU + Tmeasure + TLTM-RRC-processing +TLTM-processing + Tfirst-RS + TRS-proc+ TLTM-IU as baseline, then discuss whether TRRC is needed based on the progress from other work group.

Proposal 4: Use DCLTM = [TRRC] + TEvent_DU + Tmeasure + TLTM-RRC-processing +TLTM-processing + Tfirst-RS + TRS-proc+ TLTM-IU as baseline and discuss whether TRRC is needed.

As to interruption, there are four options to choose according to the summary of the last meeting.

  - TCLTM_interrupt = TLTM-processing + TLTM-IU ms

  - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms

  - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

  - TCLTM_interrupt = TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

This can be summarized into two questions. Firstly, whether TLTM-RRC processing falls within the scope of interruption. We hold a negative opinion on this point, where TLTM-RRC-processing comprises the decoding and validity check of the target cell RRC configuration and is part of the cell switch delay, but not part of the interruption. In L3 CHO, TCHO_execution does not belong to interruption, and similarly, in CLTM, it should not be considered as interruption.

##### 6.1.4.2.4 Interruption time The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH. For intra-frequency or inter-frequency conditional handover, the interruption time shall be less than Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms Where: Tprocessing is time for UE processing. Tprocessing can be up to 20 ms. TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3] T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs. Tmargin is time for SSB post-processing. Tmargin can be up to 2 ms. Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. NOTE 1: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

Furthermore, we need to discuss whether T/F fine tracking can be performed before Tprocessing. TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. If T/F fine tracking is needed, we dooubt whether it can be executed before target cell parameters apply and L1/L2 change. Of course, the value of Tfirst-RS and TRS-proc can be zero under some condition, such as early sync is supported and maintain.

Proposal 5: use TCLTM_interrupt = TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms as baseline and further discuss whether T/F fine tracking (TΔ) if needed can be executed at first and then TLTM-processing during conditional LTM.

2.3 Subsequent Conditional LTM delay

As we mentioned before, the difference between conditional LTM delay and subsequent conditional LTM delay is whether to consider RRC procedure. Therefore, Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Meanwhile, the definition of TEvent_DU need to be updated, which is similar as subsequent CPAC in R18.

RAN4#106bis Issue 1-1: starting point of subsequent CPC in RRM requirements Starting point of subsequent CPC in RRM requirements is the time when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change. Issue 1-2: ending point of subsequent CPC in RRM requirements Ending point of subsequent CPC in RRM requirements is the transmission of PRACH preamble towards the target PSCell. Issue 1-3: subsequent CPC delay requirements For subsequent CPC delay requirements: if starting point is the time when UE completes the previous CPC/CPA, Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms and the definition of each component is the same as 8.11B.2 in TS38.133 except that TEvent_DU needs to be updated. Issue 1-4: TEvent_DU in subsequent CPC delay requirements TEvent_DU is the delay uncertainty which is the time from [when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change] until a condition exists at the measurement reference point which will trigger the subsequent conditional PSCell change.

Proposal 6: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Similar as SCPAC, TRRC will not be considered in subsequent conditional LTM delay and TEvent_DU needs to be updated.

Conclusions

In this contribution, we put forward the following proposals on conditional LTM.

Proposal 1：For the case which CLTM is triggered by L1 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included.

Proposal 2: RAN4 should define RRM requirements for UE based TA measurement.

Proposal 3: For conditional LTM delay, CHO requirements framework can be used as baseline.

Proposal 4: Use DCLTM = [TRRC] + TEvent_DU + Tmeasure + TLTM-RRC-processing +TLTM-processing + Tfirst-RS + TRS-proc+ TLTM-IU as baseline and discuss whether TRRC is needed.

Proposal 5: use TCLTM_interrupt = TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms as baseline and further discuss whether T/F fine tracking (TΔ) if needed can be executed at first and then TLTM-processing during conditional LTM.

Proposal 6: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Similar as SCPAC, TRRC will not be considered in subsequent conditional LTM delay and TEvent_DU needs to be updated.

References

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2502602, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom