3GPP TSG-RAN WG4 Meeting #114bis	R4-2503863

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.17.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on maintenance for fast SCell activation based on EMR

Document for:	Discussion

# Introduction

The enhancement about fast SCell activation based on Rel-18 EMR is completed in last meeting with agreements captured in [1] and the CR is endorsed in [2]. In last RAN4 meeting, the structure of the CR is updated to captured the requirements in dedicated section [3]. In this contribution, we provide our views on maintenance about how to accommodate the new requirements in the spec.

# Discussion

In the latest version of the requirements [3], dedicated sections are introduced to captured the new requirements, and other requirements are updated accordingly when referring to the new section. In particular, for PUCCH SCell activation requirements, Tactivation_time can directly refer to the new section. Apart from that, there are other aspects which needs to be updated accordingly, which is not discussed in last meeting. Taking Rel-18 L3 reporting after SCell activation as reference, the known conditions for PL-RS should be updated. We provide TP as follows:

For the UE capable of l3-MeasUnknownSCellActivation, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 reporting after SCell activation command and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for the L3 reporting after SCell activation command - The target pathloss reference signal determination is based on the L3 reporting after SCell activation command, if UE only sends L3 reporting before receiving TCI activation command, or - The target pathloss reference signal determination is based on either the latest L1-RSRP measurement reporting or the L3 reporting after SCell activation command, if UE reports both before receiving TCI activation command - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB ------------------------------------ Text proposal 1------------------------------------ If the target SCell is known and following conditions are met: - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or, - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or, - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode, the pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for measurement report and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - The PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for the measurement reporting - The target pathloss reference signal determination is based on the reporting - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB

Proposal 1: Based on proposal 1, the requirements for unknown PUCCH SCell activation based on EMR shall be updated accordingly as proposed TP in this contribution.

The change can also be found in our companied CR in this meeting.

# Conclusions

Proposal 1: Based on proposal 1, the requirements for unknown PUCCH SCell activation based on EMR shall be updated accordingly as proposed TP in this contribution.

# References

[1] R4-2420094    WF on RRM requirements for NR_RRM_Ph5_Part2

[2] R4-2420132    draftCR on Fast SCell activation for UE supporting Rel-18 eEMR

[3] R4-2502685    draft CR for fast SCell activation for UE supporting Rel-18 EMR