3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503869

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.22.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on UE initiated BM

Document for:	Discussion

# Introduction

The Rel-19 MIMO WI is approved in [1] and further updated in [2], and RAN4 discussed the RRM impacts with agreements and issues captured in [3]. In this contribution, we further provide our views on the objective on UE initiated BM.

# Discussion

Based on the discussion in last meeting, the agreements reached are summarized as follows [3]:

Issue 1-13: Whether to enhance beam sweeping in event triggered L1 beam reporting? Agreement: Reuse current beam sweeping factor for UE/event triggered beam reporting. Issue 1-2: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1)? Agreement: The agreements under this issue is for event 2, and further discuss whether these agreements can be applicable to other events agreed in RAN1. TL1-RSRP_event triggered_measurement_Period_SSB or TL1-RSRP_event triggered_measurement_Period_CSI-RS - update based on 9.5.4.1 (SSB based), 9.5.4.2 (CSI-RS based) and 9.13.4 - TReport = 0 or be removed from the measurement period - N: Reuse the legacy definition - M: FFS pending RAN1 further agreement - TSSB (taking SSB as example) - For the same periodicity supported in RAN1, TSSB_current, or TSSB_new - RAN4 to further discuss the case of different periodicities if different periodicities will be [supported or not excluded] in RAN1 - Option A: Max{TSSB_current, TSSB_new} - Option B: When the RS periodicity is not same, UE evaluate the event after each measurement occasion of current and new beam. If the event is triggered after the current beam measurement, TSSB is TSSB_current. If the event is triggered after the new beam measurement, TSSB is TSSB_new. - Option C: others are not precluded Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM Agreement: - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM Issue 1-16: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? Agreement - The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

One FFS point is whether to consider the case when the periodicity of the Current beam and new beam are different. Based on the latest agreements in last RAN1 meeting, which is shown as follows, for cross-CC case, the new beam RS and current beam RS are in the same CC. Thus, we can support to prioritize the same periodicity case in Rel-19.

Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.

Observation 1: Based on latest RAN1 progress, for cross-CC case, the new beam RS and current beam RS are in the same CC.

Proposal 1: RAN4 to prioritize the requirements for UEIBM for same periodicity between new beam(s) and current beam.

In details, one FFS point is about the M value, which related to the configuration of timeRestrictionForChannelMeasurement. The corresponding descriptions in RAN1 spec are shown as follows:

TS 38.214

In RAN4 spec, it will impact the measurement period (i.e. scaling by 1 or 3). Based on the discussion so far, we didn’t see any discussion in RAN1 about it. Considering that the reporting configuration is based on legacy reportConfig, we assume that it can either be configured nor not configured. Thus, we provide analysis on the impacts for UEIBM w.r.t timeRestrictionForChannelMeasurement is configured and not configured.

When timeRestrictionForChannelMeasurement is configured:

For this case, the situation is less complicated. Considering that we only consider same periodicity case, which means there is always “a pair” of new results for both new beam and current beam. Thus, it is straightforward to evaluate the event based on the most recent result from both new beam and current beam. Otherwise, if the only new beam uses the most recent result but current beam uses old/out-of-dated results, it makes no sense and the event could be wrongly triggered (i.e. the results for comparation experience completely different channel).

Observation 2: When timeRestrictionForChannelMeasurement is configured, event evaluation should be based on the most recent result from both new beam and current beam. Otherwise, the event could be wrongly triggered (i.e. the results for comparation experience completely different channel).

Proposal 1: When timeRestrictionForChannelMeasurement is configured, M =1, and the event evaluation should be based on the most recent result from both new beam and current beam

When timeRestrictionForChannelMeasurement is not configured:

According to the description in RAN1/RAN4 spec, there is no clear restriction on UE measurement behaviour. Following implementations are allowed:

- A: UE measure each RS and perform filtering using 3 RS.

- B: UE measurement each RS with filtering.

- C: UE perform measurement in relaxed mode once every 3 RS.

For delay requirements perspective, similar approach could be reused that assuming M=3 to define delay requirements. However, whether to restrict filtering shall be used for both current beam and new beam is not clear. If filtering is not used, whether to restrict the samples used for evaluation should be also considered. For instance, it is not reasonable to use the first sample from current beam and the third sample from the new beam for comparation.

Proposal 2: When timeRestrictionForChannelMeasurement is not configured, whether filtering is assumed for both new beam and current beam shall be discussed.

Regarding whether to define requirements for both time window based and no time window based requirements, the progress in RAN4 are as follows:

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M? <Way forward > - Option 1: (Qualcomm, Apple, Ericsson) - Only define requirements for the number of events is met at once - Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable M - Option 2a: (ZTE) - Event1&Event 2: Case 2: multiple samples M and timer X should be considered - Event1&Event 7: FFS - Option 2b (Samsung) - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event. - Option 2c (MediaTek) - For multiple events is met by counter: - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS - C: event instance counter is configured by NW.

From our understanding, there is not much RAN4 efforts on considering this. From UE measurement behaviour/requirements, there is no difference compared with no-time window based. The only additional part is how to determine number of event instance. However, the impact on RAN4 is how to define the evaluation behaviour/requirements, which is also common for non-time window based.

Besides, we think time window-based triggering is also important for this feature, different from L3 event triggering with filtering, L1-RSRP could be impacted by instance channel fading, which may lead to wrong event triggering, and it may mislead NW following action (e.g. wrong TCI state switching).

Observation 3: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading.

Proposal 3: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering.

Regarding the time window-based event triggering, latest RAN1 progress can be found as follows:

Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point. Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured. - Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured. - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s). - Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW. - Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: T_window is the agreed time window parameter for measurement. - Note: Other options are not precluded. Agreement On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured Above is NOT applicable for event 1.

For time windowed based case, one thing may need to be discussed is how to count one event instance based on the measurement. Considering that new beam and current beam have same periodicity, similar as no-time-window based case, there is always “pair” of new results from both new beam and current beam. Considering following cases as shown in the figure, it is reasonable to evaluate the event instance based on fresh result (and closing to each other) from both new beam and current beam as shown in the upper part of Fig.1. For instance, if there is instance fading happens at point A for current beam, it will only create on event instance, UE will not trigger event to NW, which is also the motivation to have window-based UE evaluation. However, if UE only perform measurement on new beam and using history results of current beam, one bad result due to instance change fading will lead to multiple event instance and triggers wrong event to NW work.

Fig.1 Instance counting using different results

Proposal 4: For time window-based scenario with same periodicity between current and new beam, one event instance shall be evaluate based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance.

Another issue is about CSI-RS based L1-RSRP for inter-Cell BM, the status is summarized as follows:

Issue 1-7: CSI-RS based L1-RSRP for inter-cell BM <Way forward > - Option 1: (Qualcomm, Nokia, Samsung, MediaTek) - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. - Option 2: (CMCC) - For reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused. - Option 3: (Apple, Huawei) - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. - Option 4: (ZTE) - RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. - Option 5: (Ericsson) - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2 - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

From our understanding, the inter-cell/intra-cell is the definition only differentiated w.r.t SSB since UE needs detect SSB with same SSB index from different TRP/Cell with different PCI. However, for CSI-RS there is no such differentiations. Different CSI-RS resources from different TRP/Cells can QCLed to SSB with different PCI.

Observation 4: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell.

Proposal 5: CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

Another issue is about whether to consider the UE capability defined in LTE with following proposals.

Issue 1-15: Compatibility of capabilities with LTM <Way forward > - Option 1: (Apple, Ericsson) - Discuss it later. - Option 2: (CMCC, Huawei, Nokia, Samsung) - Do not mix/combine Rel-19 mobility and Rel-19 MIMO. Define the capability separately. - Option 4: (MediaTek) - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

We are not in favour of combining two WI to define a single UE capability. In one hand, different group of peoples are involved in different WI discussion, and the technical details are not fully clear to all. In the other hand, it may cause difficulties when implement the feature from both UE side and NW side since NW may need to jointly consider two features when only configure one of them.

Observation 5: Do not combine UE capabilities across different WID, and postpone the discussion in RAN4 with more RAN1/2 progress.

# Conclusions

Observation 1: Based on latest RAN1 progress, for cross-CC case, the new beam RS and current beam RS are in the same CC.

Proposal 1: RAN4 to prioritize the requirements for UEIBM for same periodicity between new beam(s) and current beam.

Observation 2: When timeRestrictionForChannelMeasurement is configured, event evaluation should be based on the most recent result from both new beam and current beam. Otherwise, the event could be wrongly triggered (i.e. the results for comparation experience completely different channel).

Proposal 1: When timeRestrictionForChannelMeasurement is configured, M =1, and the event evaluation should be based on the most recent result from both new beam and current beam

Proposal 2: When timeRestrictionForChannelMeasurement is not configured, whether filtering is assumed for both new beam and current beam shall be discussed.

Observation 3: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading.

Proposal 3: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering.

Proposal 4: For time window-based scenario with same periodicity between current and new beam, one event instance shall be evaluate based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance.

Observation 4: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell.

Proposal 5: CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

Observation 5: Do not combine UE capabilities across different WID, and postpone the discussion in RAN4 with more RAN1/2 progress.

# References

[1] RP-234007 New WID: NR MIMO Phase 5

[2] RP-242394 WID revision: NR MIMO Phase 5

[3] R4-2502595 WF on RRM requirements for NR_MIMO_Ph5_Part1