3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503872

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.31.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact due to periodic pattern for IoT NTN

Document for:	Discussion

# Introduction

The new WID of NTN IoT TDD mode was approved in [1] and was revised in [2]. RAN4 studied the impact due to periodic pattern from RAN4#113 meeting with agreements captured in [3]. The RRM impacts are initially discussed in RAN4#114 meeting with progress captured in [4]. In this paper, we provide our views on the potential impacts on RRM impacts.

# Discussion

The latest progress in RAN1/2 are summarized as follows：

Agreement @ RAN1#120

Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [48…50] ms at the ULSRP - NOTE: This corresponds to the largest separation between DL and UL Iridium slot pairs. Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA Agreement The downlink subframes within D=8 are fixed to: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) Agreement NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - FFS: handling for SIB1-NB Agreement The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications - NPUSCH is mapped as per current specifications. - NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications. - FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes” - FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor) - FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD. - FFS: handling of DL transmission gap - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. Conclusion For determining the offset between the 90ms TDD structure and the 10240ms H-SFN: the UE may derive the 90ms TDD structure based on observations of downlink signals. No specification impact. Agreement For NPRACH, further consider the two options until RAN1#120bis: - Option 1: New NPRACH periodicities (multiple of 90ms) are introduced for NB-IoT NTN TDD. - Option 2: NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s) - FFS: The unit of postponement (e.g. PRU) Agreement NPRACH format 2 is not supported in NB-IoT NTN TDD. Working assumption Uplink transmission gaps do not apply in NB-IoT NTN TDD. Conclusion RAN1 does not further discuss the issue of Kmac extension unless triggered by RAN2 LS. Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any. Agreement For segmented pre-compensation, RAN1 to further discuss at least the following options: - Option 1: Dropped/postponed NPUSCH / NPRACH are counted for segment determination - Option 2: A new pre-compensation segment is started at the beginning of each uplink burst - Option 3: UL segment should only be defined in consecutive U subframes - Option 4: Segmented pre-compensation does not apply for NB-IoT NTN TDD. - FFS: How to handle the UL segment gap

Agreement @ RAN2#129

Agreements:

- RAN2 will continue studying paging aspects based, on RAN1 progress

- RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

- RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

- RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated.

- RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

- RAN2 can continue the discussion also on RAR window

- Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

Based on progress in other WG, we provide our views on remaining RAN4 issues based on discussion in last meeting.

Regarding the general aspects, RAN4 achieved following agreements.

## Issue 0: General aspects Agreement (online): For TDD NB-IoT in NTN, the following aspects will be further considered in future RAN4 meetings: - The target SNR - Applicable coverage (whether the EC needs to be considered) - Applicable Rmax - Applicable DRX cycle - Note: The DRX cycle might not be non-uniformly periodic due to the H-SFN not matching with the new TDD pattern. This issue will be addressed by other working groups. In the meantime, RAN4 assumes the DRX cycle is uniformly periodic.

Regarding the target SNR/CNR, based on previous RAN1 agreements, the targeting operation CNR is around -1 dB consider different UE antenna gain. The current SNR level for NB-IoT is categorized into NC (-6dB) and EC (-15 dB). Firstly, we think we can focus on NC scenario in this release. Another question is whether to update the operation SNR to -1 dB. This means we may need to re-evaluate the perform for Cell search/measurement. Though we may have enhanced requirements with less samples in high SNR level, the workload and spec impacts are huge. Based on above consideration, we propose to only consider NC and with legacy SNR conditions, thus most requirements can be reused and the spec impacts are less.

Proposal 1: RAN4 only define requirements for NC with legacy SNR conditions.

For applicable Rmax, based on following RAN1 agreements, the TDD pattern will not change the definition of repetition counting. This may cause overlong repetition sequence, and it will be handled by RAN1 as listed FFS points.

Proposal 2: The applicable Rmax depends on further RAN1 discussion.

The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications. - FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes” - FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor) - FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD. - FFS: handling of DL transmission gap - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

Regarding the DRX aspects, based on following RAN1 agreements, the DRX is still uniformly distributed but the available DL sf may shift for each DRX cycle.

Conclusion For determining the offset between the 90ms TDD structure and the 10240ms H-SFN: the UE may derive the 90ms TDD structure based on observations of downlink signals. No specification impact.

Observation 1: The DRX cycle is still uniformly distributed.

For cell reselection requirements, the agreements are shown as follows:

## Issue 1: Cell reselection (4.6A.2) Agreement (online): For the cell reselection for TDD NB-IoT in NTN, discuss the following details in the next meeting: - Tdetect, Tevaluation and Tmeasure

For Cell reselection requirements, as analysed in last meeting, no blind detection is assumed at the UE side. It means there should be a SMTC like indication to locate the location of transmitted DL subframe not only for serving cell but also for neighbour cell. For the agreed TDD pattern as N = 9, there is one NPSS occasion available every N frame, and there is on NSSS occasion available every N or 2*N frames. Compared with the length of DRX cycles, the periodicity is still shorter. However, based on principles of defining requirements, UE is not required to monitor every occasion within the DRX cycles for power saving considerations. But for neighbour cell measurement, how to allocate the location of the 8 DL sf is still not clear.

Proposal 3: For Cell reselection requirements, take the existing requirements as baseline.

- FFS the impact on allocating the location of DL subframe of neighbour cell, which depends on the progress in other WG.

For RRC re-establishment requirements, the RAN4 agreements are shown as follows:

## Issue 3: RRC re-establishment (6.5A) Agreement (online): In the core requirement, For the delay requirement (TUL_grant + TUE_re-establish_delay_NB-IoT) of RRC re-establishment for TDD NB-IoT in NTN, - No update to TUL_grant - For the following parameters in TUE_re-establish_delay_NB-IoT - No update: - TPRACH_NB-IoT - TSI_NB1-NC [and TSI_NB1-EC] - Consider update, and the details are FFS: - Tsearch_NB1-NC,i [and Tsearch_NB1-EC,i] for unknown cell

The requirements are defined in absolute time, e.g 1400ms per frequency layer for NC and 14800ms per frequency layer for EC. It is straight forward that the requirements shall be scaled. For instance, if there is one NPSS/NSSS available every N frame, the requirements shall be scaled by at least N.

Proposal 4: For RRC re-establishment requirements, the search time per frequency layer (i.e.Tsearch_NB1-NC,i) shall be scaled by N=9.

For RLM measurements impacts, RAN4 agreements are as follows:

## Issue 5: RLM (7.23A) Agreement (online): Further discuss the following aspects for RLM for TDD NB-IoT in NTN: - FFS update the hypothetical NPDCCH format (including Rmax) specified in Table 7.23A.2-1 upon RAN1 design in future meetings. - Evaluation period when DRX is not in use: - Option 1: Reuse the existing requirement defined in 7.23A.2.1, unless RAN1 changes cause impact to this requirement - Option 2: The evaluation period defined in 7.23A.2.1 is scaled - Option 2a: scaled by N=9 - Option 2b: scaled by FFS - Option 2c: no scaling, and define requirement based on 8ms DL subframes - Evaluation period when DRX is in use: - FFS whether the evaluation period defined in 7.23A.2.1 needs to be scaled.

For RLM requirements, it is the same story when DRX is not configured. It is assumed 400/200 ms are need for OOS/INS evaluation when Rmax is smaller than 64 and 4000/2000 ms when Rmax is larger than 64. However, the RLM measurement is basically the SINR evaluation. For large Rmax, UE is expected to operating in lower SINR conditions, thus longer evaluation time is allowed. Regarding the applicable Rmax, as discussed above, it depends on further RAN1 progress. For the evaluation period, based on the principle in Rel-13 [5], the requirements are defined based on NRS. One way to transform the requirements for TDD is to guarantee the same number of available NRS. Taking Rmax is smaller than 64 as example, there are at least 40/20 SF available based on the legacy assumption. For TDD, according to agreed the pattern in RAN1 (i.e. [3 4 5 6 7 8 9 0]), there are 3 or 4 (not overlapping with NSSS) NRS available every 90 ms. Thus, 1200/600 ms is needed for OOS/IS evaluation. However, there is also discussion on whether to update the NRS availability for TDD.

Observation 2: Legacy RLM requirements are defined based on NRS.

Observation 3: The OOS/INS evaluation period requirements for TDD could be updated to guarantee the same number of available NRS are available.

Proposal 5: The OOS/INS evaluation period requirements should be updated as 1200/600 ms when Rmax is smaller than 64 based on the legacy assumption about available NRS.

Then, for DRX mode, same scaling factor as non-DRX mode shall apply (e.g. 3 as analysed above).

Proposal 6: When DRX is in use, the OOS/INS evaluation period requirements should be scaled by 3 based on the legacy assumption about available NRS.

For measurement requirements in connected mode, which are further divided into serving cell measurement and neighbour cell measurement. For serving cell measurement and intra-frequency measurement, same situation as RLM, when DRX is not configured, the delay requirement shall be scaled.

However, for inter-frequency measurement, the delay requirements are defined in number of available samples. RAN4 defined minimum length of occasion for detection and measurement which is 200/50ms. For TDD with periodic pattern, obviously that the minimum length shall be reconsidered. For instance, if the NSSS periodicity is 90 ms, the minimum length of one measurement occasion should be 90ms.

Proposal 7: For connected mode serving cell measurement and intra-frequency measurement, the delay requirements shall be scaled by N=9. For inter-frequency measurement, the minimum length of measurement occasions shall be defined as the max {existing value, periodicity of corresponding RS}.

# Conclusions

Proposal 1: RAN4 only define requirements for NC with legacy SNR conditions.

Proposal 2: The applicable Rmax depends on further RAN1 discussion.

Observation 1: The DRX cycle is still uniformly distributed.

Proposal 3: For Cell reselection requirements, take the existing requirements as baseline.

- FFS the impact on allocating the location of DL subframe of neighbour cell, which depends on the progress in other WG.

Proposal 4: For RRC re-establishment requirements, the search time per frequency layer (i.e.Tsearch_NB1-NC,i) shall be scaled by N=9.

Observation 2: Legacy RLM requirements are defined based on NRS.

Observation 3: The OOS/INS evaluation period requirements for TDD could be updated to guarantee the same number of available NRS are available.

Proposal 5: The OOS/INS evaluation period requirements should be updated as 1200/600 ms when Rmax is smaller than 64 based on the legacy assumption about available NRS.

Proposal 6: When DRX is in use, the OOS/INS evaluation period requirements should be scaled by 3 based on the legacy assumption about available NRS.

Proposal 7: For connected mode serving cell measurement and intra-frequency measurement, the delay requirements shall be scaled by N=9. For inter-frequency measurement, the minimum length of measurement occasions shall be defined as the max {existing value, periodicity of corresponding RS}.

# References

[1] RP-242415 New WID on introduction of IoT-NTN TDD mode

[2] RP-243293 Revised WID on introduction of IoT-NTN TDD mode

[3] R4-2419795 Way Forward for [113][317] IoT_NTN_TDD

[4] R4-2502607 WF on RRM requirements for IoT_NTN_TDD

[5] R4-162872 Way forward on NB-IoT RLM

[2] R4-164550 WF on NB-IoT cell identification