3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503900

Wuhan, China, 7 – 11 April, 2025

Title: 	Discussion on RRM requirements for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	7.26.4.2

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#114, and the outcomes are captured in WF [1]. Based on [1] the following issues need further discussion.

- General aspects

- LP-WUR measurement

- MR measurement

- LP-WUR at CONNECTED mode

In this paper, we will provide our views on RRM requirements related to LP-WUR.

Discussion

General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

We do not support to define Case 2.

- There is no clear necessity to require MR to measure the serving cell since LR is already measuring the serving cell.

  - In particular, the MR measurement will be with relaxation, so it is much more infrequent compared to the LR measurement.

  - Some companies commented that LR measurement may be less accurate than that of MR. In our view, however, it is both necessary and feasible to keep similar accuracy between LR and MR.

  - Some companies commented that MR measurement may be used by UE to assist LR measurement. We believe this can be left to UE implementation rather than mandated by requirements.

- There is additional power consumption due to MR measurement even with relaxation.

- It will complicate the spec since another threshold (coverage level) is needed to switch between Case #1 and Case 2.

Proposal 1: RAN4 not to define Case 2 for MR and LR measurement.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

In RAN4#114 it is agreed that at legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation, but it is FFS whether to define requirements for LR measurement.

We do not support to define LR measurement requirements at legacy state. We understand this is a transient period where LR measurement is needed only for evaluation of the entry condition, and this is quite different from Case 1 or Case 3 where LR measurement is for serving cell evaluation that is important for mobility. In other words, we do not see much significance to define the requirements. Functional wise, UE must evaluate the LR based criterion if configured by NW for the entry condition. This evaluation is not time critical, and it can be up to UE to decide how fast it would like to complete the evaluation as it only impacts when UE can get the power saving gain.

In addition, at legacy state MR measurement is ongoing without relaxation, and it may be difficult to define LR measurement requirements considering UE is not assumed to simultaneously operate LR and MR.

Finally, it would be difficult to test this requirement via test since when and how to turn on LR for serving cell measurement is up to UE implementation.

Proposal 2: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Issue 1-1-10: Considerations on higher priority frequency layer

In our view, UE should always perform measurement on higher priority carriers, if configured. Configuring higher property carriers is an important tool from NW perspective to manage mobility or load balancing in RRC_IDLE or RRC_INACTIVE, and it should not be totally compromised due to power saving.

In RAN4#114, the concern to support measuring higher priority carriers in Case 1 is that by definition the MR should be totally turned off in Case 1. We understand it is mainly a terminology issue, and we can call it Case 4 in RAN4. Technically, we understand when UE has met the entry condition of offloading (as defined by RAN2), the UE is in Case 1 if it is not configured with higher priority carriers, and is in Case 4 otherwise.

On the exact requirements, we support to follow R16 relaxed requirements for higher priority carriers. i.e. each higher priority carriers should be searched at least every 1 hour.

Proposal 3: Support higher priority frequency layer search when UE meets the entry condition of offloading and is configured with higher priority frequency layers. UE should search each higher priority carriers at least every 1 hour.

LP-WUR measurement

Issue 1-2-1: Accuracy requirements

We support to define accuracy requirements for LR measurement in the same way as cell reselection requirements, where accuracy is implicitly defined as margin (highlighted in yellow in the copied requirements). So far, dedicated accuracy requirement in clause 10 are all for reported measurement in RRC_CONNECTED, and we prefer to keep same principle for this WI.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that: when rangeToBestCell is not configured: - the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

Proposal 4: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement, and reflect the accuracy performance as a margin in the core requirement.

Issue 1-2-2: Periodicity for LP-WUR measurement

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

We support to define lower limit for LP-SS measurement periodicity. RAN4#112-bis agreed that LP-SS measurement periodicity shall be defined based on LP-SS periodicity. In our view, the agreement was made based on the assumption of LP-SS periodicity equalling to 320ms. However, RAN1 has considered LP-SS periodicity from 80ms to 10240ms.

In our view, smaller LP-SS periodicity like 80ms or 160ms is targeting for sync for LP-WUS decoding, but the LR measurement does not have to be so frequent. As discussed below for SSB measurement periodicity, even the measurement periodicity is LO periodicity, the impact to user experience is still acceptable. On the other hand, too frequent measurement will compromise the power saving gain of measurement offloading.

Proposal 5: Define lower bound for LP-SS measurement periodicity as 320ms.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

In RAN4#113, it was agreed that the periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity, no matter if LP-SS is configured or not. One remaining issue is whether an upper bound is needed for the measurement periodicity. It is essentially a trade-off between power saving gain and how fast UE can exit LR when the condition for LR is not met.

Assuming the measurement period for LR measurement is n samples, and m measurements are needed for determining the exit condition is met (m=1 or m>1 is discussed in Issue 1-2-3), the total time between the time point when the channel condition gets insufficient for LR to the time point when UE determines to exit LR would be

Texit = n * Tsample + (m-1) * Tsample

where Tsample is the measurement periodicity. Assuming no upper limit is defined, UE may miss (n+m-1) LP-WUS since UE may not be able to decode LP-WUS under the channel condition. We understand n and m will be in the range of {1,2,3,4}, and it should not be big issue to user experience considering the probability of actual paging is low. Therefore, we suggest not to define upper limit for the measurement periodicity.

Another open issue is the measurement periodicity when LP-SS is configured and LP-SS periodicity is larger than LO periodicity. As RAN1 has not concluded on LP-SS periodicity, it is not clear whether this case exists or not, so we suggest to postpone the discussion for this case until LP-SS periodicity is clear in RAN1.

Proposal 6: Measurement periodicity for SSB is the LO periodicity, i.e. no upper limit is defined. Postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity.

Issue 1-2-4: On LR measurement/evaluation requirements

Issue 1-2-4-1: On LR measurement requirement

Issue 1-2-4-1: On LR measurement filtering requirement

In RAN4#114 following is agreed regarding the LR evaluation requirements.

Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

The common understanding behind the agreement was to define LR requirements similar to existing MR requirements for neighbour cell. It is noted that in existing requirements for MR neighbour cell (as copied below), both Tevaluate and Tmeasure are defined, so we support to define Tmeasure.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5) for intra-frequency cells that are identified and measured according to the measurement rules. The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

On the two options to define LR measurement requirements, we support option 1, i.e. define the minimum interval where one LR measurement shall be executed. This is same as existing requirements, and it defines how frequent LR should perform serving cell measurement. It would ensure that UE would not evaluate LR threshold based on a single measurement which is unreliable due to channel fading.

As to option 2, i.e. define the minimum duration to meet accuracy requirement, we do not see much value because it effectively defines with how many samples UE must be able to get an accurate measurement result. It is a consideration factor in defining Tevaluate, but does not need to be defined explicitly. It is noted that it is not defined in existing requirements either. It is clear that UE is not expected to meet the accuracy (defined as reselection margin) with single measurement occasion, but exactly how many samples are needed is not specified.

On the filtering requirements, we do not have strong view since the fading impact can be addressed by defining Tmeasure, but we slightly prefer to define them in the same way as in existing requirements.

Proposal 7: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed. UE shall filter the LR measurements using at least 2 measurements with enough separation.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

In RAN4#114 the framework of the LR evaluation requirements has been agreed, and the remaining issue is to decide on the exact number of x1 and y1 based on x and y which are determined via simulation.

According to our simulation results, x = y = 2 should be sufficient to meet the target BB accuracy of 3.5dB for both LP-SS (including all {M,L} combinations) and SSB, and we suggest to define x1 = y1 = 4 which allows UE to use multiple measurements for the evaluation.

Proposal 8: Define LR evaluation requirement based on x1 = y1 = [4].

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

We do not see the need to define cell selection evaluation requirements for LR. In a reasonable deployment, when UE is close to cell edge where S criterion needs to be evaluated, UE should have already exited LR measurement, and MR measurement should have started.

In last meeting, some companies raised up the scenario where the coverage of Case 1 is same or similar as cell coverage, i.e. UE is still in Case 1 when it is at very cell edge. We understand NW is allowed to make such configuration, but we do not see it as a typical case. It is noted that if such a scenario is considered, it means UE is also monitoring LP-WUS at cell edge (otherwise there is not much power saving gain). We are not sure if this is the design target of LP-WUS in RAN1, so we suggest RAN4 to align with RAN1 before RAN4 agrees to consider this scenario.

Proposal 9: RAN4 not to define cell selection evaluation requirements for LR.

Issue 1-2-10: LP-WUS monitoring requirements while UE is not required to monitor paging

Issue 1-2-11: LP-WUS monitoring and MR relaxation requirements

We understand the issue is not in RAN4 scope. RAN1 has agreed that “if UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up”. It is a functional UE behaviour and should be defined in RAN1 or RAN2 spec. What RAN4 needs to do is to define requirements to make sure that UE makes timely evaluation of the entry or exit condition of LP-WUS monitoring, and this is already covered by Issue 1-2-3. Also, we understand the discussion in Issue 1-2-3 does not have any implication on whether conditions for LP-WUS monitoring, measurement offloading and MR relaxation are independent or not, which should be decided by RAN2.

MR measurement

Issue 1-2-5: On requirements for MR for entry/exit threshold evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

Issue 1-2-5-2: On MR evaluation requirements

For MR evaluation of the entry condition, we suggest to re-use the existing requirements for MR evaluation of S criterion but with Nserv changed to 1. The reason to reduce Nserv is to allow UE to move to power saving mode more quickly. Considering that in typical configurations the entry condition would be met when the serving cell quality is good, there is no need to mandate UE to make multiple evaluations. It is noted that following RAN1/2 agreement, UE is allowed instead of required to use LR and stop/relax MR when the entry condition is met, so UE can still make multiple evaluations by implementation e.g. for better reliability.

Proposal 10: For MR evaluation of the entry condition, re-use the existing requirements for MR serving cell evaluation with Nserv = 1.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR

In RAN1#120, an LS [2] was sent to RAN4 with the following question.

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

In our view, the number of SSBs assumed by RAN1 for MR synchronization is reasonable for different SSB periodicities. However, we believe a lower bound on the SSB periodicity assumed for the wake-up delay is needed considering UE processing capability because UE is not assumed to measure every SSB transmitted by the NW. In most RRM requirements, UE is only required to process SSB every 20 or even 40ms. For this wake-up case, we suggest to take 20ms as the lower bound on the SSB periodicity, which is aligned with the typical configuration in real deployment.

Proposal 11: RAN4 confirms the number of SSBs assumed by RAN1 for MR synchronization are reasonable for different SSB periodicities, and the lower bound of SSB periodicity for MR synchronization during the wake-up delay is 20ms.

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

We support to re-use the legacy MR accuracy requirements for MR measurement with relaxation, for both serving and neighbour cell. Accuracy requirements do not depend on the measurement interval, i.e. same accuracy requirements apply regardless of whether the measurement is performed once per DRX cycle or per N DRX cycles. Here, by legacy MR accuracy requirements we mean the accuracy in clause 10 for CA/DC Idle Mode Measurements. We are also open to discuss other existing accuracy numbers in RRC_IDLE or RRC_INACTIVE.

Proposal 12: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation, for both serving and neighbour cell.

LP-WUR at CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

It is true that the objective of LP-WUR at RRC_CONNECTED mode does not involve RAN4, but it may be too earlier to preclude any RRM impacts as the RAN1/2 discussions are ongoing, and there may be RRM impacts. RAN4 can first focus on the LP-WUR requirements in RRC_IDLE or RRC_INACTIVE, and postpone the discussion for RRC_CONNECTED mode until more RAN1/2 conclusions are available.

Proposal 13: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Conclusions

In this paper we provided our views on RRM requirements related to LP-WUR.

Proposal 1: RAN4 not to define Case 2 for MR and LR measurement.

Proposal 2: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Proposal 3: Support higher priority frequency layer search when UE meets the entry condition of offloading and is configured with higher priority frequency layers. UE should search each higher priority carriers at least every 1 hour.

Proposal 4: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement, and reflect the accuracy performance as a margin in the core requirement.

Proposal 5: Define lower bound for LP-SS measurement periodicity as 320ms.

Proposal 6: Measurement periodicity for SSB is the LO periodicity, i.e. no upper limit is defined. Postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity.

Proposal 7: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed. UE shall filter the LR measurements using at least 2 measurements with enough separation.

Proposal 8: Define LR evaluation requirement based on x1 = y1 = [4].

Proposal 9: RAN4 not to define cell selection evaluation requirements for LR.

Proposal 10: For MR evaluation of the entry condition, re-use the existing requirements for MR serving cell evaluation with Nserv = 1.

Proposal 11: RAN4 confirms the number of SSBs assumed by RAN1 for MR synchronization are reasonable for different SSB periodicities, and the lower bound of SSB periodicity for MR synchronization during the wake-up delay is 20ms.

Proposal 12: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation, for both serving and neighbour cell.

Proposal 13: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Reference

- R4-2502600, WF on RRM requirements for NR_LPWUS, Moderator (vivo)

- R1-2501624, LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode, RAN1