3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503901

Wuhan, China, 7 – 11 April, 2025

Title: 	Discussion on UE CLI measurement for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	7.23.3.1

Document for:	Approval

Introduction

RRM requirements for SBFD are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for UE CLI measurement.

- General issues

- Specific issues for L1-SRS-RSRP

- Specific issues for L1-CLI-RSSI

In this paper, we will provide our views on RRM requirements for UE CLI measurement in SBFD. In addition, we will also provide our analysis for the spec impact of the WI.

Discussion

General issues

Measurement reporting

In RAN4#114 the following is agreed for measurement reporting requirements.

- Agreements - Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1. - L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the aperiodic L1 based CLI measurement reporting.

In RAN1#120 the following is agreed for measurement reporting.

Agreement - Periodic L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is supported - Semi-persistent L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported - Periodic/Semi-persistent L1 SRS-RSRP reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported

Based on the latest RAN1 agreement, RAN4 should also define measurement reporting requirements for periodic L1 CLI-RSSI reporting. L1-RSRP measurement reporting requirement for periodic reporting as defined in clause 9.5.3.1 can be re-used.

Reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. The UE shall only send periodic L1-RSRP measurement reports for an active BWP, unless the UE supports [FG53-1], provided that the SSB is within the configured UE-specific CBW. The UE shall transmit the periodic L1-RSRP reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

Proposal 1: RAN4 to define measurement reporting requirements for periodic L1 CLI-RSSI reporting. L1-RSRP measurement reporting requirement for periodic reporting as defined in clause 9.5.3.1 are re-used.

Report mapping

RAN1#120 sent an LS [2] to RAN4 with the following content.

The following working assumption was made in RAN1 on L1-based CLI reporting since RAN1 specifications must capture the format and bitwidths of the CLI reports. Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.. RAN1 assumes that the reporting range and granularity for L3 SRS-RSRP measurement in Table 10.1.22.1.2-1, TS38.133 can be reused for L1 SRS-RSRP. In addition, the reporting range and granularity for L3 CLI-RSSI measurement in Table 10.1.22.2.2-1 of TS38.133 can be reused for L1 CLI-RSSI. For a CLI report, RAN1 agreed that the absolute value of the most interfered (largest) measured L1-SRS-RSRP or L1-CLI-RSSI is reported. RAN1 also agreed to use differential reporting for L1 SRS-RSRP and L1 CLI-RSSI when the number of reported measurement resources is larger than one, in the same manner as for existing L1-RSRP/SINR reporting. RAN1 would like to ask RAN4 whether the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI are reasonable. RAN1 also would like to check with RAN4 whether there is any issue with the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI.

We do not see any issue in the range and resolution for reporting of L1-SRS-RSRP and L1-CLI-RSSI as agreed by RAN1 as working assumption, for both absolute reporting and differential reporting. We suggest to confirm this to RAN1.

Another issue in the reporting mapping for L1 CLI measurement is whether to re-use the “Infinity” in Table 10.1.22.1.2-1 for L1-SRS-RSRP, which is not mentioned in the RAN1 LS. The reason behind this reporting is that UE may face very strong interference such that the receiver is saturated and UE cannot measure the SRS power for L3 SRS-RSRP reporting. Technically, the case may happen for L1-SRS-RSRP reporting, so we suggest to keep this “Infinity” report for L1-SRS-RSRP and inform this to RAN1.

Table 10.1.22.1.2-1: SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS-RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS-RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS-RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS-RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS-RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| .. | .. | … |
| SRS-RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS-RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS-RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS-RSRP_98 | Infinity |  |
| NOTE: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

Proposal 2: RAN4 to confirm the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI, including both absolute and differential reporting, are reasonable. RAN4 to inform RAN1 the “Infinite” in Table 10.1.22.1.2-1, TS38.133 is also reused for L1 SRS-RSRP.

A draft reply LS is provided in Annex.

Requirement applicability

In R16, following applicability is defined for SRS-RSRP and CLI-RSSI.

The UE capable of performing CLI measurements shall be able to measure SRS-RSRP and CLI-RSSI which are defined in TS38.215 [4] within active DL BWP. The measurements requirements in this clause apply for TDD mode only. CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resouce is configured within active BWP

For L1-SRS-RSRP and L1-CLI-RSSI, RAN1#118 agreed following.

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband FFS: Method#3: UE measures RSSI within UL subband

Given the RAN1 agreement, R16 applicability can be directly re-used.

Proposal 3: L1 based UE-to-UE CLI measurement requirements apply provided that

- when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

- when L1-CLI-RSSI measurement resource is configured within BW of DL active BWP

Specific issues for L1-SRS-RSRP

Scheduling restriction

In RAN4#114 the following is agreed for scheduling restriction due to L1-SRS-RSRP measurement.

- For L1-SRS-RSRP, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies. - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements, - Note: this time has accounted for the UL/DL switching time. - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition.

One issue to be further discussed is whether there is interruption before and after the measurement due to frequency re-tuning. Some companies commented that UE Rx BW may only cover DL subband before and after the measurement, so it needs to retune the Rx BW to UL subband or BWP to perform the measurement. This is the reason why in the agreement the skipping of scheduling restriction was limited to the symbols with SRS when UE supports FDM-ed DL reception and SRS measurement (whether scheduling restriction can be skipped on the symbol before SRS was open).

We do not support to consider the interruption due to RF retuning before and after the measurement. In our view, such RF retuning, if considered, would not be limited to CLI measurement procedures, but it may also be required for the transition between SBFD and non-SBFD symbols. However, RAN1 and RAN4 RF have only discussed transient period between SBFD and non-SBFD symbols from gNB perspective, and there has been no discussion on the transient period from UE perspective for RF retuning.

Technically, we understand such RF retuning is motivated by enabling a special RF filter on SBFD symbols whose Rx BW will only cover DL subband, so that the UE-to-UE CLI from the UL subband is suppressed. Although the benefit is there, it will increase the cost of the UE due to special RF filter, and it will casue performance loss due to interruptions e.g. at transition between SBFD and non-SBFD symbols, or before and after CLI measurement in UL subband, or even at transition between UE’s Tx and Rx in consecutive SBFD symbols. Considering the pros and cons as well as the standardization efforts in RAN1 and RAN4 RF, we suggest to not consider RF retuning before and after the measurement, i.e. UE is assumed to be able to receive in the UL subband on SBFD symbols without RF retuning.

Another issue to be further discussed in whether there is restriction on symbols after the SRS symbol considering UL/DL transition. Our analysis is shown in Figure 1 where we analyse the UL/DL transition before and after the SRS symbol. In Figure 1, we assume the measurement timing is same as Tx timing while in real world it can be earlier or later than the Tx timing depending on UE implementation of the constant offset.

- In case a), the measurement will overlap with the Rx in the previous symbol, so the Rx in the previous symbol will be impacted by the measurement, unless UE supports FDM-ed DL reception and SRS measurement in which case UE has separate FFTs for Rx and measurement.

- In case b), although the measurement will not overlap with the Tx in the previous symbol (they are both with Tx timing), a transient period is required for UE switching from Tx to Rx, so the Tx in the previous symbol will be impacted. This is already accounted in the RAN4#114 agreement that UL in Y symbols before the measurement is subject to scheduling restriction.

- In case c), the measurement will not overlap with the Rx in the following symbol, so the Rx in the following symbol will not be impacted by the measurement.

- In case d), similar to case b), the Tx in the following symbol may be impacted due to the transient period for UE switching from Rx to Tx, so we suggest that scheduling restriction applies to Y symbols after the measurement in UL. This is new compared to R16 scheduling restriction requirements.

Figure 1: Impact of UL/DL transition before and after the SRS symbol

Based on the above discussions, we suggest to update the scheduling restriction requirements for L1-SRS-RSRP as in Proposal 4.

Proposal 4: Define the scheduling restriction requirements for L1-SRS-RSRP:

- For UL, scheduling restrictions always apply, for both FR1 and FR2, and the restricted symbols include symbols for SRS resource and Y symbols before and after symbols for SRS resource, where Y is re-using same number as R16 requirements

- For DL,

  - if UE is able to support FDM-ed DL reception and SRS measurement and if RAN1 will define this optional UE capability, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2,

  - otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies.

  - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements.

Measurement restriction

In R16, following measurement restriction is defined for SRS-RSRP.

If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TSRS_RSRP_measurement_period.

In our view, R16 requirements can be re-used because the L1-SRS-RSRP measurement cannot be performed at same time as other measurement due to different measurement timing.

Proposal 5: If the SRS resources configured for L1-SRS-RSRP are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP/L1-SINR measurement or measurement gaps, measurement period requirements for L1-SRS-RSRP do not apply.

In RAN4#114, some companies discussed the measurement restriction between L1-SRS-RSRP and L1-CLI-RSSI. As L1-CLI-RSSI is based on DL timing, the two measurements cannot be performed at same time. We suggest to leave it to UE implementation which measurement to perform when measurement resources for the two measurements are partially or fully overlapping. It means measurement period requirements for both measurements will not apply. It is same as existing measurement restriction for SSB and CSI-RS based L1 measurement where UE is required to perform one but not both of the two measurements.

Proposal 6: If the SRS resources configured for L1-SRS-RSRP are partially or fully overlapping with measurement resources for L1-CLI-RSSI, UE is required to perform one but not both of the two measurements, and measurement period requirements for the two measurements do not apply.

Side conditions

On SRS RB number, we suggest to use 24 RB for the side condition based on discussions in RAN4#114. The reason is that the measurement is performed within UL subband which is only part of CBW, and in some gNB configurations the BW of UL subband may be smaller than 48 RB especially for DUD case.

On Es/Iot, we suggest to re-use the R16 side condition of 1dB. Based on our results, the gain of 3dB over 1dB Es/Iot is not much, so it is meaningful to keep using 1dB to ensure the requirement coverage.

Proposal 7: RAN4 to stick to 1dB as the on Es/Iot condition, and use 24 RB as the SRS BW condition for L1-SRS-RSRP accuracy requirements.

Measurement period

In RAN4#114 the following is agreed on measurement period for L1-SRS-RSRP.

- Agreements (from online session) - RAN4 to define one set of accuracy requirements based 1 sample. - Send LS to RAN1/2 on whether the NW configuration of timeRestrictionForChannelMeasurement or a similar configuration will apply for L1 SRS-RSRP and/or CLI-RSSI measurement. - From RAN4 requirement perspective: if it applies, RAN4 to define two sets of requirements for L1 SRS-RSRP measurement delay. - Agreements - For aperiodic resources, measurement period is based on 1 sample.

While waiting for the reply LS from RAN1, RAN4 can discuss the exact requirements to be defined based on the reply LS. According to the earlier discussion, we understand that the following is straightforward.

- If RAN1 supports NW configuration of timeRestrictionForChannelMeasurement or a similar configuration for L1-SRS-RSRP, the measurement period will be based on 1 sample if the restriction is configured, and based on 3 samples if the restriction is not configured

- If RAN1 does not support NW configuration of timeRestrictionForChannelMeasurement or a similar configuration for L1-SRS-RSRP, the measurement period will be based on 1 sample

Proposal 8: RAN4 to define L1-SRS-RSRP measurement period as follows.

- If RAN1 supports NW configuration of timeRestrictionForChannelMeasurement or a similar configuration for L1-SRS-RSRP, the measurement period will be based on 1 sample if the restriction is configured, and based on 3 samples if the restriction is not configured

- If RAN1 does not support NW configuration of timeRestrictionForChannelMeasurement or a similar configuration for L1-SRS-RSRP, the measurement period will be based on 1 sample

Specific issues for L1-CLI-RSSI

Scheduling restriction

In RAN4#114 the following is agreed for scheduling restriction due to L1-CLI-RSSI measurement.

- For L1-CLI-RSSI Methods #1, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2. - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed. - For L1-CLI-RSSI Method #3, scheduling restriction requirements are FFS

Method #1 and Method #3 are both using DL timing for measurement, and if we take assumption that UE does not need RF retuning to perform measurement on UL subband as discussed in section 2.2.1, scheduling restriction would be same for the two methods.

Figure 2 shows the analysis of the impact of UL/DL transition before and after the RSSI symbol. The main difference compared to Figure 1 is that the measurement timing is same as Rx timing, which is same as normal L1 measurement.

- In case a), the measurement does not overlap with the Rx in the previous symbol.

- In case b), the measurement does not overlap with the Tx in the previous symbol, even with the transient period for UE switching from Tx to Rx. This is because the transient period is no larger than NTA,offset, which is then no larger than TA for the Tx.

- In case c), the measurement does not overlap with the Rx in the following symbol.

- In case d), the measurement and the transient period for UE switching from Rx to Tx overlap with the Tx in the following symbol. It is clear that scheduling restriction applies to Y symbols after the measurement in UL.

Figure 2: Impact of UL/DL transition before and after the RSSI symbol

Based on the above discussions, we suggest to update the scheduling restriction requirements for L1-CLI-RSSI as in Proposal 9.

Proposal 9: Define the scheduling restriction requirements for L1-CLI-RSSI Method #1 and #3:

- For UL, scheduling restrictions always apply, for both FR1 and FR2, and the restricted symbols include symbols for RSSI resource and Y symbols after symbols for RSSI resource, where Y is re-using same number as R16 requirements

- For DL, scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, and the restricted symbols include symbols for RSSI resource.

Measurement restriction

In RAN4#113, it was agreed to adopt same principle as for R16 L3 CLI measurement to define measurement restriction for L1 CLI measurement. It means when L1-CLI-RSSI measurement resource collides with resources for L1/L3 measurement, no requirements apply for L1-CLI-RSSI measurement. One open issue is whether to consider exception cases where L1-CLI-RSSI measurement can be performed at same time with L1/L3 measurement.

We suggest to consider the exception cases. As discussed above, L1-CLI-RSSI measurement is based on DL timing and does not require RF retuning before and after the measurement, so it can be performed at same time with L1/L3 measurement in many cases, e.g. in FR1. This is quite different from L3 CLI-RSSI in R16 which is based on UL timing. In fact, L1-CLI-RSSI measurement is similar to RSSI measurement in NR-U, and no measurement restriction was defined for RSSI measurement in NR-U.

Therefore, we suggest that L1-CLI-RSSI measurement requirements do not apply only when L1-CLI-RSSI measurement cannot be performed at same time as L1/L3 measurement. In our view, they include the cases when L1-CLI-RSSI measurement resources are overlapping with

- Measurement gaps in FR1 or FR2, as UE may retune to another frequency during gaps

- SMTC window or SSB in FR2, as UE may be using a different Rx beam for L1/L3 measurement

- CSI-RS in FR2 when the CSI-RS resource and RSSI resource are not type-D QCL-ed, as UE will be using a different Rx beam for L1 measurement

Except for above cases, UE can perform L1-CLI-RSSI measurement and L1/L3 measurement at same time, and there is no need to apply the measurement restriction.

Proposal 10: For L1-CLI-RSSI Method #1 and #3, no measurement requirements apply if the L1-CLI-RSSI measurement resources are overlapping with

- Measurement gaps in FR1 or FR2, or

- SMTC window or SSB in FR2, or

- CSI-RS in FR2 when the CSI-RS resource and RSSI resource are not type-D QCL-ed.

Measurement period

In RAN4#113, it is agreed that measurement period requirements for L1-CLI-RSSI are defined based on 1 sample. In RAN4#114, following is agreed on reporting type for L1-CLI-RSSI, and one open issue is the UE measurement behavior for DUD case with resource type#2, which may impact the measurement period.

- Agreements - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements. - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types.

According to RAN1 agreements, with resource type#2, the CLI-RSSI measurement resource is configured across two DL subbands, the frequency resources for measurement is derived by excluding the frequency resources outside DL usable PRBs, and a single wideband RSSI measurement report is supported.

Our understanding of the highlighted open issue is that

- If UE is able to measure both DL subbands simultaneously with resource type#2, it means UE can get the wideband RSSI measurement result based on a single RSSI resource occasion, so the measurement period will be based on 1 sample as agreed in RAN4#113.

- If UE is not able to measure both DL subbands simultaneously with resource type#2, it means UE needs 2 RSSI resource occasions to get one wideband RSSI measurement result (sequential measurement), so the measurement period will be based on 2 samples.

Our view is that UE should be able to measure both DL subbands simultaneously with resource type#2.

- The measurement is wideband, and is supposed to reflect instantaneous total received power on the indicated time and frequency resource occasion, so the sequential measurement does not meet the target or purpose of the measurement.

- Besides, for aperiodic resource, UE anyway needs to get the wideband RSSI measurement result based on a single RSSI resource occasion according to the measurement and reporting timeline.

- From UE perspective, RSSI measurement simply reflects the total received power on the indicated time and frequency resources, and it does not require any complex signalling processing. Since UE is able to receive PDCCH and PDSCH in both DL subbands simultaneously, we do not see any issue for the UE to measure RSSI in both DL subbands simultaneously.

Besides Method #1 with resource type#2, we suggest to confirm measurement period for Method #3 is also based on 1 sample.

Proposal 11: RAN4 to confirm that measurement period for L1-CLI-RSSI is based on 1 sample for

- Method #3, and

- Method #1 with resource type#2 with the assumption that UE can measure RSSI in both DL subbands simultaneously.

Measurement accuracy

As discussed in the previous section, RSSI measurement simply reflects the total received power on the indicated time and frequency resources. In existing accuracy requirements for R16 L3 CLI-RSSI or RSSI for NR-U, no baseband estimation error was considered, and no side condition on the BW of the measurement resource was defined, so we understand the measurement accuracy is not impacted by the frequency location of the measurement resource. We suggest same accuracy requirements apply for L1-CLI-RSSI, including Method #1 resource type#1, Method #1 resource type#2 and Method #3.

Proposal 12: Same accuracy requirements apply for L1-CLI-RSSI, including Method #1 resource type#1, Method #1 resource type#2 and Method #3.

SCS for performing measurement

In RAN#120 following is agreed on how UE determines SCS of L1-CLI-RSSI measurement resource.

Agreement - UE determines the subcarrier spacing of the CLI-RSSI measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - UE determines the subcarrier spacing of the SRS-RSRP measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - Note: No RAN1 spec impact is expected.

In TS 38.133, clause 9.7.3.1, for L3 CLI-RSSI measurement we have

The subcarrier spacing for CLI-RSSI measurement resource configuration can be same or different from the subcarrier spacing of active BWP. UE shall perform CLI-RSSI measurement with the SCS of the active BWP.

Given the RAN1 agreement, SCS for L1-CLI-RSSI measurement resource configuration can only be same as the SCS of DL active BWP. Combining the R16 requirements for L3 CLI-RSSI, it is straightforward that UE shall perform L1-CLI-RSSI measurement with the SCS of the DL active BWP, for both method #1 and method #3.

Proposal 13: UE shall perform L1-CLI-RSSI measurement with the SCS of the DL active BWP, for both method #1 and method #3.

Spec impact of the WI

In this section, we provide our analysis for the spec impact of the WI, including both UE CLI measurement and impacts of SBFD operation.

In RAN4#114, some companies discussed the spec structure for UE CLI measurement, with two options.

- Option 1: Sub clause under 9.7. (e.g. 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements)

- Option 2: New clause for CLI measurement of the SBFD. (E.g. 9.7X Cross Link Interference measurements for SBFD)

We support option 2. The reason is that L3 CLI and L1 CLI measurement are separate features, with separate UE capability, NW configuration and PHY procedure. In addition, clause 9.7.4 (Scheduling availability of UE during CLI measurements) is a common clause for clause 9.7, and it is difficult to add scheduling restriction for L1 CLI measurement in the same clause. For simplicity, we suggest to create a new clause 9.x for L1 CLI measurement.

Based on discussions and agreements so far, we listed the impacted requirements and corresponding clauses in Table 1. We suggest to endorse it for CR drafting work.

Proposal 14: Endorse Table 1 for CR drafting work for SBFD RRM.

Table 1: Spec impacts and work split for SBFD RRM

| Objective | Impacted requirements and clauses | Contents | Company |
| --- | --- | --- | --- |
| General | 3.1 Definitions | Definition of SBFD symbol, UL and DL subband, etc. |  |
|  | 3.3 Abbreviations | Abbreviations for SBFD |  |
| RRM requirements for UE-to-UE CLI handling | 9.x L1 CLI measurement (new clause) |  |  |
|  | 9.x.1 L1-SRS-RSRP measurements |  |  |
|  | 9.x.1.1 Introduction | General description of the measurement: capability, configuration, measurement resource location and SCS, measurement timing, Rx beam determination, … |  |
|  | 9.x.1.2 Requirements applicability | Similar to 9.7.2.2 |  |
|  | 9.x.1.3 Measurement Reporting Requirements | Similar to 9.7.2.3 (only aperiodic reporting) |  |
|  | 9.x.1.4 Measurement capability | Similar to 9.7.2.4 (exact number TBD pending on RAN1 agreements) |  |
|  | 9.x.1.5 Measurement period | Including also measurement restriction |  |
|  | 9.x.1.6 Scheduling availability | Similar to 9.7.4, for L1-SRS-RSRP only |  |
|  | 9.x.2 L1-CLI-RSSI measurements |  |  |
|  | 9.x.2.1 Introduction | General description of the measurement: capability, configuration, measurement resource location and SCS, measurement timing, Rx beam determination, … |  |
|  | 9.x.2.2 Requirements applicability | Similar to 9.7.3.2 |  |
|  | 9.x.2.3 Measurement Reporting Requirements | Similar to 9.7.3.3 (only periodic and aperiodic reporting) |  |
|  | 9.x.2.4 Measurement capability | Similar to 9.7.3.4 (exact number TBD pending on RAN1 agreements) |  |
|  | 9.x.2.5 Measurement period | Including also measurement restriction |  |
|  | 9.x.2.6 Scheduling availability | Similar to 9.7.4, for L1-CLI-RSSI only |  |
|  | Others, if identified |  | TBD |
| RRM impacts for SBFD operation | 8.1.3.2 Minimum requirement | Update CSI-RS based RLM evaluation period for impact of SBFD operation. Changes should be aligned with 8.5.3.2 and 8.5.6.2. |  |
|  | 8.5.3.2 Minimum requirement | Update CSI-RS based BFD evaluation period for impact of SBFD operation. Changes should be aligned with 8.1.3.2 and 8.5.6.2. |  |
|  | 8.5.6.2 Minimum requirement | Update CSI-RS based CBD evaluation period for impact of SBFD operation. Changes should be aligned with 8.1.3.2 and 8.5.3.2. |  |
|  | 9.5.4.2 CSI-RS based L1-RSRP Reporting | Update CSI-RS based L1-RSRP measurement period for impact of SBFD operation. Changes should be aligned with 9.8.4. |  |
|  | 9.8.4.1 L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured | Update CSI-RS based L1-SINR measurement period for impact of SBFD operation. Changes should be aligned with 9.5.4.2. |  |
|  | 9.8.4.2 L1-SINR reporting with SSB based CMR and dedicated IMR configured |  |  |
|  | 9.8.4.3 L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured |  |  |
|  | 9.10.1 Introduction | Clarification of L3 CSI-RS measurement requirement. |  |
|  | Others, if identified | Based on discussions so far, possible impacts could be for - Inter-frequency SSB measurement without gap - RACH requirements - MG and BWP switch for term “UL slot” | TBD |

Conclusions

In this paper we provided RRM requirements for UE CLI measurement in SBFD.

Proposal 1: RAN4 to define measurement reporting requirements for periodic L1 CLI-RSSI reporting. L1-RSRP measurement reporting requirement for periodic reporting as defined in clause 9.5.3.1 are re-used.

Proposal 2: RAN4 to confirm the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI, including both absolute and differential reporting, are reasonable. RAN4 to inform RAN1 the “Infinite” in Table 10.1.22.1.2-1, TS38.133 is also reused for L1 SRS-RSRP.

Proposal 3: L1 based UE-to-UE CLI measurement requirements apply provided that

- when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

- when L1-CLI-RSSI measurement resource is configured within BW of DL active BWP

Proposal 4: Define the scheduling restriction requirements for L1-SRS-RSRP:

- For UL, scheduling restrictions always apply, for both FR1 and FR2, and the restricted symbols include symbols for SRS resource and Y symbols before and after symbols for SRS resource, where Y is re-using same number as R16 requirements

- For DL,

  - if UE is able to support FDM-ed DL reception and SRS measurement and if RAN1 will define this optional UE capability, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2,

  - otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies.

  - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements.

Proposal 5: If the SRS resources configured for L1-SRS-RSRP are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP/L1-SINR measurement or measurement gaps, measurement period requirements for L1-SRS-RSRP do not apply.

Proposal 6: If the SRS resources configured for L1-SRS-RSRP are partially or fully overlapping with measurement resources for L1-CLI-RSSI, UE is required to perform one but not both of the two measurements, and measurement period requirements for the two measurements do not apply.

Proposal 7: RAN4 to stick to 1dB as the on Es/Iot condition, and use 24 RB as the SRS BW condition for L1-SRS-RSRP accuracy requirements.

Proposal 8: RAN4 to define L1-SRS-RSRP measurement period as follows.

- If RAN1 supports NW configuration of timeRestrictionForChannelMeasurement or a similar configuration for L1-SRS-RSRP, the measurement period will be based on 1 sample if the restriction is configured, and based on 3 samples if the restriction is not configured

- If RAN1 does not support NW configuration of timeRestrictionForChannelMeasurement or a similar configuration for L1-SRS-RSRP, the measurement period will be based on 1 sample

Proposal 9: Define the scheduling restriction requirements for L1-CLI-RSSI Method #1 and #3:

- For UL, scheduling restrictions always apply, for both FR1 and FR2, and the restricted symbols include symbols for RSSI resource and Y symbols after symbols for RSSI resource, where Y is re-using same number as R16 requirements

- For DL, scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, and the restricted symbols include symbols for RSSI resource.

Proposal 10: For L1-CLI-RSSI Method #1 and #3, no measurement requirements apply if the L1-CLI-RSSI measurement resources are overlapping with

- Measurement gaps in FR1 or FR2, or

- SMTC window or SSB in FR2, or

- CSI-RS in FR2 when the CSI-RS resource and RSSI resource are not type-D QCL-ed.

Proposal 11: RAN4 to confirm that measurement period for L1-CLI-RSSI is based on 1 sample for

- Method #3, and

- Method #1 with resource type#2 with the assumption that UE can measure RSSI in both DL subbands simultaneously.

Proposal 12: Same accuracy requirements apply for L1-CLI-RSSI, including Method #1 resource type#1, Method #1 resource type#2 and Method #3.

Proposal 13: UE shall perform L1-CLI-RSSI measurement with the SCS of the DL active BWP, for both method #1 and method #3.

Proposal 14: Endorse Table 1 for CR drafting work for SBFD RRM.

Table 1: Spec impacts and work split for SBFD RRM

| Objective | Impacted requirements and clauses | Contents | Company |
| --- | --- | --- | --- |
| General | 3.1 Definitions | Definition of SBFD symbol, UL and DL subband, etc. |  |
|  | 3.3 Abbreviations | Abbreviations for SBFD |  |
| RRM requirements for UE-to-UE CLI handling | 9.x L1 CLI measurement (new clause) |  |  |
|  | 9.x.1 L1-SRS-RSRP measurements |  |  |
|  | 9.x.1.1 Introduction | General description of the measurement: capability, configuration, measurement resource location and SCS, measurement timing, Rx beam determination, … |  |
|  | 9.x.1.2 Requirements applicability | Similar to 9.7.2.2 |  |
|  | 9.x.1.3 Measurement Reporting Requirements | Similar to 9.7.2.3 (only aperiodic reporting) |  |
|  | 9.x.1.4 Measurement capability | Similar to 9.7.2.4 (exact number TBD pending on RAN1 agreements) |  |
|  | 9.x.1.5 Measurement period | Including also measurement restriction |  |
|  | 9.x.1.6 Scheduling availability | Similar to 9.7.4, for L1-SRS-RSRP only |  |
|  | 9.x.2 L1-CLI-RSSI measurements |  |  |
|  | 9.x.2.1 Introduction | General description of the measurement: capability, configuration, measurement resource location and SCS, measurement timing, Rx beam determination, … |  |
|  | 9.x.2.2 Requirements applicability | Similar to 9.7.3.2 |  |
|  | 9.x.2.3 Measurement Reporting Requirements | Similar to 9.7.3.3 (only periodic and aperiodic reporting) |  |
|  | 9.x.2.4 Measurement capability | Similar to 9.7.3.4 (exact number TBD pending on RAN1 agreements) |  |
|  | 9.x.2.5 Measurement period | Including also measurement restriction |  |
|  | 9.x.2.6 Scheduling availability | Similar to 9.7.4, for L1-CLI-RSSI only |  |
|  | Others, if identified |  | TBD |
| RRM impacts for SBFD operation | 8.1.3.2 Minimum requirement | Update CSI-RS based RLM evaluation period for impact of SBFD operation. Changes should be aligned with 8.5.3.2 and 8.5.6.2. |  |
|  | 8.5.3.2 Minimum requirement | Update CSI-RS based BFD evaluation period for impact of SBFD operation. Changes should be aligned with 8.1.3.2 and 8.5.6.2. |  |
|  | 8.5.6.2 Minimum requirement | Update CSI-RS based CBD evaluation period for impact of SBFD operation. Changes should be aligned with 8.1.3.2 and 8.5.3.2. |  |
|  | 9.5.4.2 CSI-RS based L1-RSRP Reporting | Update CSI-RS based L1-RSRP measurement period for impact of SBFD operation. Changes should be aligned with 9.8.4. |  |
|  | 9.8.4.1 L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured | Update CSI-RS based L1-SINR measurement period for impact of SBFD operation. Changes should be aligned with 9.5.4.2. |  |
|  | 9.8.4.2 L1-SINR reporting with SSB based CMR and dedicated IMR configured |  |  |
|  | 9.8.4.3 L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured |  |  |
|  | 9.10.1 Introduction | Clarification of L3 CSI-RS measurement requirement. |  |
|  | Others, if identified | Based on discussions so far, possible impacts could be for - Inter-frequency SSB measurement without gap - RACH requirements - MG and BWP switch for term “UL slot” | TBD |

Reference

- R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, Apple

- R1-2501620, LS on L1 based UE-to-UE CLI measurement and reporting, RAN1

Annex: draft reply LS on L1 based UE-to-UE CLI measurement and reporting

3GPP TSG-RAN WG4 Meeting #114-bis R4-250xxxx Wuhan, China, 7 – 11 April, 2025 Title: Reply LS on L1 based UE-to-UE CLI measurement and reporting Response to: R1-2501620 Release: Release 19 Work Item: NR_duplex_evo-Core Source: RAN4 To: RAN1 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 thanks RAN1 for the information in LS R1-2501620. RAN4 discussed the report mapping for L1 based UE-to-UE CLI measurement in RAN4#114-bis, and would like to provide following answers to the questions in R1-2501620. - Is the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI in the RAN1 working assumption reasonable from RAN4 perspective? [RAN4] Yes, RAN4 considers the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI in the RAN1 working assumption as reasonable from RAN4 perspective. In addition, RAN4 agrees that the reporting of “Infinite” in Table 10.1.22.1.2-1, TS38.133 is also reused for L1 SRS-RSRP. - Is the range and resolution for differential reporting of L1 SRS-RSRP and CLI-RSSI reporting reasonable from RAN4 perspective? [RAN4] Yes, RAN4 considers the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI reporting in the RAN1 working assumption as reasonable from RAN4 perspective. RAN4 respectfully asks RAN1 to take the above information into account in the future work for L1 based UE-to-UE CLI measurement and reporting. 2. Actions: To RAN1: RAN4 respectfully asks RAN1 to take the above information into account in the future work for L1 based UE-to-UE CLI measurement and reporting. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #115 May 19 – May 23, 2025 Malta, MT RAN WG4 Meeting #116 August 25 – August 29, 2025 India (TBC), IN